3GPP TSG-RAN WG2 NR Ad hoc 1807	R2-1810686

Montreal, Canada, 2nd July – 6th July 2018

Agenda Item	: 10.3.1.4.3 (NR_newRAT-Core)

Source	: LG Electronics Inc.

Title	: Msg3 problem in switching between CBRA and CFRA

Document for	: Discussion and Decision

1.	Introduction

In RAN2#102, [R2-1806919] pointed out that Msg3 may not be able to be transmitted due to switch between CBRA and CFRA.

In this contribution, we investigate whether there is a problem with Msg3 in this case.

2.	Discussion

In NR, it is possible for a UE to switch between CBRA and CFRA because the UE performs random access resource selection from the scratch. It was pointed out in [R2-1806919] that, if the RA procedure for handover switches from CBRA to CFRA, the UE may not transmit the Msg3 may because RA is considered successful when RAR is received successfully.

As per TS 38.321, the MAC entity shall flush the HARQ buffer for transmission of MAC PDU:

- if the Contention Resolution is considered not successful;

- Upon completion of the Random Access procedure

In addition, the MAC entity shall flush the Msg3 buffer either when the Random Access procedure is initiated or upon MAC reset. Therefore, although CFRA is successful, the MAC still has Msg3 in the Msg3 buffer.

In the meanwhile, when Random Access Response reception is considered successful, the UE shall process the received UL grant even for CFRA. From HARQ entity point of view, if the delivered UL grant is received in RAR, the UE will obtain the MAC PDU from the Msg3 buffer and performs a new transmission by storing the MAC PDU in the HARQ buffer.

In summary, once handover complete message is stored in Msg3 buffer, it is to be transmitted even after CFRA completion. Moreover, it is transmitted via RLC AM and RLC retransmission also works in this case. Therefore, we see no clarification is needed for this.

Proposal. It is clear from the specification that Msg3 buffer stores the handover complete message even after CFRA completion and there is no problem in transmission of Msg3.

3.	Conclusion

In this contribution, we investigated to see if there is any problem in Msg3 transmission when the MAC entity switches to CFRA and successfully completes it.

Proposal. It is clear from the specification that Msg3 buffer stores the handover complete message even after CFRA completion and there is no problem in transmission of Msg3.