3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810862

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.4.1.6.8

Source:	Nokia, Nokia Shanghai Bell

Title:	TP offline #26

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

After further checking changes on introduction chapter of system information is not needed. In fact changes proposed were misleading and better to avoid changes there. But then in actions to acquire SIB1 (after reading MIB) was seen better to clarify only to acquire if indicated to be transmitted by ssb-SubCarrierOffset.

# 2	Text Proposal

##### --------------------------------------------NEXT CHANGES--------------------------------------------

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE or if the UE is in RRC_CONNECTED while T311 is running: [FFS]

2> if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if intraFreqReselection is set to notAllowed:

4> consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20].

3>	else:

4> consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20].

2>	else;

3>	apply the received pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position andacquire SIB1,  if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell [13].