3GPP TSG-RAN WG2 NR AH1807	    R2-1809738

Montreal, Canada, 2nd – 6th July 2018                                                   Revision of R2-1807385

Agenda Item:	10.3.4.3

Souce:	MediaTek Inc.

Title:	Some remaining issues for QoS flow remapping

Document for:		Discussion and decision

# 1 Introduction

In RAN2#102, the topic of QoS flow remapping was discussed and the following agreement was reached [1].

=>	A one byte SDAP PDU (a SDAP control PDU) is used as end marker PDU.

The format of the SDAP control PDU is shown in Figure 1.

Figure 1: End-Marker Control PDU

The purpose of this document is to discuss some related scenarios that have not been fully fleshed out, especially when UL SDAP header is not configured, and present our views of when and how the SDAP control PDU can be used to support in-sequence QoS flow remapping.

# 2 Discussion

##### 2.1 Non-symmetric flow remapping

In previous discussion on this topic, we assumed that both the “old” and “new” DRBs are configured with UL SDAP header. The presence/absence of UL SDAP header depends on network configuration, and in some scenarios the gNB may not configure the UL SDAP header on the “old” and/or “new” DRB. (e.g., when only a single QoS flow is mapped to a DRB). We refer to such scenarios as non-symmetric and analyze how the SDAP control PDU can be used these cases.

Case 1: “Old” DRB has UL SDAP configured, but “new” DRB does not

In this case, the UE can always send a SDAP control PDU on the “old” DRB, so we do not think any particular enhancement is needed.

Case 2: “Old” DRB does not have UL SDAP header configured, but “new” DRB has

In this case, the UE cannot send a SDAP control PDU on the “old” DRB. However, note that the SDAP control PDU can be sent on the “new” DRB. So one possible scheme is that when the UE finishes transmitting all pending packets which belonged to the remapped flow on the “old” DRB (so ensuring that the gNB has received all the “old” packets), it sends the SDAP control PDU on the “new” DRB. When the gNB receives the SDAP control PDU, it can safely start forwarding packets received on the “new” DRB to higher layers in-sequence. In this solution, it is NOT required for the UE to buffer new packets, just that the UE is required to send the SDAP control PDU only when all the “old” packets have been delivered to the gNB.

Case 3: Neither the “old” nor the “new” DRB have SDAP configured

In this case, the only possibility (to ensure no out-of-sequence delivery) appears to rely on the UE (transmitter) to buffer all new packets till the packets belonging to the QoS flow that are being sent on the “old” DRB are delivered.

A table outlining these options is shown below.

| UL SDAP header |  | “New” DRB |  |
| --- | --- | --- | --- |
|  |  | Present | Not present |
| “Old” DRB | Present | SDAP control PDU on old DRB | SDAP control PDU on old DRB |
|  | Not present | SDAP control PDU on new DRB | Transmitter buffering |

Table 1: Mechanisms for in-sequence QoS flow relocation

Based on the analysis above, we propose the following.

Proposal 1: If the “old” DRB is not configured with UL SDAP header, and the “new” DRB is configured with the UL SDAP header, then the UE will send the SDAP control PDU on the new DRB, once all packets on the “old” DRB are delivered.

Proposal 2: If neither the “old” DRB nor the “new” DRB have UL SDAP configured, then the UE will buffer packets belonging to the “new” DRB till all packets for the “old” DRB are delivered.

##### 2.3 Configurability of in-sequence

A solution for in-sequence flow remapping may not always be desired by the network. For example, in some cases, when UL SDAP header is not configured (see Proposal 1), it might result in additional delay. For this reason, we think the use of in-sequence delivery mechanisms should be configurable by the network.

Proposal 3: The network can configure whether or not the UE implements mechanisms to support in-sequence QoS flow remapping.

If proposal 3, is agreeable then the granularity at which the configuration applies needs to be decided. The potential candidates are PDU session, DRB, and QoS flow. Specifying at DRB level can lead to inconsistencies (what if the “new” and “old” DRBs are configured differently?), and specifying at the QoS flow level seems to require additional signaling which is undesirable. So we think for Rel-15 at least, it suffices to allow network configurability at PDU session level.

Proposal 4: The network can configure applicability of in-sequence QoS flow remapping mechanisms at PDU session level.

Finally, these mechanisms require the UE to perform additional processing and possibly buffering. We think these mechanisms can be part of UE capability.

Proposal 5: A UE capability is defined for support of in-sequence QoS flow remapping.

# 3 Conclusions

In this paper, we have made the following observations and proposals:

Proposal 1: If the “old” DRB is not configured with UL SDAP header, and the “new” DRB is configured with the UL SDAP header, then the UE will send the SDAP control PDU on the new DRB, once all packets on the “old” DRB are delivered.

Proposal 2: If neither the “old” DRB nor the “new” DRB have UL SDAP configured, then the UE will buffer packets belonging to the “new” DRB till all packets for the “old” DRB are delivered.

Proposal 3: The network can configure whether or not the UE implements mechanisms to support in-sequence QoS flow remapping.

Proposal 4: The network can configure applicability of in-sequence QoS flow remapping mechanisms at PDU session level.

Proposal 5: A UE capability is defined for support of in-sequence QoS flow remapping.

# 4 References

[1]. R2-1808732, Report NR UP breakout session, Vice Chairman (MediaTek Inc.).