3GPP TSG-RAN WG3#100	R3-183303

Busan, Korea, 21 – 25 May 2018

Agenda item:	10.5.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Response to R3-183028

Document for:	Discussion and Decision

# Description and proposal

RAN3 received the LS R3-182562 at this meeting where SA2 indicates that they have re-discussed the topic of length of QFI. Here is an excerpt of the LS:

SA2 has re-discussed this topic and would like to confirm that limiting the QFI to 6 bits (i.e. 64 values) across the 5G System is considered adequate for Rel-15.

In SA2#127, following discussions on some companies’ proposal, SA2 concluded that a potential extension of the QFI beyond 6 bits may be considered in a future release (e.g. in Rel-16). SA2 would like to stress that Release 15 should be defined such that future extensibility should not be precluded.

From above we can clearly see that SA2 has confirmed that QFI range 64 is good enough for release 15. This means that in TS 38.415 and SDAP we can optimize to use encoding in 6 bits.

Imagine in release 16 we introduce a 14 bits QFI, then it should be done in a backwards compatible way i.e. in the PDU session setup the NG-RAN must clearly be made aware that 5GC intends to use the “extended QFI”. This means that in TS 38.415 we would define in release 16 (using the extension mechanism explained last RAN3 for TS 38.425) a new “extended QFI” field of length 14 bits.

NG-RAN node should clearly understand which version of TS38.415 it must use. Also if NG-RAN is release 15 and 5GC is release 16 and request use of extended QFI, the NG-RAN node shall fail the PDU Session setup to signal that it doesn’t support this encoding.

It is therefore necessary to adopt in release 15 a QFI encoding which follows two criteria:

- Includes only 64 values,

- Is extensible in a backwards compatible manner to manage the different release between 5GC node and NG-RAN node.

one way that RAN3 has already used so far is to have a choice structure which would only include the basic QFI of range 64 in release 15, which choice can be extended in release 16. Once extended it would inherit the criticality of the upper level IE.

Proposal: agree the TP below for QFI in release 15 in order to implement the future extensibility requested by SA2.

# Text Proposal for TS 38.423

#### 9.2.2.34	QoS Flow Indicator

This IE identifies a QoS Flow within a PDU Session. Definition and use of the QoS Flow Indicator is specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE QoS Flow Indicator | M |  |  |  |
| >Basic QFI |  |  |  |  |
| >>Basic QFI | M |  | INTEGER (0 ..63) |  |

# Annex

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | Spare |  |  |  | 1 |
| Spare | RQI or spare | QoS Flow Identifier |  |  |  |  |  | 1 |
| Padding |  |  |  |  |  |  |  | 0-3 |

PDU SESSION INFORMATION (PDU Type 0) Format

# A.1	Example of using Future Extension Field

| New IE flag 7(E) | New IE flag 6 | New IE flag 5 | New IE flag 4 | New IE flag 3 | New IE flag 2 | New IE flag 1 | New IE flag 0 | 1 Octet New IE Flags |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| New IE 1 |  |  |  |  |  |  |  | 3 Octets |
| New IE 2 |  |  |  |  |  |  |  | 2 Octets |

Figure X.Y: Example of future Extension Field

In the Example of the future Extension Field, New IE flag 0 indicates if the New IE 1 is present or not. New IE flag 1 indicaes if the new IE 2 is present or not.

## A.1.1	New IE Flags

Description: The New IE Flags IE is only present if at least one new IE is present. The New IE Flags IE contains flags indicating which new IEs that are present following the New IE Flags IE. The last bit position of the New IE Flags IE is used as the Extension Flag to allow the extension of the New IE Flags IE in the future. Extension octets of the New IE Flags IE shall follow directly after the first octet of the New IE Flags IE. When an extension octet of the New IE Flags IE is present, then all previous extension octets of the New IE Flags IE and the New IE Flags IE shall also be present, even if they have all their flag bits indicating no presence of their respective new IEs.