3GPP TSG RAN WG3 NR Meeting#100	R3-182750

Busan, Korea, 21 - 25 May 2018

Agenda Item:	10.10.7

Source:	Ericsson

Title:	E1 support for UP version handling

Document for:	pCR TS 38.463

# Introduction

In R3-182749 proposals were made to enable UP version handling via the CP. In this paper a TP reflecting those proposals is presented.

# Annex 1: TP for the BL CR to TS 38.463

Start of Text Proposal 1 for TS 38.463

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

If the gNB-CU-UP Highest Supported UP Version IE is included in the BEARER CONTEXT SETUP RESPONSE, the gNB-CU-CP shall take this value as the highest UP protocol version supported by the gNB-CU-UP and it shall forward it to concerned gNB-DUs over the F1-C interface.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: Bearer Context Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot establish the requested bearer context, it should respond with a BEARER CONTEXT SETUP FAILURE message and appropriate cause value.

#### 8.3.1.4	Abnormal Conditions

Not applicable.

End of Text Proposal 1 for TS 38.463

Start of Text Proposal 2 for TS 38.463

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

Editor’s note: Further description of this procedure is FFS.

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modifcation procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Common Highest Supported UP Version IE is included in the BEARER CONTEXT MODIFICATION REQUEST, the gNB-CU-UP shall consider this as the version to use for UP communication for the concerned bearer.

#### 8.3.2.3	Unsuccessful Operation

Figure 8.3.2.3-1: Bearer Context Modification procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot modify the requested bearer context, it should respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

#### 8.3.2.4	Abnormal Conditions

Not applicable.

End of Text Proposal 2 for TS 38.463

Start of Text Proposal 3 for TS 38.463

#### 9.2.2.2	BEARER CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

Editor’s note: The presence of all the IEs in this message needs further checking. The definition of the IEs is FFS.

Editor’s note: Whether to introduce E-RAB ID for E-UTRAN is FFS.

Editor’s note: Whether it is needed to distinguish between different interfaces (F1, X2, Xn) is FFS.

Editor’s note: How to encode information about NAS reflective QoS (RQA) and AS reflective QoS is FFS (i.e., whether to have separate IEs or include it in the QoS flow level information).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List |  | 1 |  |  | YES | ignore |
| >>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>S1 DL UP Transport Layer Information | M |  |  |  | - | - |
| >>>>Data forwarding information | O |  |  |  | - | - |
| >>>>UL UP Transport Parameters | M |  | 9.3.1.13 |  | - | - |
| gNB-CU-UP Highest Supported UP Version | O |  | 9.3.1.x |  |  |  |
| >>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>Cause | M |  |  |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List |  | 1 |  |  | YES | ignore |
| >>>PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  |  |  | - | - |
| >>>>PDU Session Resource Setup Response Transfer | M |  |  | Defined in TS 38.413 [6]. | - | - |
| >>>>DRB Setup List |  | 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>UL UP Parameters | M |  | 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | 9.3.1.12 |  |  |  |
| >>>>>>Flow Failed List | M |  | 9.3.1.12 |  |  |  |
| >>>>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>Cause | M |  |  |  | - | - |
| >>PDU Session Resource Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  |  |  | - | - |
| >>>>Cause | M |  |  |  | - | - |

End of Text Proposal 3 for TS 38.463

Start of Text Proposal 4 for TS 38.463

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

Editor’s note: The presence of all the IEs in this message needs further checking. The definition of the IEs is FFS.

Editor’s note: Whether to introduce E-RAB ID for E-UTRAN is FFS.

Editor’s note: Whether it is needed to distinguish between different interfaces (F1, X2, Xn) is FFS.

Editor’s note: How to encode information about NAS reflective QoS (RQA) and AS reflective QoS is FFS (i.e., whether to have separate IEs or include it in the QoS flow level information).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates to Suspend or Resume the Bearer Context | YES | reject |
| CHOICE system | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>PDCP Configuration | M |  | OCTET STRING | PDCP-config as defined in TS 38.331 [10]. | - | - |
| >>>>E-UTRAN QoS | M |  |  |  | - | - |
| >>>>S1 UL UP Trasnport Layer Information | M |  |  |  | - | - |
| >>>>Data forwarding information | O |  |  |  | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>DRB To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>PDCP Configuration | O |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y]. | - | - |
| >>>>E-UTRAN QoS | O |  |  |  | - | - |
| >>>>S1 UL UP Parameters | O |  |  |  | - | - |
| >>>>Data forwarding information | O |  |  |  | - | - |
| >>>>PDCP UL COUNT | O |  |  | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>PDCP DL COUNT | O |  |  | PDCP count for next DL packet to be assigned. | - | - |
| >>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>Cell Group To Add | O |  | 9.3.1.12 |  | - | - |
| Highest Common Supported UP Version | O |  | 9.3.1.x |  |  |  |
| >>>>Cell Group To Modify | O |  | 9.3.1.12 |  | - | - |
| >>>>Cell Group To Remove | O |  | 9.3.1.12 |  | - | - |
| >>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  |  |  | - | - |
| >>>>S-NSSAI | O |  |  |  | - | - |
| >>>>PDU Session Resource Setup Request Transfer | M |  |  | Defined in TS 38.413 [6]. | - | - |
| >>>>DRB To Setup List |  | 1 |  |  |  |  |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>SDAP Configuration | M |  | OCTET STRING | SDAP-config as defined in TS 38.331 [10]. | - | - |
| >>>>>>PDCP Configuration | M |  | OCTET STRING | PDCP-config as defined in TS 38.331 [10] | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | O |  | 9.3.1.12 |  | - | - |
| >>PDU Session Resource To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>DRB To Setup List |  | 1 |  |  |  |  |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>SDAP Configuration | O |  | OCTET STRING | SDAP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>PDCP Configuration | O |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y] | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | O |  | 9.3.1.12 |  | - | - |
| >>>>DRB To Modify List |  | 0.. 1 |  |  |  |  |
| >>>>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>SDAP Configuration | O |  | OCTET STRING | SDAP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>PDCP Configuration | O |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>PDCP UL COUNT | O |  |  | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL COUNT | O |  |  | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>>>Cell Group To Add | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Modify | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Remove | O |  | 9.3.1.11 |  | - | - |
| >>>>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>PDU Session Resource To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  |  |  | - | - |

End of Text Proposal 4 for TS 38.463

Start of Text Proposal 5 for TS 38.463

#### 9.3.1.x	UP Protocol Version

The UP Protocol Version is an index to the version of the UP protocol supported by the node that signals the IE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UP Protocol Version | M |  | INTEGER (0 .. 255) | Each value corresponds to a version of the UP protocol specification TS38.425. Value “0” corresponds to version 15.0.0, Value “1” corresponds to version 15.1.0. Each following index corresponds to the next available version of TS38.425. |

End of Text Proposal 5 for TS 38.463