3GPP TSG-RAN WG3 #100	Tdoc R3-183224

Busan, Korea, 21st – 25th May

Agenda Item:	23.3

Source:	Ericsson

Title:	E1 Setup procedure for 38.460

Document for:	pCR TS 38.460

# Introduction

This paper follows the discussion in R3-183211 on the E1 setup procedure. Specifically, this contribution aims at providing the required specification text for the following proposal:

Proposal 1: 	The CU-CP and CU-UP: shall support the “gNB-CU-CP E1 Setup” procedure and may support the “gNB-CU-UP E1 Setup” procedure. This aligns the E1 interface with similar interfaces in the (4G and 5G) CN.

Note that the text proposed in this paper is inspired from TS 29.244 (section 6.2.6.1) where the interface between the control plane function and the user plane function in the core network is described.

# TP for TS 38.460

<<<<<<	START TEXT PROPOSAL	>>>>>>

### 5.1.1	E1 interface management function

The error indication function is used by the gNB-CU-UP or gNB-CU-CP to indicate to the gNB-CU-CP or gNB-CU-UP that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-CU-UP and the gNB-CU-CP.

The E1 setup function allows to exchange application level data needed for the gNB-CU-UP and gNB-CU-CP to interoperate correctly on the E1 interface. The E1 setup is initiated by both the gNB-CU-UP and gNB-CU-CP. The gNB-CU-CP and the gNB-CU-UP shall support the E1 setup procedure initiated by the gNB-CU-CP. The gNB-CU-CP and the gNB-CU-UP may additionally support the E1 setup procedure initiated by the gNB-CU-UP.

The gNB-CU-UP Configuration Update and gNB-CU-CP Configuration Update functions allow to update application level configuration data needed between the gNB-CU-CP and the gNB-CU-UP to interoperate correctly over the E1 interface.

The E1 setup and gNB-CU-UP Configuration Update functions allow to inform S-NSSAI(s) and PLMN-ID(s) supported by the gNB-CU-UP.

Editor’s note: Further contexts to be exchanged over E1 are FFS.

<<<<<<	END TEXT PROPOSAL	>>>>>>