3GPP TSG RAN WG3 #100	Tdoc R3-183363

# Busan, Korea, 21st- 25th May 2018

Agenda Item:	31.3.1.3

Source:	Ericsson

Title:	(TP for EN-DC BL CR for TS 38.473): Measurement configuration

Document for:	TP for EN-DC BL CR for TS 38.473

# Introduction

The following TP captures agreements on measurement configuration for EN-DC.

# Annex I: TP for SA BL CR for TS 38.473

<<<<<<<<<<<<<<<<<<<< Requested Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.25	CU to DU RRC Information

This IE contains the RRC Information that are sent from gNB-CU to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CG-ConfigInfo | O |  | OCTET STRING | CG-ConfigInfo, as defined in TS 38.331 [8]. |
| UE-CapabilityRAT-ContainerList | O |  | OCTET STRING | UE-CapabilityRAT-ContainerList, as defined in TS 38.331 [8]. |
| MeasConfig | O |  | OCTET STRING | MeasConfig, as defined in TS 38.331 (without MeasGapConfig). For EN-DC operation, includes the list of FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps. This IE may be redefined. |

#### 9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. |
| MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in TS 38.331 [8]. For EN-DC operation, includes the gap for FR2, as requested by the gNB-CU via MeasConfig IE. This IE may be redefined. |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>