3GPP TSG-RAN WG3 #101	Tdoc R3-184985

Gothenburg, Sweden, 20th – 24th August

Agenda Item:	31.3.1.14

Source:	Ericsson

Title:	PLMN information over F1

Document for:	TP for BL CR for TS 38.473

# Introduction

At the previous meeting the serving PLMN was agreed to be added in the UE Context Setup Request message. However, there are still cases where the PLMN information needs to be exchanged over F1 that have not been addressed yet. Since the gNB-DU broadcasts the available PLMNs in SIB1, it needs to be aware of the availability status of the PLMNs. In case that the gNB-CU loses NG connectivity toward one or more AMFs, it may not be possible to connect to some PLMNs anymore. In this case, the gNB-CU needs to inform the gNB-DU, which in turn needs to update the content of SIB1.

In this contribution, we elaborate further on this issue and propose solutions.

# Discussion

The gNB-DU is responsible for sending out system information comprising among other things ‘supported PLMNs’ in SIB1. This information is dependent on NG status, for instance if the NG connections supporting a certain PLMN are not functioning, SIB1 needs to be updated. Currently this information is not transferred from gNB-CU to gNB-DU. Consequently, gNB-DU is unaware of the actual availability status of the various PLMNs, leading to delays and errors. So, if due to for instance a failure in the transport network that connects the gNB-CU to the 5G core or a failure in the 5G core itself the gNB-CU temporary loses NG connectivity to all the AMFs serving a given PLMN, the gNB-CU has no means to inform the gNB-DU about the unavailability of these PLMNs. Consider also special deployment such as defense systems and temporary deployments for emergency and other dedicated deployments. In these cases, it is even more crucial that delays and errors are avoided due to the inability of the gNB-CU to inform the gNB-DU about the unavailable PLMNs.

Further, we need to consider special deployments where a PLMN can be served by a single AMF. It could for example be the case of a factory that employs an AMF and a PLMN ID to provide special services to its customers/employees. The factory may decide to keep the AMF accessible only during the working days. In the weekend, the factory may switch-off the AMF and the corresponding services. In such scenario, it would be beneficial to allow the gNB-CU to inform the gNB-DU about the PLMN availability of the PLMNs, so that UEs will not try to access a PLMN that is temporary not present.

A straightforward solution that covers the cases described, is that a list of available PLMNs is added in the F1 Setup Response message in the ‘cells to be activated’ list for each cell. To cover the case of subsequent changes of the NG status, analogous information is needed in the gNB-CU Configuration Update message.  Therefore, based on the above we propose to add a list of “available PLMNs” per each cell that is to be activated in the F1 Setup Response message and the gNB-CU Configuration Update message.

Proposal 1: It is proposed to add a list of “available PLMNs” per each cell that is to be activated in the F1 Setup Response message and the gNB-CU Configuration Update message.

Proposal 2: RAN3 is kindly requested to agree with the text proposal in Annex 1.

# Conclusion

In this contribution, we elaborated on the PLMN information needed over the F1 interface.

Proposal 1: It is proposed to add a list of “available PLMNs” per each cell that is to be activated in the F1 Setup Response message and the gNB-CU Configuration Update message.

Proposal 2: RAN3 is kindly requested to agree with the text proposal in Annex 1.

# Annex 1: TP for the SA BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  |  |  |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | YES | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - | - |
| >>Available PLMN List |  | 0.. 1 |  | List of available PLMNs | YES | reject |
| >>>Available PLMN List Item |  | 1.. <maxnoofBPLMNs> |  |  | EACH | reject |
| >>>>PLMN ID | M |  | 9.3.1.14 |  | - | - |
| >>>>PLMN Status | M |  | 9.3.1.x |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |

Start of Text Proposal 2 for TS 38.473

#### 9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  | List of cells to be activated | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - | - |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | - | - |
| >>Available PLMN List |  | 0.. 1 |  | List of available PLMNs | YES | reject |
| >>>Available PLMN List Item |  | 1.. <maxnoofBPLMNs> |  |  | EACH | reject |
| >>>>PLMN ID | M |  | 9.3.1.14 |  | - | - |
| >>>>PLMN Status | M |  | 9.3.1.x |  | - | - |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |

Start of Text Proposal 3 for TS 38.473

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - | - |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU |  |  |
| >>Available PLMN List |  | 0.. 1 |  | List of available PLMNs | YES | reject |
| >>>Available PLMN List Item |  | 1.. <maxnoofBPLMNs> |  |  | EACH | reject |
| >>>>PLMN ID | M |  | 9.3.1.14 |  | - | - |
| >>>>PLMN Status | M |  | 9.3.1.x |  | - | - |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | YES | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | YES | ignore |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | YES | ignore |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | YES | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | YES | ignore |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | YES | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | YES | ignore |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | YES | ignore |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - | - |
| >> Cell Barred | M |  | ENUMERATED (barred, not-barred, …) |  |  |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 0 .. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | YES | reject |
| >>List of E-UTRA Cells |  | 0 .. <maxCellineNB> |  | List of applicable E-UTRA cells. | YES | reject |
| >>>EUTRA Cell ID | M |  | OCTET STRING | Indicates the E-UTRAN Cell Global Identifier as defined in subclause 9.2.14 in TS 36.423 [9]. | – | – |
| >>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum nunmerbs of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-CU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |

Start of Text Proposal 4 for TS 38.473

#### 9.3.1.x	PLMN Status

This information element indicates whether the PLMN in question is available

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Status | M |  | ENUMERATED (available, not available, …) |  |

Start of Text Proposal 5 for TS 38.473

### 9.4.5	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-Available-PLMN-Item,

id-Available-PLMN-List,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes

Start of Text Proposal 6 for TS 38.473

-- A

Active-Cells-Item ::= SEQUENCE {

nRCGI		NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Active-Cells-ItemExtIEs } }	OPTIONAL,

...

}

Active-Cells-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow  ::= INTEGER (0..63) -- this IE may need to be refined

Available-PLMN-List	::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ProtocolIE-SingleContainer { { Available-PLMN-ItemIEs} }

Available-PLMN-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Available-PLMN-Item					CRITICALITY ignore	TYPE Available-PLMN-Item			PRESENCE mandatory	},

...

}

Available-PLMN-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

pLMN-Status					PLMN-Status,

iE-Extensions				ProtocolExtensionContainer { { Available-PLMN-ExtIEs} } OPTIONAL,

...

}

Available-PLMN-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Start of Text Proposal 7 for TS 38.473

Cells-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

nRPCI		NRPCI		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Activated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-gNB-CUSystemInformation	CRITICALITY reject	EXTENSION GNB-CUSystemInformation		PRESENCE optional }|

{ ID id-Available-PLMN-List		CRITICALITY reject	EXTENSION Available-PLMN-List			PRESENCE optional },

...

}

Cells-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Deactivated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Start of Text Proposal 8 for TS 38.473

-- P

PacketDelayBudget ::= INTEGER (0..63) -- this IE may need to be refined

PacketErrorRate ::= INTEGER (0..63) -- this IE may need to be refined

PagingCell-Item ::= SEQUENCE {

nRCGI		NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { PagingCell-ItemExtIEs } }	OPTIONAL

}

PagingCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PagingDRX ::= INTEGER (0..63) -- this IE may need to be refined

PagingIdentity ::=	CHOICE {

rANUEPagingIdentity	RANUEPagingIdentity,

cNUEPagingIdentity	CNUEPagingIdentity,

choice-extension			ProtocolExtensionContainer { { PagingIdentity-ExtIEs } },

...

}

PagingIdentity-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PagingPriority ::= ENUMERATED { priolevel1, priolevel2, priolevel3, priolevel4, priolevel5, priolevel6, priolevel7, priolevel8,...}

PLMN-Identity ::= OCTET STRING (SIZE(3))

PLMN-Status :: ENUMERATED {

available,

not-available,

...

}

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

Potential-SpCell-Item ::= SEQUENCE {

potential-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Potential-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Potential-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PWS-Failed-NR-CGI-Item ::= SEQUENCE {

nRCGI				NRCGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { { PWS-Failed-NR-CGI-ItemExtIEs } }	OPTIONAL,

...

}

PWS-Failed-NR-CGI-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PWSSystemInformation ::= OCTET STRING

Start of Text Proposal 9 for TS 38.473

### 9.4.7	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel									ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 16

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB								INTEGER ::= 256

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionStopIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCRconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Active-Cells-Item								ProtocolIE-ID ::= 88

id-Active-Cells-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI										ProtocolIE-ID ::= 95

id-SpCellULConfigured							ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest					ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse					ProtocolIE-ID ::= 98

id-DRB-Activity-Item							ProtocolIE-ID ::= 99

id-DRB-Activity-List							ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container 		ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container 	ProtocolIE-ID ::= 102

id-SpectrumSharingGroupID								ProtocolIE-ID ::= 103

id-ListofEUTRACellsinGNBDUCoordination					ProtocolIE-ID ::= 104

id-Protected-EUTRA-Resources-List						ProtocolIE-ID ::= 105

id-RequestType 											ProtocolIE-ID ::= 106

id-ServCellndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation				ProtocolIE-ID ::= 108

id-ExecuteDuplication							ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item								ProtocolIE-ID ::= 112

id-PagingCell-List								ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 								ProtocolIE-ID ::= 115

id-SIBtype-List									ProtocolIE-ID ::= 116

id-UEIdentityIndexValue							ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation						ProtocolIE-ID ::= 118

id-HandoverPreparationInformation				ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item			ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List			ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV									ProtocolIE-ID ::= 126

id-PagingIdentity								ProtocolIE-ID ::= 127

id-DUtoCURRCContainer							ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List						ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item						ProtocolIE-ID ::= 130

id-TAISliceSupportList							ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List	ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item	ProtocolIE-ID ::= 135

id-DRB-Notify-Item								ProtocolIE-ID ::= 136

id-DRB-Notify-List								ProtocolIE-ID ::= 137

id-NotficationControl							ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation							ProtocolIE-ID ::= 140

id-RepetitionPeriod								ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest						ProtocolIE-ID ::= 142

id-ConcurrentWarningMessageIndicator				ProtocolIE-ID ::= 143

id-Cells-To-Be-Broadcast-List					ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item					ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 				ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 				ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 				ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 				ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 				ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 				ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 						ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 						ProtocolIE-ID ::= 155

id-ConfirmedUEID								ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator		ProtocolIE-ID ::= 157

id-DRXConfigurationIndicator					ProtocolIE-ID ::= 159

id-ServingPLMN									ProtocolIE-ID ::= 164

id-Available-PLMN-Item							ProtocolIE-ID ::= xxx

id-Available-PLMN-List							ProtocolIE-ID ::= xxx

END

End of Text Proposal 9 for TS 38.473