3GPP TSG RAN WG3 #101	R3-185057

Gothenburg, Sweden, 20th – 24th August 2018

Agenda Item:	31.3.5

Source:	Ericsson

Title:	Rapporteur Update for BL CR for TS 38.463

Document for:	TP for BL CR for TS 38.463

# Introduction

This document presents the proposed rapporteur’s updates for the BL CR for TS 38.463.

# Discussion

The following ASN.1 changes are proposed:

- Choice extension:

  - Based on the agreements taken at RAN3-AH-1807, the choice extensions have been introduced throughout the specification as illustrated in R3-184244.

- GLOBAL vs EACH:

  - Based on the agreements taken at RAN3-AH-1807, the tables and ASN.1 structures for ALL the “Lists” have been updated to reflect a GLOBAL structure. The criticality is assigned to the “List”, while all the elements inside the “List” are pure ASN.1 without criticality.

  - NOTE: the only exception is for the “UE-associated logical E1-connection list” in the Reset message, for which an EACH structure is still used for alignment with other interfaces. If needed, other exception (e.g., TNLAs to add /remove list) may be considered.

- Overlapping lists in Bearer Context Setup / Modification procedures:

  - Some lists are used in both the Bearer Context Setup message and the Bearer Context Modification messages (e.g., DRB-To-Setup-List-EUTRAN). For future proofness, new lists have been defined in ASN.1 (e.g., DRB-To-Setup-Mod-List-EUTRAN), so that in the future it will be possible to add/remove IEs in a message without affecting the other message.

The following changes are proposed in the procedural text:

- Section 3.2: added a list of abbreviations.

- Section 8.2.3-4: updated the description of the E1 Setup procedure to reflect the fact that multiple SCTP associations are now supported over the E1. The text has been harmonized with F1 spec.

- Section 8.2.5: editorial correction on the name of the procedure.

- Section 8.3.3: editorial corrections (changed CU-UP to CU-CP).

NOTE: all changes are shown as marked as “Rapp-RAN3-101”. This is done to be able to easily distinguish them from the changes from RAN3-AH-18-07.

NOTE: about gNB-CU-UP Cell Group Related Information IE

- In the way-forward R3-184299 that was discussed at RAN3-AH-1807, it was suggested to remove the gNB-CU-UP Cell Group Related Information IE and replace it with the UP Parameters IE in the Bearer Context Modification Required message. Specifically, the proposal was to: (1) include the UL Configuration in the UP Parameters IE and (2) remove the gNB-CU-UP Cell Group Related Information IE. After offline discussion, the proponents seem to have withdrawn the original concerns that led to proposing this change. Therefore, the Rapporteur suggests keeping the current IE definitions.