3GPP TSG-RAN WG3#101	R3-184437

Gothenburg, Sweden, 20 - 24 August 2018

Agenda item:	31.3.4.12

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR BL CR for TS 38.413): Correction of Path Switch Failure

Document for:	Discussion and Decision

# 1	Introduction

At last RAN3#NR AH1807 tdoc [3] was agreed to fix the transparency of SM-containers in NGAP.

However, a couple of points remained open. This paper deals with the open point on Path Switch Failure message.

# 2	Description and proposal

In order to determine the best encoding of the Path Switch Failure message, it is useful to first recall the decision for encoding of the Path Switch Request / Path Switch Request Acknowledge taken at last RAN3#NR AH1807.

The Path Switch Request now includes each and every active PDU session and will be handled by the SMF per PDU session. The SMF therefore generates as many NGAP containers as PDU sessions. Either:

- The Path is successfully switched and the SMF will generate a container which can either be empty or include an UL tunnel endpoint if UL TEID has changed or may include security information.

- The path could not be switched and the SMF will generate a container which contains a cause value.

When receiving the different containers, AMF just needs to bundle them together. This coding which has been decided with CT4 last time has the following advantages:

- The AMF forwards transparently the information sent from the different SMF nodes,

- Each SMF can potentially fail for a different cause: the NG-RAN node can detect if the SMF had e.g a software error.

Each SMF is unaware of the decision made by the other SMF and therefore will behave the same, even in the case where all the paths happen to fail to switch.

Consequently, when all the paths fail to switch the AMF receives as many containers as PDU sessions, each of them including their own cause of failure.

If the AMF strips out all containers before replying to the NG-RAN using a simple Path Switch Failure message without containers, this implicitly means that:

- The AMF has a new behaviour which is different from what it does when at least one path is successful,

- The AMF has extra work because it needs to generate itself a global cause value i.e. factorizing the various causes received into one overall cause value. This also removes information compared to what the SMF nodes sent.

There seems to be no good reason to introduce a new behaviour which does not bring added value but even could be seen as removing value.

We therefore propose the simple approach:

Proposal: agree to build the Path Switch Failure message with full AMF transparency by including the containers received from the different SMFs.

This is also what CT4 has asked for and what RAN3 has committed in [4].

# 3	Text Proposal for TS 38.413

#### 9.2.3.10	PATH SWITCH REQUEST FAILURE

This message is sent by the AMF to inform the NG-RAN node that a failure has occurred in the 5GC during the Path Switch Request procedure.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| PDU Session Resource Released List |  | 1 |  |  | YES | ignore |
| >PDU Session Resource Released Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Unsuccessful Transfer | M |  | OCTET STRING | Containing the Path Switch Request Unsuccessful Transfer IE specified in subclause 9.3.4.20. | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

NGAP part not modified

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformation,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EUTRA-CGI,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverCommandTransfer,

HandoverPreparationUnsuccessfulTransfer,

HandoverRequiredTransfer,

HandoverRequestAcknowledgeTransfer,

HandoverResourceAllocationUnsuccessfulTransfer,

HandoverType,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

LocationReportingRequestType,

MaskedIMEISV,

MessageIdentifier,

MobilityRestrictionList,

NAS-PDU,

NewSecurityContextInd,

NGRAN-CGI,

NGRANTraceID,

NR-CGI,

NRPPa-PDU,

NumberOfBroadcastsRequested,

PagingDRX,

PagingOrigin,

PagingPriority,

PathSwitchRequestAcknowledgeTransfer,

PathSwitchRequestSetupFailedTransfer,

PathSwitchRequestTransfer,

PathSwitchRequestUnsuccessfulTransfer,

PDUSessionID,

PDUSessionResourceModifyConfirmTransfer,

PDUSessionResourceModifyIndicationTransfer,

PDUSessionResourceModifyRequestTransfer,

PDUSessionResourceModifyResponseTransfer,

PDUSessionResourceModifyUnsuccessfulTransfer,

PDUSessionResourceNotifyTransfer,

PDUSessionResourceNotifyReleasedTransfer,

PDUSessionResourceReleaseCommandTransfer,

PDUSessionResourceSetupRequestTransfer,

PDUSessionResourceSetupResponseTransfer,

PDUSessionResourceSetupUnsuccessfulTransfer,

PLMNSupportList,

PWSFailedCellIDList,

RANNodeName,

RANPagingPriority,

RANStatusTransfer-TransparentContainer,

RAN-UE-NGAP-ID,

RelativeAMFCapacity,

RepetitionPeriod,

RoutingID,

RRCEstablishmentCause,

RRCInactiveTransitionReportRequest,

RRCState,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-TransparentContainer,

SupportedTAList,

TAI,

TAIListForRestart,

TargetID,

TargetToSource-TransparentContainer,

TimeStamp,

TimeToWait,

TNLAssociationList,

TNLAssociationUsage,

TNLAssociationWeightFactor,

TraceActivation,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-associatedLogicalNG-ConnectionItem,

UEContextRequest,

UEIdentityIndexValue,

UE-NGAP-IDs,

UEPagingIdentity,

UEPresenceInAreaOfInterestList,

UERadioCapability,

UERadioCapabilityForPaging,

UESecurityCapabilities,

UnavailableGUAMIList,

UserLocationInformation,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupItem,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddItem,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveItem,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateItem,

id-AMF-TNLAssociationToUpdateList,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformation,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-EUTRA-CGI,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

id-HandoverType,

id-IMSVoiceSupportIndicator,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-LocationReportingRequestType,

id-MaskedIMEISV,

id-MessageIdentifier,

id-MobilityRestrictionList,

id-NAS-PDU,

id-NASC,

id-NewAMF-UE-NGAP-ID,

id-NewSecurityContextInd,

id-NGAP-Message,

id-NGRAN-CGI,

id-NGRANTraceID,

id-NR-CGI,

id-NRPPa-PDU,

id-NumberOfBroadcastsRequested,

id-OldAMF,

id-PagingDRX,

id-PagingOrigin,

id-PagingPriority,

id-PDUSessionResourceAdmittedItem,

id-PDUSessionResourceAdmittedList,

id-PDUSessionResourceFailedToModifyItemModRes,

id-PDUSessionResourceFailedToModifyListModRes,

id-PDUSessionResourceFailedToSetupItemCxtRes,

id-PDUSessionResourceFailedToSetupItemHOAck,

id-PDUSessionResourceFailedToSetupItemPSReq,

id-PDUSessionResourceFailedToSetupItemSURes,

id-PDUSessionResourceFailedToSetupListCxtRes,

id-PDUSessionResourceFailedToSetupListHOAck,

id-PDUSessionResourceFailedToSetupListPSReq,

id-PDUSessionResourceFailedToSetupListSURes,

id-PDUSessionResourceHandoverItem,

id-PDUSessionResourceHandoverList,

id-PDUSessionResourceItemHORqd,

id-PDUSessionResourceListHORqd,

id-PDUSessionResourceModifyItemModCfm,

id-PDUSessionResourceModifyItemModInd,

id-PDUSessionResourceModifyItemModReq,

id-PDUSessionResourceModifyItemModRes,

id-PDUSessionResourceModifyListModCfm,

id-PDUSessionResourceModifyListModInd,

id-PDUSessionResourceModifyListModReq,

id-PDUSessionResourceModifyListModRes,

id-PDUSessionResourceNotifyItem,

id-PDUSessionResourceNotifyList,

id-PDUSessionResourceReleasedItemNot,

id-PDUSessionResourceReleasedItemPSAck,

id-PDUSessionResourceReleasedItemPSFail,

id-PDUSessionResourceReleasedListNot,

id-PDUSessionResourceReleasedListPSAck,

id-PDUSessionResourceReleasedListPSFail,

id-PDUSessionResourceSetupItemCxtReq,

id-PDUSessionResourceSetupItemCxtRes,

id-PDUSessionResourceSetupItemHOReq,

id-PDUSessionResourceSetupItemSUReq,

id-PDUSessionResourceSetupItemSURes,

id-PDUSessionResourceSetupListCxtReq,

id-PDUSessionResourceSetupListCxtRes,

id-PDUSessionResourceSetupListHOReq,

id-PDUSessionResourceSetupListSUReq,

id-PDUSessionResourceSetupListSURes,

id-PDUSessionResourceSwitchedItem,

id-PDUSessionResourceSwitchedList,

id-PDUSessionResourceToBeSwitchedDLItem,

id-PDUSessionResourceToBeSwitchedDLList,

id-PDUSessionResourceToReleaseItemHOCmd,

id-PDUSessionResourceToReleaseItemRelCmd,

id-PDUSessionResourceToReleaseListHOCmd,

id-PDUSessionResourceToReleaseListRelCmd,

id-PLMNSupportList,

id-PWSFailedCellIDList,

id-RANNodeName,

id-RANPagingPriority,

id-RANStatusTransfer-TransparentContainer,

id-RAN-UE-NGAP-ID,

id-RelativeAMFCapacity,

id-RepetitionPeriod,

id-ResetType,

id-RoutingID,

id-RRCEstablishmentCause,

id-RRCInactiveTransitionReportRequest,

id-RRCState,

id-SecurityContext,

id-SecurityKey,

id-SerialNumber,

id-ServedGUAMIList,

id-SliceSupportList,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-TransparentContainer,

id-SupportedTAList,

id-TAI,

id-TAIItem,

id-TAIList,

id-TAIListForRestart,

id-TargetID,

id-TargetToSource-TransparentContainer,

id-TimeStamp,

id-TimeToWait,

id-TraceActivation,

id-TraceCollectionEntityIPAddress,

id-UEAggregateMaximumBitRate,

id-UE-associatedLogicalNG-ConnectionItem,

id-UE-associatedLogicalNG-ConnectionListResAck,

id-UEContextRequest,

id-UEIdentityIndexValue,

id-UE-NGAP-IDs,

id-UEPagingIdentity,

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapabilityForPaging,

id-UESecurityCapabilities,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

maxnoofNGConnectionsToReset,

maxnoofTAIs,

maxnoofTNLAssociations,

maxnoofPDUSessions

FROM NGAP-Constants;

Asn1 not modified

-- **************************************************************

--

##### -- PATH SWITCH REQUEST FAILURE

--

-- **************************************************************

PathSwitchRequestFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestFailureIEs} },

...

}

PathSwitchRequestFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListPSFail		CRITICALITY ignore	TYPE PDUSessionResourceReleasedListPSFail				PRESENCE mandatory	}|,

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

PDUSessionResourceReleasedListPSFail ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF ProtocolIE-SingleContainer { {PDUSessionResourceReleasedItemPSFailIEs} }

PDUSessionResourceReleasedItemPSFailIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourceReleasedItemPSFail	 	CRITICALITY ignore 	TYPE PDUSessionResourceReleasedItemPSFail		PRESENCE mandatory	},

...

}

PDUSessionResourceReleasedItemPSFail ::= SEQUENCE {

pDUSessionID											PDUSessionID,

pathSwitchRequestUnsuccessfulTransfer				OCTET STRING (CONTAINING PathSwitchRequestUnsuccesfulTransfer),

iE-Extensions										ProtocolExtensionContainer { {PDUSessionResourceReleasedItemPSFail-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemPSFail-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

Asn1 not modified

id-UserLocationInformation								ProtocolIE-ID ::= 130

id-WarningAreaList										ProtocolIE-ID ::= 131

id-WarningMessageContents								ProtocolIE-ID ::= 132

id-WarningSecurityInfo									ProtocolIE-ID ::= 133

id-WarningType											ProtocolIE-ID ::= 134

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 135

id-RRCState												ProtocolIE-ID ::= 136

id-PDUSessionResourceFailedToModifyItemModRes			ProtocolIE-ID ::= 137

id-PDUSessionResourceFailedToSetupItemCxtRes			ProtocolIE-ID ::= 138

id-PDUSessionResourceFailedToSetupItemHOAck				ProtocolIE-ID ::= 139

id-PDUSessionResourceFailedToSetupItemPSReq				ProtocolIE-ID ::= 140

id-PDUSessionResourceFailedToSetupItemSURes				ProtocolIE-ID ::= 141

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 142

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 143

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 144

id-PDUSessionResourceReleasedItemNot					ProtocolIE-ID ::= 145

id-PDUSessionResourceReleasedItemPSAck					ProtocolIE-ID ::= 146

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 147

id-PDUSessionResourceToReleaseItemHOCmd					ProtocolIE-ID ::= 148

id-PDUSessionResourceToReleaseItemRelCmd				ProtocolIE-ID ::= 149

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 150

id-PDUSessionResourceReleasedItemPSFail					ProtocolIE-ID ::= 1xx

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 1xx

END

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-184281, Clean-up of SMF IEs

- R3-183432, Reply LS on NGAP SMF IEs