3GPP TSG-RAN WG3 Meeting #101	     R3-184736

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item:	10.8.5

Source:	Ericsson

Title:	(TP for NR BL CR for TS 38.420) Update to XnAP list of functions

Document for:	pCR for TS 38.420

# Introduction

At the RAN#80 plenary, the X2 signalling for E-UTRA - NR Cell Resource Coordination procedure was included in the TS 36.423. In order to enable E-UTRA – NR spectrum sharing between nodes connected to the 5GC, Xn signalling support is required. The TP to TS 38.423, presented in R3-184735 proposes the Xn signalling solution for E-UTRA – NR Cell Resource Coordination, while this TP proposes to add E-UTRA – NR spectrum sharing function to the list of Xn interface functions in TS 38.420.

# TP for TS 38.420

------------------------------------------Change 1-------------------------------------------

## 5.2	Functions of Xn-C

<Skipping Unchanged Text>

### 5.2.x	E-UTRA – NR spectrum sharing function

This function allows uplink and downlink spectrum sharing between a number of E - UTRA and a number of NR cells with overlapping coverage. This function is implemented by means of E-UTRA – NR Cell Resource Coordination procedure.

<Skipping Unchanged Text>

------------------------------------------Change 2-------------------------------------------

### 6.2.2	Dual Connectivity procedures

The dual connectivity procedures are used to add, modify and releases resources for the operation of Dual Connectivity:

-	S-NG-RAN-node Addition Preparation

-	S-NG-RAN-node Reconfiguration Completion

-	M-NG-RAN-node initiated S-NG-RAN-node Modification Preparation

-	S-NG-RAN-node initiated S-NG-RAN-node Modification

-	M-NG-RAN-node initiated S-NG-RAN-node Release

-	S-NG-RAN-node initiated S-NG-RAN-node Release

-	S-NG-RAN-node Counter Check

-	RRC Transfer

-	E-UTRA – NR Cell Resource Coordination

------------------------------------------End of changes-------------------------------------------