3GPP TSG-RAN WG3 #101	Tdoc R3-185021

Gothenburg, Sweden, 20th – 24th August

Agenda Item:	31.3.4.8

Source:	Ericsson

Title:	NAS Non-delivery in higher layer split deployments

Document for:	Discussion

# 1	Introduction

Section 8.6.4.1 and section 8.6.4.2 in NGAP [1] contains the requirements:

“The NAS Non Delivery Indication procedure is used when the NG-RAN node decides not to start the delivery of a NAS message that has been received over a UE-associated logical NG-connection or the NG-RAN node is unable to ensure that the message has been received by the UE.”

and

“The NG-RAN node initiates the procedure by sending a NAS NON DELIVERY INDICATION message to the AMF. The NG-RAN node shall report the non-delivery of a NAS message by including the non-delivered NAS message within the NAS-PDU IE and an appropriate cause value within the Cause IE, e.g., “NG intra system handover triggered”, “NG inter system handover triggered” or “Xn handover triggered”.” respectively.

The parts of the requirements which are addressed in this document are:

R1: The NAS Non Delivery Indication procedure is used when the NG-RAN node is unable to ensure that the message has been received by the UE.

R2: The NG-RAN node shall report the non-delivery of a NAS message by including the non-delivered NAS message within the NAS-PDU IE.

# 2	Discussion

2.1 Background

Messages received in the DOWNLINK NAS TRANSPORT messages are sent to the UE using acknowledged (AM) mode. In a higher layer split deployment, where a gNB consists of a gNB-CU and one or more gNB-DU(s) connected via F1 interface, the protocol stack in the gNB-DU is PHY/MAC/RLC and in the gNB-CU is PDCP/RRC. The services provided by RLC to its upper layer (PDCP) for AM mode are described in section 4.3.1 in [2]:

S1: “AM data transfer, including indication of successful delivery of upper layers PDUs.”

To fulfil requirement R1 the gNB-CU needs to understand if an RRC message containing a NAS PDU has been delivered to the UE. The service S1 provided by RLC enables the gNB-DU to understand when a message has been delivered but there is currently no support for sending this information from the gNB-DU to the gNB-CU in the F1AP [3].

Conclusion 1: To fulfil the requirements R1 and R2 a mechanism indicating successful delivery of upper layer PDUs to the gNB-CU is needed in F1AP.

A solution solving R1 and R2 is presented in the sections below.

2.2 Solution description

For each RRC message the gNB-CU needs acknowledged, the gNB-CU could store an identifier RRC message ID uniquely identifying the RRC message and associated information such as (implementation specific):

- the SRB on which the NAS-PDU is sent,

- an action to be taken if the message is not delivered to the UE

- Any data needed to perform the action (e.g. a NAS PDU).

2.3 Impact F1AP

The CU needs to understand which RRC messages have not been delivered when the gNB-DU no longer serves the UE. This would be applicable in the following scenarios:

- Radio link failure

- Mobility

- UE release

The inclusion of an RRC message ID in a message sent from the gNB-CU to the gNB-DU also indicates that a delivery report is needed.

Proposal 1: Include an RRC message ID which identifies the RRC message in the RRC-Container

Proposal 2: The presence of an RRC message ID indicates that a delivery report shall be sent to the gNB-CU from the gNB-DU when the content of the RRC-Container has been delivered to the UE.

Proposal 3: Introduce a new message sent from the gNB-DU to the gNB-CU reporting successful delivery of an RRC message when requested by the gNB-CU.

# 3	Conclusions and proposals

Because of Conclusion 1: “To meet the requirements R1 and R2 a mechanism indicating successful delivery of upper layer PDUs to the gNB-CU is needed in F1AP” the following proposals are made which are implemented below:

Proposal 1: Include an RRC message ID which identifies the RRC message in the RRC-Container

Proposal 2: The presence of an RRC message ID indicates that a delivery report shall be sent to the gNB-CU from the gNB-DU when the content of the RRC-Container has been delivered to the UE.

Proposal 3: Introduce a new message sent from the gNB-DU to the gNB-CU reporting successful delivery of an RRC message when requested by the gNB-CU.

The CR covering the above proposals is provided in R3-184952

# 4	References

[1]	TS38.413; NG-RAN; NG Application Protocol (NGAP) v1.0.0

[2]	TS38.322; Radio Link Control (RLC) protocol specification v15.1.0

[3]	TS38.473; NG-RAN; F1 Application Protocol (F1AP) v15.2.1

[4]	TS38.323; NR; Packet Data Convergence Protocol (PDCP) specification v15.1.0