3GPP TSG-RAN WG3 Meeting #101	R3-185172

# Gotherburg, Sweden, 20th- 24th August  2018

Agenda Item:	31.3.4.3

Source:	Ericsson

Title: 	Summary of offline discussion on UE AMBR

Document for:	Discussion and Approval.

# 1	Summary

CB: # 45_UE-AMBR

-  When this is not sent to AMF from within the CN, what can AMF send to NG-RAN? (is this a relevant scenario?)

- What happens if NG-RAN does not receive it?

AMF retrieves the subscribed UE Aggregate Maximum Bit Rate from UDM and sends it to NG-RAN node to allow the

traffic policing.

The subscribed UE AMBR might not be available from UDM. It is optional present in UDM.

We still need to check what it means “optional present in UDM”, is it optional in the database, or it is only optional in

some interface.

Some companies think AMF in one way or another can figure out the UE-AMBR value and always send it to NG-RAN node. Thus the IE is mandatory present in NGAP Initial Context Setup (e.g.) as in the current spec.

Some companies think that in the case that the subscribed UE AMBR is absent in UDM and there is no other means to get the value, AMF is allowed to not to send this IE. Thus the IE is optional present in NGAP Initial Context Setup (e.g.). In this case, it is up to NG-RAN implementation how to handle the traffic policing.

There is no conclusion at this meeting.