3GPP TSG-RAN WG3#101	R3-185242

Gothenburg, Sweden, 20 - 24 August 2018

Agenda item:	31.3.4.12

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR BL CR for TS 38.413): Correction of Path Switch Failure

Document for:	Discussion and Decision

# 1	Introduction

At last RAN3#NR AH1807 tdoc [3] was agreed to fix the transparency of SM-containers in NGAP.

However, a couple of points remained open. This paper deals with the open point on Path Switch Failure message.

# 2	Description and proposal

In order to determine the best encoding of the Path Switch Failure message, it is useful to first recall the decision for encoding of the Path Switch Request / Path Switch Request Acknowledge taken at last RAN3#NR AH1807.

The Path Switch Request now includes each and every active PDU session and will be handled by the SMF per PDU session. The SMF therefore generates as many NGAP containers as PDU sessions. Either:

- The Path is successfully switched and the SMF will generate a container which can either be empty or include an UL tunnel endpoint if UL TEID has changed or may include security information.

- The path could not be switched and the SMF will generate a container which contains a cause value.

When receiving the different containers, AMF just needs to bundle them together. This coding which has been decided with CT4 last time has the following advantages:

- The AMF forwards transparently the information sent from the different SMF nodes,

- Each SMF can potentially fail for a different cause: the NG-RAN node can detect if the SMF had e.g a software error.

Each SMF is unaware of the decision made by the other SMF and therefore will behave the same, even in the case where all the paths happen to fail to switch.

Consequently, when all the paths fail to switch the AMF receives as many containers as PDU sessions, each of them including their own cause of failure.

If the AMF strips out all containers before replying to the NG-RAN using a simple Path Switch Failure message without containers, this implicitly means that:

- The AMF has a new behaviour which is different from what it does when at least one path is successful,

- The AMF has extra work because it needs to generate itself a global cause value i.e. factorizing the various causes received into one overall cause value. This also removes information compared to what the SMF nodes sent.

There seems to be no good reason to introduce a new behaviour which does not bring added value but even could be seen as removing value.

We therefore propose the simple approach:

Proposal: agree to build the Path Switch Failure message with full AMF transparency by including the containers received from the different SMFs.

This is also what CT4 has asked for and what RAN3 has committed in [4].

# 3	Text Proposal for TS 38.413

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to request the switch of a downlink GTP tunnel towards a new GTP tunnel endpoint.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF.

After all necessary updates including the UP path switch have been successfully completed in the 5GC for at least one of the PDU session resources included in the PATH SWITCH REQUEST, the AMF shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the NG-RAN node and the procedure ends.

The list of accepted QoS flows shall be included in the PATH SWITCH REQUEST message within the Path Switch Request Transfer IE. The SMF shall handle this information as specified in TS 23.502 [10].

The list of PDU sessions which failed to be setup, if any, shall be included in the PATH SWITCH REQUEST message. The AMF shall handle this information as specified in TS 23.502 [10].

For each PDU session for which the User Plane Security Information IE is included in the Path Switch Request Transfer IE of the PATH SWITCH REQUEST message, the SMF shall behave as specified in TS 33.501 [13] and may send back the Security Indication IE within the Path Switch Request Acknowledge Transfer IE of the PATH SWITCH REQUEST ACKNOWLEDGE message.

If the Security Indication IE is included within the Path Switch Request Acknowledge Transfer IE of the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall behave as specified in TS 33.501 [13].

If the UL NG-U UP TNL Information IE is included within the Path Switch Request Acknowledge Transfer IE of the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall store this information and use it as the uplink termination point for the user plane data for this PDU session.

If the Core Network Assistance Information IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for e.g. the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and

-	report to the AMF the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state in case the RRC Inactive Transition Report Request IE is set to "subsequent state transition report"; or

-	send one RRC INACTIVE TRANSITION REPORT message but no subsequent messages if the UE is in RRC_CONNECTED state and the RRC Inactive Transition Report Request IE is set to "single RRC connected state report", or

-	send one RRC INACTIVE TRANSITION REPORT message plus one subsequent RRC INACTIVE TRANSITION REPORT message when the RRC state transitions to RRC_CONNECTED state if the UE is in RRC_INACTIVE state and the RRC Inactive Transition Report Request IE is set to "single RRC connected state report", or

-	stop reporting to the AMF the RRC state of the UE in case the RRC Inactive Transition Report Request IE is set to "cancel report".

If the New Security Context Indicator IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall use the information as specified in TS 33.501 [13].

Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message the NG-RAN node shall store the received Security Context IE in the UE context and the NG-RAN node shall use it as specified in TS 33.501 [13].

If the UE Security Capabilities IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall handle it accordingly (TS 33.501 [13]).

If the PDU Session Resource Released List IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall release the corresponding QoS flows and regard the PDU session(s) indicated in the PDU Session Resource Released List IE as being released. The appropriate cause value for each PDU session released is included in the Path Switch Request Unsuccessful Transfer IE contained in the PATH SWITCH REQUEST ACKNOWLEDGE message.

#### 8.4.4.3	Unsuccessful Operation

Figure 8.4.4.3-1: Path switch request: unsuccessful operation

If the 5GC fails to switch the downlink GTP tunnel endpoint towards a new GTP tunnel endpoint for all PDU session resources, the AMF shall send the PATH SWITCH REQUEST FAILURE message to the NG-RAN node.

The NG-RAN node shall release the corresponding QoS flows and regard the PDU session(s) indicated in the PDU Session Resource Released List IE included in the PATH SWITCH REQUEST FAILURE message as being released.

The appropriate cause value for each PDU session released is included in the Path Switch Request Unsuccessful Transfer IE contained in the PATH SWITCH REQUEST FAILURE message.

#### 9.2.3.10	PATH SWITCH REQUEST FAILURE

This message is sent by the AMF to inform the NG-RAN node that a failure has occurred in the 5GC during the Path Switch Request procedure.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| PDU Session Resource Released List |  | 1 |  |  | YES | ignore |
| >PDU Session Resource Released Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Unsuccessful Transfer | M |  | OCTET STRING | Containing the PDU session Path Switch Request Unsuccessful Transfer IE specified in subclause 9.3.4.20. | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

NGAP part not modified

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformation,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EUTRA-CGI,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverCommandTransfer,

HandoverPreparationUnsuccessfulTransfer,

HandoverRequiredTransfer,

HandoverRequestAcknowledgeTransfer,

HandoverResourceAllocationUnsuccessfulTransfer,

HandoverType,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

LocationReportingRequestType,

MaskedIMEISV,

MessageIdentifier,

MobilityRestrictionList,

NAS-PDU,

NewSecurityContextInd,

NGRAN-CGI,

NGRANTraceID,

NR-CGI,

NRPPa-PDU,

NumberOfBroadcastsRequested,

PagingDRX,

PagingOrigin,

PagingPriority,

PathSwitchRequestAcknowledgeTransfer,

PathSwitchRequestSetupFailedTransfer,

PathSwitchRequestTransfer,

PathSwitchRequestUnsuccessfulTransfer,

PDUSessionID,

PDUSessionResourceModifyConfirmTransfer,

PDUSessionResourceModifyIndicationTransfer,

PDUSessionResourceModifyRequestTransfer,

PDUSessionResourceModifyResponseTransfer,

PDUSessionResourceModifyUnsuccessfulTransfer,

PDUSessionResourceNotifyTransfer,

PDUSessionResourceNotifyReleasedTransfer,

PDUSessionResourceReleaseCommandTransfer,

PDUSessionResourceSetupRequestTransfer,

PDUSessionResourceSetupResponseTransfer,

PDUSessionResourceSetupUnsuccessfulTransfer,

PLMNSupportList,

PWSFailedCellIDList,

RANNodeName,

RANPagingPriority,

RANStatusTransfer-TransparentContainer,

RAN-UE-NGAP-ID,

RelativeAMFCapacity,

RepetitionPeriod,

RoutingID,

RRCEstablishmentCause,

RRCInactiveTransitionReportRequest,

RRCState,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-TransparentContainer,

SupportedTAList,

TAI,

TAIListForRestart,

TargetID,

TargetToSource-TransparentContainer,

TimeStamp,

TimeToWait,

TNLAssociationList,

TNLAssociationUsage,

TNLAssociationWeightFactor,

TraceActivation,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-associatedLogicalNG-ConnectionItem,

UEContextRequest,

UEIdentityIndexValue,

UE-NGAP-IDs,

UEPagingIdentity,

UEPresenceInAreaOfInterestList,

UERadioCapability,

UERadioCapabilityForPaging,

UESecurityCapabilities,

UnavailableGUAMIList,

UserLocationInformation,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupItem,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddItem,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveItem,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateItem,

id-AMF-TNLAssociationToUpdateList,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformation,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-EUTRA-CGI,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

id-HandoverType,

id-IMSVoiceSupportIndicator,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-LocationReportingRequestType,

id-MaskedIMEISV,

id-MessageIdentifier,

id-MobilityRestrictionList,

id-NAS-PDU,

id-NASC,

id-NewAMF-UE-NGAP-ID,

id-NewSecurityContextInd,

id-NGAP-Message,

id-NGRAN-CGI,

id-NGRANTraceID,

id-NR-CGI,

id-NRPPa-PDU,

id-NumberOfBroadcastsRequested,

id-OldAMF,

id-PagingDRX,

id-PagingOrigin,

id-PagingPriority,

id-PDUSessionResourceAdmittedItem,

id-PDUSessionResourceAdmittedList,

id-PDUSessionResourceFailedToModifyItemModRes,

id-PDUSessionResourceFailedToModifyListModRes,

id-PDUSessionResourceFailedToSetupItemCxtRes,

id-PDUSessionResourceFailedToSetupItemHOAck,

id-PDUSessionResourceFailedToSetupItemPSReq,

id-PDUSessionResourceFailedToSetupItemSURes,

id-PDUSessionResourceFailedToSetupListCxtRes,

id-PDUSessionResourceFailedToSetupListHOAck,

id-PDUSessionResourceFailedToSetupListPSReq,

id-PDUSessionResourceFailedToSetupListSURes,

id-PDUSessionResourceHandoverItem,

id-PDUSessionResourceHandoverList,

id-PDUSessionResourceItemHORqd,

id-PDUSessionResourceListHORqd,

id-PDUSessionResourceModifyItemModCfm,

id-PDUSessionResourceModifyItemModInd,

id-PDUSessionResourceModifyItemModReq,

id-PDUSessionResourceModifyItemModRes,

id-PDUSessionResourceModifyListModCfm,

id-PDUSessionResourceModifyListModInd,

id-PDUSessionResourceModifyListModReq,

id-PDUSessionResourceModifyListModRes,

id-PDUSessionResourceNotifyItem,

id-PDUSessionResourceNotifyList,

id-PDUSessionResourceReleasedItemNot,

id-PDUSessionResourceReleasedItemPSAck,

id-PDUSessionResourceReleasedItemPSFail,

id-PDUSessionResourceReleasedListNot,

id-PDUSessionResourceReleasedListPSAck,

id-PDUSessionResourceReleasedListPSFail,

id-PDUSessionResourceSetupItemCxtReq,

id-PDUSessionResourceSetupItemCxtRes,

id-PDUSessionResourceSetupItemHOReq,

id-PDUSessionResourceSetupItemSUReq,

id-PDUSessionResourceSetupItemSURes,

id-PDUSessionResourceSetupListCxtReq,

id-PDUSessionResourceSetupListCxtRes,

id-PDUSessionResourceSetupListHOReq,

id-PDUSessionResourceSetupListSUReq,

id-PDUSessionResourceSetupListSURes,

id-PDUSessionResourceSwitchedItem,

id-PDUSessionResourceSwitchedList,

id-PDUSessionResourceToBeSwitchedDLItem,

id-PDUSessionResourceToBeSwitchedDLList,

id-PDUSessionResourceToReleaseItemHOCmd,

id-PDUSessionResourceToReleaseItemRelCmd,

id-PDUSessionResourceToReleaseListHOCmd,

id-PDUSessionResourceToReleaseListRelCmd,

id-PLMNSupportList,

id-PWSFailedCellIDList,

id-RANNodeName,

id-RANPagingPriority,

id-RANStatusTransfer-TransparentContainer,

id-RAN-UE-NGAP-ID,

id-RelativeAMFCapacity,

id-RepetitionPeriod,

id-ResetType,

id-RoutingID,

id-RRCEstablishmentCause,

id-RRCInactiveTransitionReportRequest,

id-RRCState,

id-SecurityContext,

id-SecurityKey,

id-SerialNumber,

id-ServedGUAMIList,

id-SliceSupportList,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-TransparentContainer,

id-SupportedTAList,

id-TAI,

id-TAIItem,

id-TAIList,

id-TAIListForRestart,

id-TargetID,

id-TargetToSource-TransparentContainer,

id-TimeStamp,

id-TimeToWait,

id-TraceActivation,

id-TraceCollectionEntityIPAddress,

id-UEAggregateMaximumBitRate,

id-UE-associatedLogicalNG-ConnectionItem,

id-UE-associatedLogicalNG-ConnectionListResAck,

id-UEContextRequest,

id-UEIdentityIndexValue,

id-UE-NGAP-IDs,

id-UEPagingIdentity,

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapabilityForPaging,

id-UESecurityCapabilities,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

maxnoofNGConnectionsToReset,

maxnoofTAIs,

maxnoofTNLAssociations,

maxnoofPDUSessions

FROM NGAP-Constants;

Asn1 not modified

-- **************************************************************

--

##### -- PATH SWITCH REQUEST FAILURE

--

-- **************************************************************

PathSwitchRequestFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestFailureIEs} },

...

}

PathSwitchRequestFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListPSFail		CRITICALITY ignore	TYPE PDUSessionResourceReleasedListPSFail				PRESENCE mandatory	}|,

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

PDUSessionResourceReleasedListPSFail ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF ProtocolIE-SingleContainer { {PDUSessionResourceReleasedItemPSFailIEs} }

PDUSessionResourceReleasedItemPSFailIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourceReleasedItemPSFail	 	CRITICALITY ignore 	TYPE PDUSessionResourceReleasedItemPSFail		PRESENCE mandatory	},

...

}

PDUSessionResourceReleasedItemPSFail ::= SEQUENCE {

pDUSessionID											PDUSessionID,

pathSwitchRequestUnsuccessfulTransfer				OCTET STRING (CONTAINING PathSwitchRequestUnsuccesfulTransfer),

iE-Extensions										ProtocolExtensionContainer { {PDUSessionResourceReleasedItemPSFail-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemPSFail-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

Asn1 not modified

id-UserLocationInformation								ProtocolIE-ID ::= 130

id-WarningAreaList										ProtocolIE-ID ::= 131

id-WarningMessageContents								ProtocolIE-ID ::= 132

id-WarningSecurityInfo									ProtocolIE-ID ::= 133

id-WarningType											ProtocolIE-ID ::= 134

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 135

id-RRCState												ProtocolIE-ID ::= 136

id-PDUSessionResourceFailedToModifyItemModRes			ProtocolIE-ID ::= 137

id-PDUSessionResourceFailedToSetupItemCxtRes			ProtocolIE-ID ::= 138

id-PDUSessionResourceFailedToSetupItemHOAck				ProtocolIE-ID ::= 139

id-PDUSessionResourceFailedToSetupItemPSReq				ProtocolIE-ID ::= 140

id-PDUSessionResourceFailedToSetupItemSURes				ProtocolIE-ID ::= 141

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 142

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 143

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 144

id-PDUSessionResourceReleasedItemNot					ProtocolIE-ID ::= 145

id-PDUSessionResourceReleasedItemPSAck					ProtocolIE-ID ::= 146

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 147

id-PDUSessionResourceToReleaseItemHOCmd					ProtocolIE-ID ::= 148

id-PDUSessionResourceToReleaseItemRelCmd				ProtocolIE-ID ::= 149

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 150

id-PDUSessionResourceReleasedItemPSFail					ProtocolIE-ID ::= 1xx

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 1xx

END

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-184281, Clean-up of SMF IEs

- R3-183432, Reply LS on NGAP SMF IEs