3GPP TSG-RAN WG3 #101-Bis	Tdoc R3-185920

Chengdu, China, 08th– 12th October 2018

Agenda Item:	31.3.4.8

Source:	Ericsson

Title:	Bearer type change indication

Document for:	Discussion, Decision

# Introduction

In the past RAN2 meetings, bearer type change for EN-DC was extensively discussed. It was proposed in the previous RAN3 meeting to introduce a bearer type change IE in UE CONTEXT MODIFICATION REQUEST message, in order to enable the gNB-CU to inform the gNB-DU of bearer type change with key change and allow the latter to make decision for RLC/MAC layer handling [1]. In this paper, layer 2 handling of bearer type change for gNB-CU/gNB-DU case is discussed.

# Discussion

For EN-DC, when a bearer type change that involves changing the security key is performed (e.g., from MN to SN terminated bearer) and when a key change is performed (e.g., a new KgNB is derived), the gNB-DU needs to either reset MAC/RLC layer or generate new LCH ID(s) for the affected bearer(s). The table below summarizes the different L2 handling:

Table A-1: L2 handling for bearer type change with and without security key change

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note 1 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note 1 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 2 SCG MAC: See Note 2 | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: See Note 2 SCG MAC: See Note 2 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 2 SCG MAC: See Note 2 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 2 SCG MAC: See Note 2 |

NOTE 1:	For MCG, the MAC/RLC behaviour depends on the solution selected by the network. It can be “MAC reset + RLC re-establishment”, or “change of LCID + RLC re-establishment”.

NOTE 2:	For SCG, MAC/RLC behaviour depends on the solution selected by the network. It can be “MAC reset by reconfiguration with Sync + RLC re-establishment”, or “change of LCID + RLC bearer release and add”.

Based on these two options, and since currently only the gNB-CU is aware of the bearer type change or security key change, it was suggested in [1] to introduce a new “bearer type change” IE in the UE CONTEXT MODIFICATION REQUEST message to inform the gNB-DU. In this way, the gNB-DU will make the decision on “MAC reset reconfiguration with Sync+RLC re-establishment” or “change of LCID + RLC bearer release and add”.

However, it is our understanding that every time a security key change occurs, the gNB-CU needs to provide to the gNB-DU a new UL TEID for the bearer(s). Therefore, the reception of new UL TEIDs can be interpreted by the gNB-DU as an implicit indication that the security key for the bearer has changed.

Observation 1	The reception of new UL TIEDs by the gNB-DU everytime a security change occurs can be interpreted by the gNB-DU as an implicit indication that the security key for the bearer has changed.

Observation 2	Even in case that the gNB-CU decides to change the UL TEID (e.g., for changing the IP address) a MAC/RLC layer reset or change of LCH ID is still needed at the gNB-DU.

Based on this, we believe that a new indication is not needed. However, a clarification is needed so as to explain the behaviour of the gNB-DU when receiving new UL TEIDs.

Add the TP in [2] to clarify the behavior of the gNB-DU upon receiving new UL TEIDs.

# Conclusion

In section 2 we made the following observations:

Observation 1	The reception of new UL TIEDs by the gNB-DU everytime a security change occurs can be interpreted by the gNB-DU as an implicit indication that the security key for the bearer has changed.

Observation 2	Even in case that the gNB-CU decides to change the UL TEID (e.g., for changing the IP address) a MAC/RLC layer reset or change of LCH ID is still needed at the DU.

Based on the discussion in section 2 we propose the following:

- Add the TP in [2] to clarify the behavior of the gNB-DU upon receiving new UL TEIDs.

# References

R3-184903, CR to 38.473 on bearer type change indication over F1, Huawei, 3GPP TSG-RAN3 Meeting #101, Gothenburg, Sweden, 20 - 24 August

[2] 	R3-185921, TP for TS 38.473 on Bearer type change indication