3GPP TSG-RAN WG3 Meeting #101-bis 	R3-186139

# Chengdu, P.R.China, 8-12 Oct 2018

Agenda item:	31.3.1.32

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of the off-line discussion on the support of the multiple SCTPs in NG-RAN

Document for:	Discussion

# 1	Introduction

At the meeting, it was discussed if and how multiple SCTPs can be supported on different interfaces. At the online discussion, the following working agreement was made:

WA: We introduce multiple TNLA functionality on Xn and align with F1, E1 NG as feasible (ambition level is to have a set of common functions across multiple interfaces; current NG specifications have a superset of such functions); X2 is FFS

Therefore, in this paper we review the status of the support of multiple SCTP on the NR interfaces, taking the functionality agreed for NG as the example.

# 2	Discussion

The discussion started from the review of the functional status of different interfaces. So far there are two methods used:

- Requesting TNLA addition / removal / modification, including TNLA role

- Sending CONFIG UPDATE from a new TNLA with node ID

Both methods require signalling specification (TS 38.4x3) and usage description (TS 38.4x2).

| Node / interface | Can multiple IP addresses be used? | Method | Status |
| --- | --- | --- | --- |
| AMF / NG | Yes | Adding / Removing / Updating TNLA | Fully supported |
| NGRN / NG | Yes | Sending CONFIG UPDATE with node ID | Agreed, signalling missing (TPs at the meeting: [1], [2]) |
| CU / F1 | Yes | Adding / Removing / Updating TNLA | Fully supported |
| DU / F1 | No | Sending CONFIG UPDATE with node ID | Part of “full package” |
| CP / E1 | Yes | Adding / Removing / Updating TNLA | Fully supported |
| UP / E1 | Yes | Sending CONFIG UPDATE with node ID | Agreed, description missing |
| NGRN / Xn | No | Adding / Removing / Updating TNLA | Agreed, signalling and description missing |

Additional aspects to be discussed in the context of the Xn:

- Triangulation function of UE-associated messages (i.e. a response message in a class-1 procedure may be sent on a different TNLA than the one used for the initiating message), as supported on the NG.

- Disassociation of a TNLA.

- Stage-2, as present for F1 in TS 38.401.

# 3	Conclusion

Way forward:

- To complete the work on the functionalities marked as “Agreed”, where some part is missing (considering the additional functions from NG);

- To discuss further on the parts marked as “full package”.

# 4	References

- R3-185670

- R3-185931

- R3-185930