3GPP TSG RAN WG3 Meeting #102	R3-186452

Spokane, WA, USA, 12th – 16th November 2018

Agenda Item:	31.3.4.10

Source:	Ericsson

Title:	DDDS polling clarification

Document for:	Discussion and agreement

# Introduction

At the RAN3#101 meeting, a solution for triggering a DDDS upon delivery of a specific PDCP PDU was agreed (R3-185072).

This contribution proposes a clarification of the above concept.  The corresponding CR is presented in R3-186453.

# Discussion

The clause 5.4.1.1 of TS 38.425 states the following:

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered.

According to the above text, the node hosting the PDCP entity request the corresponding node to send the DDDS once the NR PDCP PDU with PDCP PDU SN has been delivered. However, according to the TS 38.425, the DDDS can only report according to Highest (or Latest) successfully delivered NR PDCP SN and the requested PDCP PDU SN can only be reported when all “earlier” PDCP PDU SNs has been delivered.

It is therefore necessary to clarify in the specification text that the above means that the DDDS will be sent only when all PDCP PDU SNs up to this specific number are successfully delivered in-sequence. A CR to TS 38.425 reflecting the above discussion is presented in R3-186453.

Proposal: RAN3 to agree the CR to TS 38.425 presented in R3-186453.

# Conclusion

In this contribution it is proposed to further clarify the DDDS polling with respect to a particular PDCP PDU SN. The following proposal is raised:

Proposal: RAN3 to agree the CR to TS 38.425 presented in R3-186453.