3GPP TSG RAN WG3 Meeting #102	R3-186474

Spokane, WA, USA, 12th – 16th November 2018

Agenda Item:	22.3.2

Source:	Ericsson

Title:		TP to TR 38.866: The way forward on RIM backhaul RIM frameworks

Document for:	pCR to TR 38.866

# Introduction

The RIM discussions at the RAN3#101bis meeting resulted in the LS reply to RAN1 (R3-186224), on the feasibility of RIM backhaul-based frameworks.

This contribution proposes a way forward on backhaul-based RIM frameworks and a TP to TR 38.866.

# Backhaul-based RIM frameworks

There are currently two backhaul-based candidate frameworks included in the RIM TR:

- Framework-2.1, where the following messages are sent over the backhaul:

  - An indication from the aggressor to the victim that the RS has been received.

  - An indication from the aggressor to the victim that the RS has disappeared.

- Framework-2.2, where the following messages are sent over the backhaul:

  - An indication from the aggressor to the victim that the RS has been received.

  - An indication from the aggressor to the victim that the RS has disappeared.

  - RIM Assistance Information (RAI) from the victim to the aggressor, to facilitate the RIM.

However, it seems that there is no consensus in RAN3 about the benefits of specifying the Framework-2.2. Namely, at the RAN3#101bis meeting it was argued that the need for sending the RAI from the victim to the aggressor side is unnecessary, for two reasons:

- Unclear benefits: The aggressor may extract the important information necessary for remote interference (RI) mitigation already from the RS. In particular, the aggressor gNB can infer the propagation delay of the RS based on the time of reception of the RS. Based on the propagation delay, the aggressor knows how many DL symbols it should mute, in order to eliminate the RI at the victim side. Which additional information could be sent between victim and aggressor and how it can be used by the aggressor to further improve the network performance in the mitigation scheme applied, is not clear.

- Restricted implementation freedom: Even if there would exist a RAN3 consensus about the benefits of Framework-2.2, agreeing on the exact RAI types to be sent from the victim to the aggressor seems opportunistic and far-fetched. In addition, it is important to note that not specifying the RAI does not preclude a proprietary RAI exchange, while specifying the exact RAI types would heavily restrict the implementation freedom.

Conclusion 1: The aggressor node may be able to extract from the received RS all the information it may need, in order to apply RI mitigation measures.

Conclusion 2: Specifying the RIM Assistance Information types would heavily restrict the implementation freedom.

# The proposed way forward

Having in mind the above, it seems reasonable to specify one generic RIM backhaul-based framework (provided that RAN1 decides to adopt any backhaul-based frameworks). The generic RIM backhaul framework can have the following properties:

- The RIM backhaul consists of a single message type.

- All messages defined within the message type can be sent from any gNB.

- The RIM backhaul message content can be as follows:

  - Source node ID,

  - Destination node ID,

  - RIM-specific information content:

    - 'RS detected',

    - 'RS disappeared'.

Proposal 1: RAN3 to agree on a generic backhaul-based framework enabling the bidirectional exchange of messages carrying the information about detection and disappearance of the RS.

Proposal 2: RAN3 to agree the TP describing the generic framework given in the Annex of this paper.

If the above concepts are agreed, it would be necessary to send an LS informing the RAN1 about the decision to define a single generic backhaul-based RIM framework.

Proposal 3: RAN3 to send an LS informing the RAN1 about the decision to define a single generic backhaul-based RIM framework. The draft LS is presented in R3-186476.

# Conclusion

In this contribution, we discuss the way forward on backhaul-based RIM frameworks. The following conclusions are drawn:

Conclusion 1: The aggressor node may be able to extract from the received RS all the information it may need, in order to apply RI mitigation measures.

Conclusion 2: Specifying the RIM Assistance Information types would heavily restrict the implementation freedom.

Based on the above conclusions, the following proposals were raised:

Proposal 1: RAN3 to agree on a generic backhaul-based framework enabling the bidirectional exchange of messages carrying the information about detection and disappearance of the RS.

Proposal 2: RAN3 to agree the TP describing the generic framework given in the Annex of this paper.

If the above concepts are agreed, it would be necessary to send an LS informing the RAN1 about the decision to define a single generic backhaul-based RIM framework.

Proposal 3: RAN3 to send an LS informing the RAN1 about the decision to define a single generic backhaul-based RIM framework. The draft LS is presented in R3-186476.

# ANNEX: TP to TR 38.866

-------------------------------------------Change 1-------------------------------------------

## 6.x	Conclusions on backhaul-based RIM frameworks

There is no consensus in RAN3 about the benefits of sending the RIM Assistance Information (RAI) from the victim to the aggressor side. RAN3 decided not to proceed with normative work on RAI signalling over the backhaul, for two main reasons:

- Unclear benefits: the aggressor may extract the important information necessary for remote interference (RI) mitigation already from the RS. In particular, the aggressor gNB can infer the propagation delay of the RS based on the time of reception of the RS. Based on the propagation delay, the aggressor knows how many DL symbols it should mute, in order to eliminate the RI at the victim side. Which additional information could be sent between victim and aggressor and how it can be used by the aggressor to further improve the network performance in the mitigation scheme applied, is not clear.

- Restricted implementation freedom: even if there would exist a consensus among RAN3 companies about the benefits of Framework-2.2, agreeing on the exact RAI types to be sent from the victim to the aggressor seems opportunistic and far-fetched. In addition, it is important to note that not specifying the RAI does not prevent the implementation of RAI exchange, while specifying the exact RAI types would heavily restrict the implementation freedom.

RAN3 recommends to start the RIM normative work on a single backhaul-based RIM framework with the following properties:

- The RIM backhaul consists of a single message type.

- All messages defined within the message type can be sent from any gNB.

- The RIM backhaul message content is as follows:

  - Source node ID,

  - Destination node ID,

  - RIM-specific information content:

    - 'RS detected',

    - 'RS disappeared'.

------------------------------------------End of Changes-------------------------------------------