3GPP TSG-RAN W3#102				R3-187031

Spokane, USA, 12 – 16 November, 2018

Agenda Item:	 24.1.1.2

Source: 	 Telstra

Title:	 IAB QoS implementation options and roadmap

Document for:	 Discussion and Decision

# Introduction

The IAB node is a compelling concept for reducing the cost of deployment for 5G networks, particularly contemplating the characteristics of mm-wave spectrum. While there has been limited commercial success for LTE relay products, Telstra has had experience with so-called “UE-relays”. These have a number of limitations, which highlight the need to ensure that QoS is handled adequately for IABs.

In this contribution, we discuss the need for fine-grained QoS support for IABs based on our experience, however we also acknowledge that there may be a need to standardise a simpler solution in the shorter term.

# Experience with LTE relays

LTE relays have had standardised support since R10, but, to the best of our knowledge, the standardised solution has not found commercial success. This has been in large part due to the lack of support by the major infrastructure suppliers. Nevertheless, smaller players have brought so-called “UE-relays” to market, which have provided an adequate, but limited solution for specific niche use cases.

UE-relays act as a node that emulates a UE towards the donor eNode-B, while emulating an eNode-B towards UEs on its service side. The donor eNode-B is not able to differentiate the relay from any other UE and is thus unable to make optimised RRM decisions to suit the requirements of the traffic served by the relay.

This limitation is acceptable for very light traffic cases, typically where the relay is used to provide coverage infill for small coverage holes. The UE relay solution in this context offers an alternative to conventional eNode-Bs for cases that have difficult site acquisition challenges.

However, where the relay or the donor eNode-B is anticipated to experience resource contention, this limitation quickly becomes problematic: the relay is unable to compete with other UEs to secure sufficient resources to serve its own traffic. The operator must then choose between one of two unattractive options:

- accept that resource contention will cause quality issues for traffic served by the relay; or

- define special QCIs dedicated for relay use to carry relay traffic in aggregate back to the donor eNode-B.

Ultimately the operator is unable to provide adequate quality of service to the relay traffic without over-provisioning resources configured for dedicated relay QCIs. Even then, it may not be practical to over-provision the special QCIs sufficiently to assure quality in all traffic conditions.

This issue is immediately apparent when contemplating a simple traffic mix of voice (VoLTE) and best effort data traffic. An oversimplified worked example of such a case is depicted in Figure 1.

Figure 1 Oversimplified worked example

In real networks, however, more sophisticated product sets are in market. For example, Telstra’s 4G network provides QoS support for differentiated traffic handling of voice (VoLTE), video (ViLTE), public safety and low priority services, with future plans for additional applications requiring yet more granular QoS support.

# Discussion

We are aware that different models for the provision of QoS support have been discussed in RAN3 to date. We understand that, broadly speaking, two models are under discussion. These are:

- A model in which IAB traffic of each service type is carried back to the donor on an aggregated pipe, which can be dynamically scaled by negotiation between the IAB and the donor; or

- A model in which finely grained QoS is supported with the donor having full knowledge and enforcement of individual UE QoS and RRM requirements across bearers and IAB hops.

Contemplating our dependency on fine-grained QoS support in current and future products and considering the likely utility of IABs in high traffic areas, it is clear that a model providing fine grained QoS support will be essential. The advent of network slicing and finer-grained QoS that will come with the 5GC will create further opportunity for the creation of even more differentiated product and service offerings.

Operators will require the implementation of a fine-grained QoS model for IABs to support current and future product and service offerings in mature 5G networks

While this long-term requirement for a fine-grained QoS model is clear, we recognise that the timing is not immediate. In fact, it will take time to migrate traffic from 4G to 5G and similarly, it will take time to migrate QoS based products and services from 4G to 5G.

We also recognise that the IAB is likely to be valuable in the early and mid-phases of mm-wave network rollout and so time to market is an important consideration. A lower complexity implementation will likely assist with time to market for first generation IAB products.

With these considerations in mind, we take the view that a simpler model of QoS support using dynamic scaling of aggregated pipes would be an acceptable trade-off to improve IAB time to market.

An aggregated pipe with dynamic scaling may be an acceptable interim solution to encourage shorter time to market for IAB products

Taking a pragmatic view to balance the short-term need for IAB time to market and the long-term need for fine grained QoS support, it is clear that a road-mapped approach is necessary. This approach must have the following characteristics:

- An easy to implement aggregate pipe with dynamic scaling focused on reduced complexity;

- An agreed timeframe to implement the fine-grained QoS solution (we suggest R17); and

- A consistent architectural approach for both implementations that makes it feasible to perform field-upgradable changes to deployed IABs to support the fine-grained QoS solution once it is standardised

QoS support should be implemented in a two-stage approach, in which aggregated pipes with dynamic scaling are supported in R16 and a fine-grained QoS model supported in R17

The two-stage approach to implementing QoS support must be based on a consistent architecture such that IABs can be upgraded in-field to support fine-grained QoS capability when it is standardised in R17

# Conclusion and Proposals

In this contribution we have provided a summary of our experience with LTE relays, which provides a basis for decision making in selecting a QoS support model for the IAB. We make the following observations and proposals:

- Operators will require the implementation of a fine-grained QoS model for IABs to support current and future product and service offerings in mature 5G networks

- An aggregated pipe with dynamic scaling may be an acceptable interim solution to encourage shorter time to market for IAB products

- QoS support should be implemented in a two-stage approach, in which aggregated pipes with dynamic scaling are supported in R16 and a fine-grained QoS model supported in R17

The two-stage approach to implementing QoS support must be based on a consistent architecture such that IABs can be upgraded in-field to support fine-grained QoS capability when it is standardised in R17