3GPP TSG RAN WG3 Meeting #102        			            R3-187200
Chengdu, China, 8-12 October 2018

Agenda item:	23

Source:		Vice-Chairman (Intel Corporation)

Title:		V2X session report

Document for:	agreement

# 1	Agreements

Agreements:

V2X-capable UE shall transmit in the sidelink only if authorized by the serving gNB; this authorization is signaled over NGAP and XnAP.

NR V2X UE authorization should allow at least “vehicle” and “pedestrian” services authorization, not precluding a UE being authorized for both at the same time.

Signaling the UE sidelink AMBR to NG-RAN should be supported in NGAP and XnAP.

Only the following papers have been treated: R3-186423, R3-186424, R3-186428, R3-186833 and R3-186427.

# 2	Comebacks

# V2X_CB_1

- The intention is to have an endorsed TR skeleton with basic description for RAN3 aspects of the study

- Use R3-186424 as the starting point

- Reword text on AMBR and authorization as discussed

- Implement agreements from R3-186833

- Merge “In disaggregated gNBs, the resources for NR V2X sidelink is assigned by the gNB-DU.

- For NR V2X mode 1, the gNB-DU sends the configuration information for V2X sidelink communication to the gNB-CU which forwards these to the UE over RRC message.” from R3-186427 with FFS as appropriate

- Merge “The NR V2X shall support the use of slicing similar to what is done in NG-RAN, i.e. the same general principles as captured in 38.300 shall apply” from R3-186428

- Reword as needed

(HW)

R3-187222

# V2X_CB_2

- Rapporteur to prepare a list of open issues to be discussed in the next meeting

(LG)

R3-187223

# 3	Minutes

| # 23. Study on NR V2X SID [NR_V2X]: RP-182111 (target: RAN #83) [TU: 0.5 (0.5 0.5)] |  |  |
| --- | --- | --- |
| ## 23.1. General Scenarios, time plan, skeletons, BLs Previous summary of offline disc (R3-186225), noted |  |  |
| R3-186423 | TR 38.885 (for information) (Huawei) | discussion noted |
| R3-186424 | Proposed skeleton for RAN3 part of TR 38.885 (Huawei) | discussion E///: reworking on AMBR is needed HW: ack LG: authentication needs to be reworded as authorization # V2X_CB_1 - Rewording as discussed - Implement agreements from 6833 - Merge “In disaggregated gNBs, the resources for NR V2X sidelink is assigned by the gNB-DU. - For NR V2X mode 1, the gNB-DU sends the configuration information for V2X sidelink communication to the gNB-CU which forwards these to the UE over RRC message.” With FFS from 6427 - Merge “The NR V2X shall support the use of slicing similar to what is done in NG-RAN, i.e. the same general principles as captured in 38.300 shall apply” from 6428 - Reword as needed (HW) R3-187222 # V2X_CB_2 - Rapporteur to prepare a list of open issues to be discussed in the next meeting (LG) R3-187223 |
| ## 23.2. Support for Advanced V2X Use Cases Identify enhancements, if any, which are needed Avoid overlaps with other NR and LTE enhancements from Rel-16 WIs/SIs |  |  |
| R3-186422 | Considerations on NR V2X (ZTE Corporation) | discussion |
| R3-186370 | Discussion on NR eV2X (CATT) | discussion |
| R3-186351 | Issues on V2X Service Authorization and UE Sidelink AMBR in NG-RAN (LG Electronics) | discussion |
| R3-186425 | NR V2X Service Authorization (Huawei) | discussion |
| R3-186426 | UE Sidelink AMBR in NG-RAN (Huawei) | discussion |
| R3-186428 | Slicing for V2X (Huawei) | discussion There is no impact on RAN for supporting standardised SST values The general principles for slicing should also apply for NR V2X Merged into the TR 38.885 as part of V2X_CB_1 |
| R3-186587 | Considerations on the issues for MR-DC based NR V2X (LG Electronics) | discussion |
| R3-186833 | Study on V2X (Ericsson) | discussion Move to 23.2 V2X-capable UE shall transmit in the sidelink only if authorized by the serving gNB; this authorization is signaled over NGAP and XnAP. NR V2X UE authorization should allow at least “vehicle” and “pedestrian” services authorization, not precluding a UE being authorized for both at the same time. Signaling the UE sidelink AMBR to NG-RAN should be supported in NGAP and XnAP. WA: Enforcing the use of V2X over Uu vs. PC5 should be possible by the eNB by controlling the transmission grants for the UE, with no network signaling impact; monitor RAN2 progress on this issue. RAN3 should consider the possibility that NR V2X Service Authorization can be per PC5 RAT and coordinate with the other WGs on this solution. RAN3 should consider if additional subscription parameters will be needed for eV2X communication RAN3 should coordinate with SA2 on the potential impacts on RAN and operation interfaces for supporting eV2X use cases Merged into the TR 38.885 as part of V2X_CB_1 |
| F1 ASPECTS |  |  |
| R3-186353 | Issues of F1 interface to support NR sidelink resource allocation mode 1 and mode 2 (LG Electronics) | discussion |
| R3-186427 | F1 signalling for SL resources (Huawei) | Discussion for NR V2X Mode 1, it is proposed gNB-DU to be responsible for the resources configuration and scheduling of sidelink. for NR V2X Mode 1, gNB-DU informs gNB-CU about the configuration information of sidelink and gNB-CU send this to the UE over RRC. For NR V2X Mode 2, it is proposed gNB-DU to be responsible for the resources configuration of sidelink to be broadcasted. For NR V2X Mode 2, it is proposed RAN3 to discuss whether gNB-DU needs to send V2X SL configurtion to gNB-CU in order for it to encode the other SI. LG: we can agree proposals 1 and 2 (for mode 1), mode 2 FFS CATT: agree E///: agree, DU is responsible for SL coordinate for mode 1, mode 2 is too premature (wait for RAN2) QCOM: question configuration isn’t in the CU always? ZTE: we have doubts about DU information CU re resource configuration (for both mode 1 and mode 2) Merge “For NR V2X mode 1, the gNB-DU sends the configuration information for V2X sidelink communication to the gNB-CU which forwards these to the UE over RRC message. “ into the TR Merged into the TR 38.885 as part of V2X_CB_1 |
| ## 23.3. RAT and Interface Selection for Operation Study support for any additional mechanisms (if needed) to select LTE PC5, NR PC5, LTE Uu or NR Uu for operation (in coordination with SA2) NB This is pending RAN2 work, so we should focus on RAN3 aspects only (if any) |  |  |
| ## 23.4. Others |  |  |
| R3-186600 | Consideration on MBMS Service for V2X (LG Electronics) | discussion |

Agenda color coding

10. Agenda Item

10.1. Sub Agenda Item

10.1.1. Sub-sub Agenda Item

##### 10.1.1.1. Sub-sub-sub Agenda Item

TOPIC GROUPING (used to group and highlight a topic, but it is not an Agenda Item)

Chairman’s notes color coding

| R3-xxxxxx | Available but not yet treated document | Chairman notes |
| --- | --- | --- |
| R3-xxxxxx | This document has low priority |  |
| R3-xxxxxx | This document was not available at submission deadline | Chairman notes |
| R3-xxxxxx | This document was treated at this meeting and a decision has been made | Chairman notes |
| R3-xxxxxx | Agreed (or approved) proposal: e.g. Working Assumption, tdoc proposal, etc... | Chairman notes Approved used for Report and Agenda Agreed used for CR to be sent to RAN or LS out p-Approved (partially approved) used for BL CR or TR subject to tdoc allocation by MCC for next meeting Merged used for a tdoc agreed to be merge in a BL CR or TR Endorsed used for CR to be agreed by other WG e.g. TS 36.300 or Draft Status Report (DSR) produced in meeting |
| R3-xxxxxx | Request for ComeBack (CB) during the meeting | Chairman notes a Come Back is required during the meeting CB: # n_FolderName - comments (Company Owner) |
| R3-xxxxxx | Open issue which might require further clarification in next meeting | Chairman notes Comments (no agreement) |
| R3-xxxxxx | E-mail discussion | Chairman notes Email#01 Deadline (Company) |
| R3-xxxxxx | “To be continued” discussion: there was no agreement at this meeting and the discussion may continue at the next meeting | Chairman notes To be continued |
| R3-xxxxxx | Important warning for consideration | Chairman notes Important warning for consideration |