3GPP TSG-RAN WG3 Meeting #102	R3-186911

# Spokane, U.S.A. 12th- 18th November 2018

Agenda Item:	31.3.4.14

Source:	Ericsson

Title: 	Extending GUMMEI Type

Document for:	Discussion and Approval

# 1	Discussion

SA2 has recently agreed the CR S2-188869 to TS 23.501 and S2-188806 is agreed/approved CR to 23.502.

It clarifies that during idle mode mobility from 5GS to EPS, the UE includes a GUMMEI in the RRC connection setup complete message mapped from the AMF identifier part (i.e. GUAMI) of the 5G-GUTI.

According to the SA2 CR, to avoid having to coordinate the CN identities used in 5GS and EPS and the CN identities used in 5GS and 2G/3G, the UE should also indicate that the GUMMEI was mapped from 5G-GUTI.

The gummei-Type field in the RRCConnectionSetupComplete message is extended with a new value mappedFrom5G to indicate that the GUMMEI was mapped from (the GUAMI part of the) 5G-GUTI.

This change should be reflected in TS 36.413 as well.

Proposal 1:   RAN3 to agree to extend the GUMMEI Type in the Initial UE Message with an additional code point.

# 2	Proposal

Proposal 1:   RAN3 to agree to extend the GUMMEI Type in the Initial UE Message with an additional code point.