3GPP TSG RAN WG3 Meeting #102 																R3-187113

Spokane/USA, 12-16 November 2018														revised from R3-186355

Title:	NTN architecture with transparent satellite

Source: 	Thales, ZTE

Type:	pCR

Document for:	Agreement

Agenda Item:	20.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at correcting the clause 5.1 Transparent satellite based NG-RAN architecture.

# Discussion

During the RAN3 meeting in Chengdu, it was identified that the Figure 5.2-2: Transparent-satellite based NG-RAN with mapping to QoS flows

Should be revised to take into account the figure 12.1 in 38.300 (the QoS flows should be inside the PDU session, between a UE and the UPF).

Proposal 1: To revise Figure 5.2-2 taking into account the figure 12.1 of 38.300

# Conclusion

Proposal 1: To revise Figure 5.2-2 taking into account the figure 12.1 of 38.300 (see TP below)

# Text Proposal for TR 38.821 v0.2.0

START OF CHANGES

# 5	NTN-based NG-RAN Architectures

Editor’s note: includes the description of NG-RAN reference architectures supporting the NTN scenarios defined in Clause 4 of this document: Transparent-satellite based NG-RAN architecture, Regenerative-satellite based NG-RAN architecture with gNB on board, Regenerative-satellite based NG-RAN architecture with gNB-DU on board.

Editor’s note: The study should minimize the need to define new interfaces and protocols in the NG-RAN.

## 5.1	Transparent satellite based NG-RAN architecture (FFS)

### 5.1.1	Overview

The satellite payload implements frequency conversion and a Radio Frequency amplifier in both up link and down link direction. It corresponds to an analogue RF repeater.

Hence the satellite repeats the NR-Uu radio interface from the feeder link (between the NTN gateway and the satellite) to the service link (between the satellite and the UE) and vice versa.

The Satellite Radio Interface (SRI) on the feeder link is the NR-Uu. In other words, the satellite does not terminate NR-Uu.

Figure 5.2-1: Networking-RAN architecture with transparent satellite

Note: Whilst several gNBs may access a single satellite payload, the description has been simplified to a unique gNB accessing the satellite payload, without loss of generality.

### 5.1.2	Detailed description of the architecture (FFS)

The architecture of a transparent-satellite based NG-RAN is depicted in the following figure. The mapping to QoS flows is also highlighted.

Figure 5.2-2: Transparent-satellite based NG-RAN with mapping to QoS flows (FFS)

UE has access to the 5G system via a 3GPP NR based radio interface.

The UE user plane protocol stack for a PDU session is described hereafter.

Figure 5.2-3: UE User plane Protocol stack for a PDU session (Transparent satellite)

RF processing & Frequency Switching

The NAS and the PDU layer for the UE are transported towards the 5GC. This is applicable for both directions.

The UE control plane protocol stack for a PDU session is described hereafter.

Figure 5.2-4: UE Control plane Protocol stack for a PDU session (Transparent satellite)

|  | RF processing & Frequency Switching |
| --- | --- |

The NAS (NAS-SM and NAS-MM) signaling for the UE and the NG-AP signaling for the gNB are transported towards the 5GC. This is applicable for both directions.

### 5.1.3	NG-RAN impacts

FFS

END OF CHANGES