3GPP TSG-RAN WG3 Meeting #103	R3-190151

Athens, Greece, EU, 25 Feb – 1 Mar 2019

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	PSCell information for Lawful Interception

# 1	Introduction

RAN3 has received an LS from SA3 LI group asking to provide information about the serving cell at the SN [1]. The LS discusses that the existing information about the serving cell at the MN may not be sufficient for localisation information required to be provided for Lawful Interception purposes. Here, we address this request and suggest needed changes.

# 2	Discussion

The information about the serving cells needs to be provided to the MME or the AMF. In case of MR-DC, the signalling with the CN is provided from the MN. Therefore, the information about the MN and the SN serving cells (PCell and PSCell respectively) shall be provided over S1 or NG interfaces. At this moment, only the PCell is provided, PSCell is missing.

Proposal 1: Request to report PSCell and the PSCell information shall be added to the S1AP and NGAP signalling.

In order to include the PSCell in the signalling towards the CN, the MN shall know the cell ID. In EN-DC, it was assumed that the PSCell management is fully under the SN’s responsibility and therefore the MN does not need to know the cell ID. Information about PSCell is delivered from the SN to the UE either via the MN, or directly, using SRB3. In the latter case, if the security key does not need to be changed, the MN is not even aware that the PSCell has changed. But even if the MN is involved in the signalling, it is not sure if the MN is able to read the NR RRC messages.

The request to report PSCell shall be forwarded also in case of HO (and context retrieve in NR). Therefore, the same request that is added in S1 and NG shall be added in the existing Location Reporting Information.

Proposal 2: Request to report PSCell and the PSCell information shall be added to the X2AP and XnAP signalling.

Including PSCell in the X2AP or XnAP signalling is not difficult and can perfectly be done in backward-compatible way. In most scenarios it will just be an additional IE in messages that have to be sent anyway. However, in case of intra-SN PSCell change, when SRB3 is used, normally the MN is not bothered – this helps speed up the procedure. This existing functionality shall not be affected. Therefore, the signalling of the PSCell shall be independent from the signalling over SRB3, so that the PSCell change itself is not slowed down: the SN shall be allowed to execute needed PSCell change and only then it shall inform about it the MN.

Proposal 3: The information about PSCell change shall be independent from the signalling over SRB3.

It can be assumed that PSCell for LI will be used rarely. In order to spare the SN from unnecessary signalling, the need to report UE’s location shall be indicated by the MN. In theory, such indication can also be used to switch off the reporting. However, it creates complications related to the difference between Addition and Modification: ‘stop’ does not make sense for the former. Hence, considering the reporting will be requested rarely, it is probably all right to keep it activated once done.

Proposal 4: The MN shall indicate the PSCell information is needed.

# 3	Conclusions

In this short paper, we’ve reviewed the situation and the constrains for the solution to provide the PSCell for the LI purposes. There are three points to observe:

- Request to report PSCell and the PSCell information shall be added to the S1AP and NGAP signalling.

- Request to report PSCell and the PSCell information shall be added to the X2AP and XnAP signalling.

- The information about PSCell change shall be independent from the signalling over SRB3.

- The MN shall indicate the PSCell information is needed.

The changes apply therefore to S1AP [2], NGAP [3], X2AP [4], XnAP [5] and stage-2 [6].

# 4	References

- R3-190119, RAN3 #103;

- R3-180401, RAN3 #103;

- R3-180402, RAN3 #103;

- R3-180152, RAN3 #103;

- R3-180153, RAN3 #103;

- R3-180154, RAN3 #103;