3GPP TSG RAN WG3 Meeting #103	R3-190355

Athens, Greece, February 25th – March 1st 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0028 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB Node Integration Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introducing the IAB node integration procedure into the NG-RAN architecture specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IAB node integration procedure. The corresponding discussion paper is presented in R3-190354. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB node integration procedure not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

8.9.x 	IAB node initial access

Figure 8.9.x-1 shows the IAB node initial access procedure.

Figure 8.9.x-1: IAB node initial access procedure

- The IAB node sends an RRCSetupRequest message to the gNB-DU.

- The gNB-DU includes the RRC message and, if the IAB node is admitted, the corresponding lower layer configuration for the NR Uu interface in the INITIAL UL RRC TRANSFER message and transfers to the gNB-CU. The INITIAL UL RRC TRANSFER message includes the C-RNTI allocated by the gNB-DU.

- The gNB-CU allocates a gNB-CU UE F1AP ID for the IAB node and generates RRCSetup message towards the IAB node. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message.

- The gNB-DU sends the RRCSetup message to the IAB node.

- The IAB node sends the RRC CONNECTION SETUP COMPLETE message to the gNB-DU. The S-NSSAI IE in the RRC CONNECTION SETUP COMPLETE message indicates the IAB node.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

- The gNB-CU sends the INITIAL UE MESSAGE to the AMF. This could be a dedicated AMF serving only the IAB nodes.

At this point, the IAB node will perform registration (including authentication and key generation) without establishing a PDU session.

8. The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

9. The gNB-CU sends the IAB CONTEXT SETUP REQUEST message to establish the IAB node context in the gNB-DU. In this message, it may also encapsulate the SecurityModeCommand message.

10. The gNB-DU sends the SecurityModeCommand message to the IAB node.

11. The gNB-DU sends the IAB CONTEXT SETUP RESPONSE message to the gNB-CU.

12. The IAB node responds with the SecurityModeComplete message.

13. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

14. The gNB-CU generates the RRCReconfiguration message and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration could include a configuration of one or more IAB backhaul bearers.

15. The gNB-DU sends RRCReconfiguration message to the IAB node.

16. The IAB node sends RRCReconfigurationComplete message to the gNB-DU.

17. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

18. The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

-------------------------------------------End of changes ------------------------------------------