3GPP TSG-RAN WG3 Meeting #103	R3-190554

Athens, Greece, Feb. 25th – Mar.1st, 2019

Agenda item:		23.6

Source:	LG Electronics Inc.

Title:	Considerations on the issues for MR-DC based NR V2X

Document for:		Discussion

1. Introduction

In last meeting, there were some proposals on MR-DC based scenarios. This paper is to investigate potential issues of it based on the scenarios captured in TR 38.885. The corresponding proposals are also provided.

2. Discussion

For NR advanced V2X services, it has been agreed for the operation scenarios, which were captured in TR 38.885 [4] and also shown as follows:

4.1 Operation Scenarios … Figure 4.1-4, Figure 4.1-5 and Figure 4.1-6 illustrate the MR-DC scenarios to support V2X SL communication. Particularly: 1) In scenario 4, a UE's V2X communication in LTE SL and NR SL is controlled/configured by Uu while the UE is configured with EN-DC; 2) In scenario 5, a UE's V2X communication in LTE SL and NR SL is controlled/configured by Uu while the UE is configured in NE-DC; 3) In scenario 6, a UE's V2X communication in LTE SL and NR SL is controlled/configured by Uu while the UE is configured in NGEN-DC. Figure 4.1-4: Scenario 4 Figure 4.1-4: Scenario 5 Figure X-4: Scenario 6

As described in TR38.885, the study prioritized Scenarios 1 and 2, and MN controlling/configuring both NR SL and LTE SL in Scenario 5 and 6 which is covered by Scenario 1 and 2. But for RAN3, there are two issues to think on MR-DC support for V2X. Let us take NE DC as an example:

- For mode 2, how the other RAT’s resource pool is notified/configured to MN in case that MN is in charge of configuration to UE

- For mode 1, how the MN schedules the other RAT’s sidelink resource to UE and whether the SN should be involved

On the first issue, there may have two solutions. The first solution is OAM configuration based scheme, that is, OAM configures the other RAT’s resource pool information to MN, which can use it and broadcast to UE. This solution is lack of flexibility. The other solution can be SN notifies the resource pool information to the MN. In addition, if SN modifies the pool according to its RAT’s situation, it can also update the resource pool information to MN.

On the second issue, there may also have two solutions. The first one is based on the two solutions of the first issue, that is, the pool information is known by the MN, which takes the role of sidelink scheduling for the UE. The other solution is that SN always involves the scheduling for the UE. In case that the UE requests the sidelink resource to MN, MN always passes the request to SN, which takes the role of scheduling and notifies to MN again. Finally, MN signals to UE on the resource grant.

The issues and solutions are just the initial thinking on impacts of RAN3 for supporting MR-DC based NR V2X. Of course, the progress depends on RAN1 and RAN2’s final agreements on the scenarios.

Based on the analysis above, the following proposal is suggested to RAN3:

Proposal): it is suggested for RAN3 to investigate the following issues for supporting MR-DC based NR V2X:

- For mode 2, how the other RAT’s resource pool is notified/configured to MN in case that MN is in charge of configuration to UE

- For mode 1, how the MN schedules the other RAT’s sidelink resource to UE and whether the SN should be involved

3. Conclusion

In this contribution, the potential issues of MR-DC based NR V2X were investigated. The following proposal is suggested to RAN3:

Proposal): it is suggested for RAN3 to investigate the following issues for supporting MR-DC based NR V2X:

- For mode 2, how the other RAT’s resource pool is notified/configured to MN in case that MN is in charge of configuration to UE

- For mode 1, how the MN schedules the other RAT’s sidelink resource to UE and whether the SN should be involved

4. References

- RP-182111, “Study on NR V2X”, LG Electronics

- R3-187223, “Summary of offline discussion: WF for “Study on NR V2X” in RAN3”, LG Electronics

- 3GPP TR 23.786 V1.1.0: Study on architecture enhancements for EPS and 5G System to support advanced V2X services

- 3GPP TR 38.885 V1.0.0: Study on Vehicle-to-Everything