3GPP TSG-RAN WG3 Meeting #103	R3-190807

# Athens, Greece, 25th, February – 1st March 2019

Agenda Item:	15.5

Source:	Ericsson

Title: 	Enhancement on Intra eNB CP Relocation for NB-IoT

Document for:	Discussion and Approval

# 1	Discussion

For NB-IOT UEs using Control Plane CIoT EPS optimizations, in the case of, for example, Radio Link Failure, UE will initiate a RRC Re-establishment procedure. The eNB receives the RRC Connection Reestablishment uses the eNB CP Relocation Indication procedure to request authentication and establishment of the S1 connection. There are quite a lot of procedures involved between the MME and the new eNB/ old eNB.

New eNB <-> MME

Step 1: The eNB receives the RRC Connection Reestablishment should trigger a eNB CP Relocation procedure;

Step 2: MME initiates the Connection Establishment procedure to setup the S1 connection;

(See below)

Old eNB <-> MME

Step 3: MME initiates MME CP Relocation procedure, to inform the eNB served the UE early that UE has now moved to another eNB

Step 4: eNB informs MME the NAS-PDUs not delivered.

(See below)

Old eNB <-> MME

Eventually, the UE Context and the associated S1 is released in the old eNB and MME.

Step 5:	The old eNB triggers UE Context Release (Optional)

Step 6: MME triggers UE Context Release procedure to release in the old eNB.

Now in the case of Intra eNB CP Relocation, the new eNB and the old eNB are the same. The MME upon the reception of the eNB CP Relocation can determine that the UE is doing the RRC Connection Reestablishment towards the same eNB.

In our opinion, then the above steps could be simplified and enhanced.

For example, some of the steps towards the old eNB, from Step 3 to Step 6 can be skipped to simplify the handling.

Proposal 1:   RAN3 to discuss the enhancements related to Intra eNB CP Relocation

# 2	Proposal

Proposal 1:   RAN3 to discuss the enhancements related to Intra eNB CP Relocation