3GPP TSG RAN WG3 Meeting #103 	R3-190896

Athens/Greece, 25 February-1 March 2019

Title:	Response to R3-190138 (NTN architecture)

Source: 	Thales

Type:	Discussion

Document for:	Agreement

Agenda Item:	20.2.1

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document provides clarification to comments/questions raised in the following TDOC submitted to RAN3#103 about the NTN Gateway: R3-190138 “Further clarification of NTN architecture aspects”

It aims at trigering a discussion during the NTN on line session.

# Discussion

Question 1: Clarify the interface between NTN-GW and on ground NTN gNB for Transparent Satellite, so as to avoid inter-vendor IOT trouble ?

| Organisation | Answer |
| --- | --- |
| Thales | In the context of transparent payload, the NTN-Gateway acts as a Remote radio header or radio unit and doesn’t process the Uu signal. The NR-Uu signal is modulated onto carriers on both feeder and service links. The satellite amplifies and frequency convert the NR-Uu signal. The interface between the NTN Gateway and the gNB can be based on CIPRI like protocol. |

Question 2: Clarify whether UDP, GTP-U and SCTP layer is needed inside NTN-GW for Regenerative Satellite, so as to achieve transport layer gains ?

| Organisation | Answer |
| --- | --- |
| Thales | In the context of regenerative payload (gNB or gNB-DU on board), the NTN-Gateway acts as a IP router that is transparent to the UDP, GTP-U and SCTP layers. the SRI provides IP trunk connections between the NTN Gateway and the satellite to transport respectively NG or F1 interfaces |

# Conclusion

Proposal 1: In the context of transparent payload, the NTN-Gateway acts as a Remote radio header or radio unit and doesn’t process the Uu signal. The NR-Uu signal is modulated onto carriers on both feeder and service links. The satellite amplifies and frequency convert the NR-Uu signal. The interface between the NTN Gateway and the gNB can be based on CIPRI like protocol.

Proposal 2: In the context of regenerative payload (gNB or gNB-DU on board), the NTN-Gateway acts as a IP router that is transparent to the UDP, GTP-U and SCTP layers. the SRI provides IP trunk connections between the NTN Gateway and the satellite to transport respectively NG or F1 interfaces