3GPP TSG RAN WG3 Meeting #103 	R3-191032

Athens/Greece, 25 February-1 March 2019

Title:	NTN feeder link and ISL performances

Source: 	Thales

Type:	pCR

Document for:	Agreement

Agenda Item:	20.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at justifying the performance of Satellite Radio Interface at the feeder link and ISL.

It proposes some modifications to the clause 8.4.

# Discussion

## 2.1 Typical characteristics of the feeder link in a satellite communication system

ITU document [1] describes the different contributions that affect the propagation loss, relative to free-space loss:

- attenuation by atmospheric gases;

- attenuation by rain, other precipitation and clouds;

- focusing and defocusing;

- decrease in antenna gain due to wave

- front incoherence;

- scintillation and multipath effects;

- attenuation by sand and dust storms

The impact of these effects depends on frequency, geographic location and elevation angle.

Most of these effects can be counteracted using uplink power control (ULPC) and/or ACM, similarly to terrestrial networks. For high frequency bands however (typ. above 20GHz), rain attenuation can be of several tens of dBs. For the feeder link, site diversity is used to increase the feeder link availability, and consists in rerouting traffic to alternate earth stations, as described in clause 2.2.4 of [1]. Fading prediction algorithms are necessary to anticipate outage and to load the context in the new gateway to ensure a seamless connection. The prediction of outage probability due to rain attenuation with site diversity is given in clause 2.2.4.1 of [1].

Typical attenuation not exceeded 99.9% of the time is given in Figure 1 for a gateway located in Agadir following ITU-R P.618-13 model [1] with carrier frequency equal to 30GHz. It shall be noted that elevation angles below 20° lead to very high attenuation.

Figure 2 gives the mean fading duration for fading events above 12dB at 30GHz. Such value is in the order of a typical outage event, considering for instance 6dB of ULPC and an accepted capacity decrease corresponding to 6dB of ACM).

Figure 1: Attenuation not exceeded 99.9% of the time at 30GHz

Figure 2: Mean fading duration for fading events above 12dB at 30GHz

Assuming N nominal gateways and P redundant gateways, it can be demonstrated that the probability that a given user is in outage due to feeder link is given by:

Poutage=i=0NiNPr⁡(P+i gateways in outage)

With (see [2])

Prk gateways  in outage= l=1Nkj=1NpjXjl*j=1N(1-pj)Xjl-1

With Nk  being the number of all possible combinations with k gateways in outage and Xjl indicates if gateway j is in outage for combination l. Figure 3 illustrates the user availability probability of a N+1 system assuming all gateways have the same individual availability probability.

Figure 3: User availability probability for a N+1 system

Observation 1: In mm-wave, user availability probability is strongly improved using site diversity

## 2.2 Typical characteristics of ISL in a satellite communication system

In the LEO case, the one-way ISL propagation delay is constellation specific. Values around 10ms may be considered as typical.

Typical availability probability of an ISL for LEO is 99.999%

Observation 2: Inter Satellite Links in LEO constellations typical feature a one-way propagation delay around 10ms and an availability probability of 99.999%.

# Conclusion

Observation 1: In mm-wave, user outage probability is strongly improved using site diversity

Observation 2: Inter Satellite Links in LEO constellations typical feature a one-way propagation delay around 10ms and an availability probability of 99.999%.

Proposal 1: To adopt the below text proposal for clause 8.4

# References

[1] ITU-R P.618-13 (12/2017) « Propagation data and prediction methods required for the design of Earth-space telecommunication systems »

[2] Mongelli, Maurizio & Cola, Tomaso & Cello, Marco & Marchese, Mario & Davoli, Franco. (2016). Feeder-Link Outage Prediction Algorithms for SDN-based High-Throughput Satellite Systems. 1-6. 10.1109/ICC.2016.7510736.

# Text Proposal for TR 38.821 v0.3.0

START OF CHANGES

## 8.4	Transport aspects (FFS)

In the transparent case, a NTN GW connects directly to one or several satellites via SRI. In the regenerative case, a NTN GW can directly connect to one or several satellites via SRI, or indirectly connect to one or several NTN GW via ISL. Hence the NG protocol is transported over SRI, and may also be transported over ISL.

A gNB is connected to the 5GCN. The transport of this logical interface can be realized over SRI and possibly over ISL.

The satellite may embark additional transport routing functions that are out of RAN scope.

SRI transports 3GPP-RAN specified protocols i.e., transmits the NG interface signaling packets.

ISL can transport:

- Xn interface signaling packets and enable coordination between gNBs on board adjacent satellites, and especially to support UE mobility, from a source gNB to a target gNB. (FFS)

- data packets, in case traffic functions are hosted on board the satellites. (FFS)

- NG interface signaling packets

## 8.4.1 Characteristics of transport links in NTN

#### 8.4.1.1 Characteristics of SRI on the feeder link

Transport over SRI may be subject to the following constraints:

Editors' Note: Some typical figures about outage probability and availability duration should be provided

- Much longer propagation delay with respect to terrestrial transport links – the typical length for an Earth-satellite link can go from a few thousands of km (LEO scenario) to several tens of thousands of km (GEO scenario). Hence the one way delay over the SRI ranges from 6 ms (LEO at 600 km and 10° elevation) to ~136 ms (GEO at 35788 km and 10° elevation);

- Possibly higher outage probability with respect to terrestrial transport links when the SRI operates at mm-wave, which is heavily impacted by atmospheric propagation impairments (e.g. rain attenuation). However this is mitigated through Uplink power control, Adaptive Coding and Modulation and/or space diversity schemes. Typically a feeder link is dimensioned to provide availability up to 99.999% with site diversity.

- The SRI may become unavailable

- due to atmospheric attenuation when SRI operates at mm-wave

- when the satellite disappears below the horizon in LEO constellation

Therefore mobility management is typically activated to ensure a seamless service continuity and 0 ms interruption time, as described in Section 8.4.1.3

#### 8.4.1.2 Characteristics of Inter Satellite link

In the LEO case, the one-way ISL propagation delay is constellation specific. Values around 10ms may be considered as typical.

Inter Satellite Links in LEO constellations typical feature an availability probability of 99.999%

END OF CHANGES