# 3GPP TSG-RAN WG3 #103	Tdoc R3-190718

Athens, Greece, 25th February – 1st March

Agenda Item:	09.3.11

Source:	Ericsson

Title:	About Cells Failed to be Activated IE in gNB-CU Configuration Update Ack

Document for:	Discussion, Decision

# Introduction

At RAN3 101bis some agreements regarding cell management over F1 were made. In light of these agreements, the Cells Failed to be Activated IE in gNB-CU Configuration Update Acknowledge has no meaning and should be removed. In the following we provide further explanation and we put forward our thoughts on the matter.

# Discussion

At RAN3 101bis the CU and DU responsibility was clarified. CU is responsible for activation and deactivation of cells. On the other hand, DU is responsible for cell addition and deletion. Furthermore, the Service Status IE was agreed.

Based on this clarification and understanding of the cell management over F1, relevant changes need to be made in TS. 38.473. Currently the Cells Failed to be Activated IE is included in gNB-CU Configuration Update Acknowledge and its usage is described as follows:

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are inactive as defined in TS 38.401 [4].

But according to the description in TS 38.401 and the clarification of the CU and DU responsibilities, the gNB-CU decides the cells that are active or inactive. The gNB-DU can’t take any such decision and inform gNB-CU. So the Cells Failed to be Activated Item IE is basically meaningless and should not be included.

To prove the above, quotes from 38.401 are reported below:

## 8.5	F1 Startup and cells activation

This function allows to setup the F1 interface between a gNB-DU and a gNB-CU and it allows to activate the gNB-DU cells.

Figure 8.5-1: F1 startup and cell activation

0. The gNB-DU and its cells are configured by OAM in the F1 pre-operational state. The gNB-DU has TNL connectivity toward the gNB-CU.

1. The gNB-DU sends an F1 SETUP REQUEST message to the gNB-CU including a list of cells that are configured and ready to be activated.

2. In NG-RAN, the gNB-CU ensures the connectivity toward the core network. For this reason, the gNB-CU may initiate either the NG Setup or the gNB Configuration Update procedure towards 5GC.

3. The gNB-CU sends an F1 SETUP RESPONSE message to the gNB-DU that optionally includes a list of cells to be activated. The cells in the list of cells to be activated in F1 SETUP RESPONSE message become active, while the cells not in the list are inactive. The cells that are active are Out-of-Service until the gNB-DU indicates that they are In-Service . The gNB-DU will initiate the gNB-DU Configuration Update procedure towards the gNB-CU and includes the cell(s) that are In-Service  and/or the cell(s) that are Out-Of-Service. The gNB-DU may also indicate cell(s) to be deleted, in which case the gNB-CU removes the corresponding cell(s) information.

4. The gNB-CU may send a GNB CU CONFIGURATION UPDATE message to the gNB-DU that optionally includes a list of cells to be activated, e.g., in case that these cells were not activated using the F1 SETUP RESPONSE message.

5. The gNB-DU replies with a GNB CU CONFIGURATION UPDATE ACKNOWLEDGE message that optionally includes a list of cells that failed to be activated, the gNB-CU regards the cells as Out-Of-Service.

6. The gNB-CU may initiate either the Xn Setup towards a neighbour NG-RAN node or the EN-DC X2 Setup procedure towards a neighbour eNB.

NOTE 1:	For NG-RAN in case that the F1 SETUP RESPONSE message is not used to activate any cell, step 2 may be performed after step 3.

Over the F1 interface between a gNB-CU and a gNB-DU pair, the following two Cell States are possible:

-	Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs;

-	Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should try to provide services to the UEs.

The gNB-CU decides whether the Cell State should be “Inactive“ or “Active“. The gNB-CU can request the gNB-DU to change the Cell State using the F1 SETUP RESPONSE, the GNB DU CONFIGURATION UPDATE ACKNOWLEDGE, or the GNB CU CONFIGURATION UPDATE messages.

The gNB-DU reports to the gNB-CU the Service Status. The Service Status is the state of the radio transmission over the air.  The Service Status is reported by the gNB-DU for cells for which the Cell State is “Active “. The following Service Status are defined:

-	In-Service: the cell is operational and able to serve UEs.

-	Out-Of-Service: the cell is not operational, and it is not able to serve UEs. The gNB-DU is trying to make the cell operational.

The gNB-DU reports the Service Status using the GNB DU CONFIGURATION UPDATE message.

NOTE 2:	If gNB-DU regards that one or more cells cannot become operational, the gNB-DU deletes them and reports them using the GNB DU CONFIGURATION UPDATE message.

As it can be seen above, TS38.473 is currently contradicting the more recent agreements captured in TS38.401 because it says that “If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message [sent by the gNB-DU], the gNB-CU shall consider that the indicated cells are inactive”.

While TS38.401 says that “The gNB-DU replies with a GNB CU CONFIGURATION UPDATE ACKNOWLEDGE message that optionally includes a list of cells that failed to be activated, the gNB-CU regards the cells as Out-Of-Service” and “The gNB-CU decides whether the Cell State should be “Inactive“ or “Active“”.

Based on that we propose to remove the Cells Failed to be Activated Item IE from the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message and remove relevant references both in TS 38.473 and TS 38.401.

Proposal 1: 	We propose to remove the Cells Failed to be Activated Item IE from the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal 2: 	RAN3 is kindly asked to agree with the CRs in R3-190719 and R3-190720.

# Conclusion

In this contribution, we discussed the implications of the cell management over F1 in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message and made the following proposal.

Proposal 1: 	We propose to remove the Cells Failed to be Activated Item IE from the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal 2: 	RAN3 is kindly asked to agree with the CRs in R3-190719 and R3-190720.