3GPP TSG RAN WG3 Meeting #103bis	R3-191381

Xi’an, P.R. China, April 8th – 12th 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0288 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1AP Signalling for Remote Interference Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI_RIM |  |  |  |  |  |  | Date: |  | 2019-03-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enabling Remote Information Management signaling over F1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced the DL Radio Information Message Transfer and UL Radio Information Message Transfer Class-2 procedures. Introduced the RIM-specific IEs. This is a backwards-compatible CR. The corresponding discussion is presented in R3-191379. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remote Information Management not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8.1, 8.x (new), 9.2.x.1 (new), 9.2.x.2 (new), 9.3.1.x (new), 9.3.1.y (new), 9.3.1.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

CN	Core Network

CG	Cell Group

CGI	Cell Global Identifier

CP	Control Plane

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IMEISV 	International Mobile station Equipment Identity and Software Version number

NSSAI	Network Slice Selection Assistance Information

RANAC	RAN Area Code

RIM	Remote Interference Management

RRC	Radio Resource Control

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

-------------------------------------------Change 2-------------------------------------------

## 8.1	List of F1AP Elementary procedures

>>>>>>>>>>>>>>Skipped text<<<<<<<<<<<<<<

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| DL Radio Information Message Transfer | DL RADIO INFORMATION MESSAGE TRANSFER |
| UL Radio Information Message Transfer | UL RADIO INFORMATION MESSAGE TRANSFER |

-------------------------------------------Change 3-------------------------------------------

## 8.x	Radio Information Transfer procedures

### 8.x.1	UL Radio Information Message Transfer

#### 8.4.1.1	General

The purpose of the UL Radio Information Message Transfer procedure is to transfer radio-related information to the gNB-CU. The procedure uses non-UE-associated signaling.

#### 8.4.1.2	Successful operation

Figure 8.4.1.2-1: UL Radio Information Message Transfer procedure.

If the DU to CU RIM Message IE is included in the UL RADIO INFORMATION MESSAGE TRANSFER, the gNB-CU should merge this message with corresponding messages received from other gNB-DUs that contain the identical Target RIM Set ID IE value and forward the merged message to the AMF.

#### 8.x.1.3	Abnormal Conditions

Not applicable.

### 8.x.2	DL Radio Information Message Transfer

#### 8.x.2.1	General

The purpose of the UL Radio Information Message Transfer procedure is to transfer radio-related information to the gNB-DU. The procedure uses non-UE-associated signaling.

#### 8.x.2.2	Successful operation

Figure 8.4.2.2-1: DL Radio Information Message Transfer procedure.

If the CU to DU RIM Message IE is included in the DL RADIO INFORMATION MESSAGE TRANSFER, the gNB-DU shall, if supported, take the information therein into account for the purpose of Remote Interference Management.

#### 8.x.2.3	Abnormal Conditions

Not applicable.

-------------------------------------------Change 4-------------------------------------------

#### 9.2.x.1	UL RADIO INFORMATION MESSAGE TRANSFER

This message is sent by a gNB-DU to a gNB-CU, to convey radio-related information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE Radio Information Type | M |  |  |  | YES | reject |
| >RIM |  |  |  |  |  |  |
| >>DU to CU RIM Message IE | O |  | 9.3.1.x |  | YES | reject |

#### 9.2.x.2	DL RADIO INFORMATION MESSAGE TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to convey radio-related information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE Radio Information Type | M |  |  |  | YES | reject |
| >RIM |  |  |  |  |  |  |
| >>CU to DU RIM Message IE | O |  | 9.3.1.y |  | YES | reject |

-------------------------------------------Change 5-------------------------------------------

#### 9.3.1.x	DU to CU RIM Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Source RIM Set ID | M |  | 9.3.1.z | ID of a RIM set. | YES | reject |
| Target RIM Set ID | M |  | 9.3.1.z | ID of a RIM set. | YES | reject |
| List of Cells |  | 0..1 |  |  |  |  |
| > List of Cells Item |  | 1..< maxCellingNBDU > |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |  |  |
| RIM message | M |  | ENUMERATED(RS detected, RS disappeared …) | The informational content of Remote Interference Management backhaul message. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

-------------------------------------------Change 6-------------------------------------------

#### 9.3.1.y	CU to DU RIM Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Source RIM Set ID | M |  | 9.3.1.z | ID of a RIM set. | YES | reject |
| Target RIM Set ID | M |  | 9.3.1.z | ID of a RIM set. | YES | reject |
| List of Cells |  | 0..1 |  |  |  |  |
| > List of Cells Item |  | 1..10000 |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |  |  |
| RIM message | M |  | ENUMERATED(RS detected, RS disappeared …) | The informational content of Remote Interference Management backhaul message. | YES | reject |

-------------------------------------------Change 7-------------------------------------------

#### 9.3.1.z	RIM Set ID

This IE defines the Remote Interference Management Set ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Set ID | M |  | BIT STRING (SIZE(22)) | The Remote Interference Management Set ID. |

-------------------------------------------End of changes ------------------------------------------