3GPP TSG-RAN WG3 Meeting #103bis	R3-191727

# Xian, P.R. China, 8th – 12th April 2019

Agenda Item:	9.3.16

Source:	Ericsson

Title: 	Location Report Request

Document for:	Discussion and Approval

# 1	Discussion

The Location Report Request is included in the HANDOVER REQUEST message, so the UE could report the location after the handover, without the need to wait for the additional LOCATION REPORTING CONTROL message from AMF.

When UEs change state from CM-IDLE to CM-CONNECTED, for any AOI event subscription retained in AMF, AMF may trigger ‘Location Report Control’ procedure again. For the similar reason that the Location Report Request is included in the HANDOVER REQUEST message, for efficiency reasons, it could be combined with ‘Initial Context Setup Request’ if AMF knows the UE is in CM-CONNECTED state.

Proposal 1:   RAN3 to agree to add Location Reporting Request Type IE in INITIAL CONTEXT SETUP.

# 2	Proposal

Proposal 1:   RAN3 to agree to add Location Reporting Request Type IE in INITIAL CONTEXT SETUP.