3GPP TSG RAN WG3 Meeting #103bis	R3-192148

Xi’an, P.R. China, April 8th – 12th 2019

Agenda Item:	13.2.1.2

Source:	Ericsson

Title:	(TP for NR-IAB BL CR for TS 38.401): Backhaul Channel Setup and Modification Procedure for IAB Networks

Document for:	Agreement

# 1	Introduction

At the RAN3#103bis meeting, the following comeback was assigned:

CB: # 34_BHchSetupMod

-  capture status of disc

- merge from 1551 as needed

- merge from 1299 if agreeable

- add FFSs etc.

- reuse current UE ctxt mgmt. procedures or introduce new ones?

- leave out RRC aspects (out of RAN3 scope)

- check details

# 2 	Conclusion

Proposal: Agree to the Backhaul RLC channel establishment procedure described in the Appendix.

# Appendix: TP for NR_IAB BL CR to TS 38.401

-------------------------------------------Change 1-------------------------------------------

# 8.9.x		Backhaul RLC channel establishment procedure

The backhaul RLC channel is an RLC channel used for backhauling between IAB-node and IAB-donor-DU, or between different IAB-nodes.

The BH RLC channel establishment and procedure is shown in Figure 8.9.x-1, on an example of UE accessing the network and establishing a DRB.

Figure 8.9.x-1: Signaling flow for IAB BH RLC channel establishment procedure

Step 1 includes steps 1-13 of the UE initial access procedure (clause 8.1). In step 9 of the UE initial access procedure the parent IAB-node (IAB-node 2) is configured with information to set up the access DRB.

2. IAB-donor-CU sends to the IAB-donor-DU an F1-AP request message for setting up the parent DU side of the BH link between IAB-donor DU and IAB-node 1. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

3. IAB-donor-DU responds to the F1-AP request message from the IAB-donor-CU from step 2.

4. IAB-donor-CU sends to the IAB-donor-DU an DL RRC MESSAGE TRANSFER message encapsulating the RRC Reconfiguration message for configuring the MT functionality of the IAB-node 1. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

5. The IAB-donor-DU decapsulates and forwards the RRC Reconfiguration message to the MT functionality of the IAB-node 1. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

6. The MT functionality of the IAB-node 1 sends to the IAB-donor-DU an RRC Reconfiguration Complete message destined to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

7. The IAB-donor-DU sends the UL RRC MESSAGE TRANSFER Message encapsulating the RRC Reconfiguration Complete message to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor DU and IAB-node 1.

8. IAB-donor-CU sends to the DU functionality of IAB-node 1 an F1-AP request message for setting up the parent DU side of the BH link between IAB-node 1 and IAB-node 2. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

9. IAB-node 1 responds to the F1-AP request message from the IAB-donor-CU from step 8.

10. IAB-donor-CU sends to the DU functionality of IAB-node 1 a DL RRC MESSAGE TRANSFER message encapsulating the RRC Reconfiguration message for configuring the MT functionality of IAB-node 2. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

11. The DU functionality of IAB-node 1 decapsulates and forwards the RRC Reconfiguration message to the MT functionality of the IAB-node 2. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

12. The MT functionality of the IAB-node 2 sends to the IAB-node 1 an RRC Reconfiguration Complete message destined to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

13. The IAB-node 1 sends the UL RRC MESSAGE TRANSFER Message encapsulating the RRC Reconfiguration Complete message to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

14. IAB-donor-CU sends to the DU functionality of IAB-node 2 an UE context modification request for the UE DRB.

15. The DU functionality of the IAB-node 2 sends the UE context modification response to the IAB-donor-CU.

16. The IAB-Donor CU generates the RRC Reconfiguration message and encapsulates it in the DL RRC MESSAGE TRANSFER message for the DU functionality of IAB-node 2.

17. The DU functionality of the IAB-node 2 sends the RRC Reconfiguration message to the UE.

18. The UE sends RRC Reconfiguration Complete message to the DU functionality of the IAB-node 2.

19. The DU functionality of IAB-node 2 encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to IAB-Donor CU.

The IAB-donor-CU uses the existing CU-DU split principles and the F1-AP signaling to configure the parent DU side of the RLC channel. The IAB-donor-CU uses RRC signaling (which is piggybacked in F1-AP messages terminating at the parent DU side of the backhaul RLC channel) to configure the child IAB-node part of the backhaul RLC channel (i.e. the MT functionality of the child node).

-------------------------------------------End of changes ------------------------------------------