3GPP TSG RAN WG3#104 	                                                                               R3-192272

Reno, USA, 13th – 17th  May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.821 | CR | pCR | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for TR38.821 Further Clarifications of NTN DC Architectures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2019-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current statement “As for terrestrial access, connectivity combining can occur for either the uplink or the downlink or both.” is confusing, as MR-DC@5GC framework and each radio link are always bi-directional, i.e. there is no uni-directional radio link. In NTN involved DC, the applicability of different bearer type should be further studied, e.g. split bearer. Such important point should be captured. For Figure 5.4.2-4, the Xn interface between cellular gNB and gNB-CU on ground is missing. For NTN DC with two gNBs on board, the availability of ISL for Xn interface is subject to different GEO/LEO satellite scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To delete the existing statement “As for terrestrial access, connectivity combining can occur for either the uplink or the downlink or both.”; To add up new statement“The applicability of different DC bearer type is FFS. Due to large different propogation delay between satellite and cellular radio links, the benefit of split bearer is FFS.” To add up the Xn interface between cellular gNB and gNB-CU on ground in Figure 5.4.2-4. To add up new statement “The Xn interface for DC over ISL is supposed to be as stable over time as possible, hence in GEO case, there is no concern, but in LEO case, the ISL between different neighbour satellites in the same LEO orbit looks more promising, but the ISL between GEO and LEO or ISL between LEOs in different orbits are FFS.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion or incompleteness regarding DC Architecture for NTN exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change, ommited text not changed ***

5.4	Multi connectivity involving NTN-based NG-RAN (FFS)

5.4.1	Overview

This clause discusses multi connectivity [5], either for transparent or regenerative NTN-based NG-RAN, and in combination or not with terrestrial-based NG-RAN (NR or EUTRA). The focus is on dual connectivity with simultaneous use of two radio access.

This may apply to transparent satellites as well as regenerative satellites with gNB or gNB-DU function on board.

A number of service scenarios as described in TS 22.261 (e.g. user in residential homes, in vehicles, in high speed trains or on board airplanes), would benefit from the combination of terrestrial and non-terrestrial access to meet the targeted service performances in terms of data rate and/or reliability.

In underserved areas, the bandwidth provided by a terrestrial based access (e.g. LTE) may be limited at cell edge. Adding a NTN based NG-RAN will enable to achieve the targeted experience data rate.

Under some scenarios such as high speed trains, the service area may not be fully homogeneous along the railway track and multi connectivity involving NTN-based NG-RAN would enable to provide the targeted reliability.

Hence a UE may be connected and served simultaneously by at least:

- One NTN-based NG-RAN and one terrestrial-based access (NR or EUTRA)

- One NTN-based NG-RAN and another NTN-based NG-RAN

5.4.2	Architecture aspects

Multi connectivity involving transparent NTN-based NG-RAN

A User Equipment is connected to a 5GCN via simultaneously a transparent NTN-based NG-RAN and a cellular NG-RAN. We assume that the NTN Gateway is located in the PLMN area of the cellular access network.

Figure 5.4.2-1: Multi connectivity involving transparent NTN-based NG-RAN and cellular NG-RAN

Both gNB of the NTN-based NG-RAN or the gNB/ng-eNB of the cellular NG-RAN could be elected as master node.

Figure 5.4.2-2: Radio Protocol Architecture for MCG, SCG and split bearers from a UE perspective in MR-DC with 5GC.

The applicability of different DC bearer type is FFS. Due to large different propogation delay between satellite and cellular radio links, the benefit of split bearer is FFS.

Another case to be considered, refers to the combination of two Transparent NTN-based NG-RAN either GEO and LEO based or a combination of both. This is of interest to provide service to UEs in unserved areas. The LEO NTN-based NG-RAN featuring relatively low latency can be used to support the delay sensitive traffic while the GEO NTN-based NG-RAN would provide additional bandwidth to meet the targeted throughput requirements. This is depicted in the figure below.

Figure 5.4.2-3: Multi connectivity between two transparent NTN-based NG-RAN

Multi connectivity involving regenerative NTN-based NG-RAN (gNB-DU on board)

Another case to be considered, refers to the combination of a regenerative NTN-based NG-RAN (gNB-DU on board) and a cellular NG-RAN. This is of interest to provide service to UEs in underserved areas. This is depicted in the figure below.

Data

Network

NG

N6

NR

-

Uu

F1

NR

-

Uu

NTN

Gateway

NG

UE

gNB

-

DU

gNB

-

CU

gNB

5G

-

CN

SRI

Data

Network

NG

N6

NR

-

Uu

F1

NR

-

Uu

NTN

Gateway

Xn

UE

gNB

-

DU

gNB

-

CU

gNB

5G

-

CN

SRI

Figure 5.4.2-4: Multi connectivity involving regenerative NTN-based NG-RAN (gNB-DU) and cellular NG-RAN

Note that the multi connectivity may also involve two regenerative NTN-based NG-RAN (gNB-DU on board)

Multi connectivity involving regenerative NTN-based NG-RAN (gNB on board)

The combination of two regenerative NTN-based NG-RAN (gNB on board) either GEO or LEO based or a combination of both with Inter Satellite Links in between is also worth to consider to provide service to UEs in unserved areas. This is depicted in the figure below.

Figure 5.4.2-5: Multi connectivity between two regenerative NTN-based NG-RAN (gNB on board)

The Xn interface for DC over ISL is supposed to be as stable over time as possible, hence in GEO case, there is no concern, but in LEO case, the ISL between different neighbour satellites in the same LEO orbit looks more promising, but the ISL between GEO and LEO or ISL between LEOs in different orbits are FFS.

Note that multi connectivity between regenerative NTN-based NG-RAN (gNB on board) and cellular NG-RAN (NR or LTE based) is not addressed because the transport of Xn protocol over the Feeder link (based on Satellite Radio interface) is For Further Study.

*** End change ***