3GPP TSG-RAN WG3 Meeting #104	R3-192444

Reno, NV, USA, May 13th – 17th 2019

Agenda Item:	9.3.13

Source:	Ericsson

Title:	Making the DBS useful for Dual Connectivity

Document for:	Agreement

# Introduction

The usefulness of desired buffer size (DBS) in dual connectivity (DC) and the solution for the consequent problem of out-of-order packet arrivals at DC-connected UEs have been discussed at the RAN3#103 and RAN3#103bis meetings. This contribution addresses the remaining concerns from the discussion. Further details of the proposed buffer dwell target time (BDTT) solution are presented in the CR to TS 38.425 (R3-192445).

# Current status of the discussion

The problem in question and its practical relevance have been thoroughly explained in our earlier contributions R3-190631 and R3-191784. The discussions reached the following common understanding:

- The DBS is a tool for DL flow control;

- The problem of massive out-of-order PDU arrivals at the UE in a DC scenario is acknowledged;

- It is acknowledged that each DU would likely calculate a different DBS value in case it had a common BDTT reference from the CU, compared to the case when it would not have it.

Nevertheless, there are a few remaining concerns from some companies, as discussed below.

## The relation to the selective PDCP duplication for URLLC

During the discussion it was commented that the issue at hand should be discussed within the IIoT WI, in the context of selective PDCP duplication on the DL. We believe that these are two separate issues, for at least two reasons:

- First, the central problem that the BDTT attempts to solve is massive out-of-order PDU arrivals from two DC legs to a UE, ultimately leading to packet discarding due to PDCP timer expiry at the UE. On the other hand, the IIoT discussion is related to the URLLC use case and PDCP duplication, where one of the proposals is to use discard timers for duplicate PDUs delivered to the corresponding node (herein referred to as the DU) at the secondary leg. It seems that the use of word ‘timer’ (albeit in different contexts) triggered some companies to relate the two issues.

- Second, the use cases for the BDTT and IIoT discussions are essentially different. The URLLC use case assumes very short packet delivery deadlines (typically a few ms) and very small amounts of data per packet. For instance, URLLC packets can carry commands to industrial robots, which can be duplicated on the secondary leg if the radio conditions on the primary leg are bad. Moreover, flow control per se (where DBS and BDTT are essential building blocks) is not appropriate for the URLLC use case, since flow control triggers transmission backoffs and ultimately leads to additional delays. Conversely, the introduction of BDTT is an attempt to harmonize the two DC legs, and the goal is to deliver large amounts of data from both legs to the UE within a certain timeframe.

Conclusion 1: The usefulness of DBS in DC is a separate issue from selective duplication for URLLC.

## Is the BDTT value static?

During the discussions it was commented that the BDTT is static, and that hence it should not be set over the UP signalling, but over the CP or OAM instead.

Let’s consider the following example. Assume:

- A split gNB architecture;

- DL delay = 15 ms, UL delay = 10 ms;

- The desired RAN total DL delay budget = 50 ms.

Consequently, if the wished total DL delay budget is 50 ms, then the node hosting the PDCP entity (herein referred to as the CU) must signal a BDTT of 50-15=35 ms to the DU. However, in reality this value is by no means static – it needs to be continuously updated by the CU based on continuously monitored UL/DL delay measurements. Therefore, it makes sense to include the BDTT in the DL USER DATA PDU, rather than signal the BDTT over the CP, which would make it more static.

In the above example, the CU must, when doing the flow control calculations:

- Consider that the DDDSs received are 10 ms old by the time they reach the CU.

- The CU also must, when determining how much data to send (based on the latest DDDS), add an amount of data corresponding to the estimated DU rate, multiplied by the UL delay (10 ms).

In addition, the CU must also compensate for the delay in sending packets down to the DU: since it takes 15 ms for the data sent to reach the DU, the CU must be 15 ms “ahead” in sending data so that the DU gets the requested data in time. The BDTT is the maximum dwell time the DU should target for data residing in the DU buffer and the compensation for transport network (TN) delay is made in the CU.

It is therefore obvious that the CU should be the one compensating for the TN delay because it makes the estimate of the rate at DU, and it needs to stay ahead of the curve when data is sent to DU. Finally, it is important to note that the TN delay is static only when small amounts of data are sent, but in principle the TN delay can change instantly, which can be handled by adjusting the BDTT over the UP.

Conclusion 2: The BDTT value is not static, as it depends on the TN delay between the CU and DU, which needs to be constantly monitored and accounted for.

## Can the CU mitigate the packet reordering in DC?

During the discussion, it was also claimed by some companies that the reordering problem in DC can be avoided with the existing set of tools, where CU can compare the DBSs received from two legs when determining the appropriate amount of data to send to the DUs.

Here it is yet again necessary to analyse the chain of events and the causality therein. The problem at hand i.e. the massive out-of-order packet arrival at the DC-connected UE, is caused by a disharmonized pace of data delivery from the two legs to the UE. The pace at which the data is delivered to the UE depends on: #1 the channel conditions towards the UE; #2 the current buffer status at the DU.

The DU has obviously no control over the factor #1 i.e. the radio conditions, but it can influence the factor #2 by requesting a certain data volume from the CU via DBS. As explained in R3-191784, when calculating the DBS, the DU targets a certain (configured) buffer dwell time, which is the maximum amount of time that any given packet for the DRB will spend in the DU buffer. In that sense, the given channel conditions will, in combination with the amount of data received from the CU, result in a certain buffer dwell time. Therefore, the buffer dwell time is a direct consequence of channel conditions and the amount of buffered data.

Conclusion 3: The dwell time of packets in the DU is directly dependent on channel conditions towards the UE and the amount of buffered data.

Once again, it is the discrepancy in the pace of data delivery from two legs that causes the reordering problem in DC. If the pace of delivery from the two legs is to be synchronized, both DUs need to calculate the DBS based on a common time reference i.e. a common buffer dwell target time. However, as of today, the DBS is not connected to any time value, which makes it useless for DC.

So, can the CU avoid the massive reordering problem by adjusting the data volume sent to both legs based on comparing the DBSs, as claimed? The answer is - no, for at least two reasons:

- The CU cannot know which buffer dwell time was assumed at each DU when calculating the DBS, and cannot harmonize the pace of delivery in both legs;

- Even if the CU knew which (unsynchronized!) buffer dwell times were assumed by the DUs, the CU cannot know how its finally decided data volume for each leg (obtained by adjustment of the data volume requested in the DBSs) will affect the pace of packet delivery to the UE.

The essential point that is overlooked by the comment is that the DUs calculate the DBS based on some targeted buffer dwell time, rather than calculating the targeted buffer dwell time based on the DBS. In turn, it should be noted that the packet dwell time in the buffer is tightly coupled with the pace of packet delivery to the UE. Therefore, the two DUs need a common target buffer dwell time reference in order to achieve harmonized delivery tempo to the UE.

Conclusion 4: The reordering problem at the UE in DC cannot be solved by the CU comparing the DBSs from the two legs and sending an adjusted data volume, because:

- The CU cannot know which buffer dwell time was assumed at each DU when calculating the DBS, and cannot harmonize the pace of delivery in both legs;

- Even if the CU knew which (unsynchronized!) buffer dwell times were assumed by the DUs, the CU cannot know how its finally decided data volume for each leg (obtained by adjustment of the data volume requested in the DBSs) will affect the pace of packet delivery to the UE.

Based on the above, the following is proposed:

Proposal: RAN3 to agree the CR to TS 38.425 presented in R3-192445.

# Conclusion

In this paper we address the remaining concerns about the BDTT an draw the following conclusions:

Conclusion 1: The usefulness of DBS in DC is a separate issue from selective duplication for URLLC.

Conclusion 2: The BDTT value is not static, as it depends on the TN delay between the CU and DU, which needs to be constantly monitored and accounted for.

Conclusion 3: The dwell time of packets in the DU is directly dependent on channel conditions towards the UE and the amount of buffered data.

Conclusion 4: The reordering problem at the UE in DC cannot be solved by the CU comparing the DBSs from the two legs and sending an adjusted data volume, because:

- the CU cannot know which buffer dwell time was assumed at each DU when calculating the DBS, and cannot harmonize the pace of delivery in both legs;

- even if the CU knew which (unsynchronized!) buffer dwell times were assumed by the DUs, the CU cannot know how its finally decided data volume for each leg (obtained by adjustment of the data volume requested in the DBSs) will affect the pace of packet delivery to the UE.

Based on the above, the following is proposed:

Proposal: RAN3 to agree the CR to TS 38.425 presented in R3-192445.