3GPP TSG-RAN3 Meeting #104	R3-192725

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | Draft | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1 support for positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2019-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to support NR positioning, new function should be defined over F1 interferface. Current version does not support postioning function in F1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Stage 2 text description on positioning function over F1. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Positioning funciton cannot be supported in F1AP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

CM	Connection Management

CMAS	Commercial Mobile Alert Service

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

GTP-U	GPRS Tunnelling Protocol

IP	Internet Protocol

LMF	Location Management Function

NAS	Non-Access Stratum

O&M	Operation and Maintenance

PWS	Public Warning System

QoS	Quality of Service

RET	Remote Electrical Tilting

RNL	Radio Network Layer

RRC	Radio Resource Control

SAP	Service Access Point

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SM	Session Management

SMF	Session Management Function

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TMA	Tower Mounted Amplifier

TMF	Transmission Measurement Function

TNL	Transport Network Layer

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 7.x	Positioning function

The positioning includes downlink only positioning techniques, uplink positioning only postioning techniques, and downlink and uplink positioning techniques. For the downlink positioning techniques, LMF requests downlink positioning information from gNB and sends it to UE for positioning measurements. For the uplink positioning techniques, LMF requests uplink positioning information from serving gNB and sends it to neighbour gNBs. LMF then requests measurement from gNB and gNB responds with uplink measurements.

The TMF may be deployed on gNB-DU and is responsible for the transmission positioning reference signals and receiving sounding reference signals for uplink measurements.

When gNB-CU receives the DL/UL positioning information requests from LMF, it requests DL/UL positoning information from gNB-DU. gNB-DU responds with DL/UL positioning information to gNB-CU, who then responds to LMF with the DL/UL postioning information.

When gNB-CU receives the measurement request from LMF, it requests the positioning measurement from gNB-DU. gNB-DU responds with positioning measurements to gNB-CU, who then responds to LMF with the measurement.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.x	Positioning procedures

### 8.x.1	Downlink Positioning Information exchange

### This procedure is used for the downlink positioning information exchange between LMF and gNB-DU in NR downlink positioning. Figure 8.x.1-1 shows the downlink positioning information exchange procedure.

### Figure 8.x.1-1 Downlink positioning information exchange procedure

### The LMF send DOWNLINK INFORMATION REQUEST message to the gNB-CU.

### The gNB-CU send DOWNLINK POSITIONING INFORMATION REQUEST message to the gNB-DU.

### The gNB-DU respond the gNB-CU with DOWNLINK POSITIONING INFORMATION RESPONSE message.

### The gNB-CU respond the LMF with DOWNLINK INFORMATION RESPONSE message.

### 8.x.2	Uplink Positioning Information exchange

This procedure is used for the uplink positioning information exchange between LMF and gNB-DU in NR uplink positioning. Figure 8.x.2-1 shows the uplink positioning information exchange procedure.

Figure 8.x.2-1 Uplink positioning information exchange procedure

- The LMF send UPLINK INFORMATION REQUEST message to the gNB-CU.

- The gNB-CU send UPLINK POSITIONING INFORMATION REQUEST message to the gNB-DU.

- The gNB-DU respond the gNB-CU with UPLINK POSITIONING INFORMATION RESPONSE message.

- The gNB-CU respond the LMF with UPLINK INFORMATION RESPONSE message.

### 8.x.3	Positioning Measurement

This procedure is used for the uplink positioning measurement transfer between LMF and gNB-DU in NR uplink positioning. Figure 8.x.3-1 shows the positioning measurement procedure.

Figure 8.x.3-1 Positioning measurement procedure

- The LMF send MEASUREMENT REQUEST message to the gNB-CU.

- The gNB-CU send POSITIONING MEASUREMENT REQUEST message to the gNB-DU.

- The gNB-DU respond the gNB-CU with POSITIONING MEASUREMENT RESPONSE message.

- The gNB-CU respond the LMF with MEASUREMENT RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>