3GPP TSG-RAN WG3 Meeting #104	 R3-192749

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0049 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.401 on secondary node modification (MN/SN initiated) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During past meetings, RRC Reconfiguration Complete Indicator with value of true/false is introduced for F1 interface. As stated in TS 38.473, gNB-CU may send RRC Reconfiguration Complete Indicator to inform the gNB-DU to start to use the new configuration. Otherwise, it depends on gNB-DU’s implementation. However, things are different in EN-DC case. When SN is composed of gNB-CU and gNB-DU, modification initiated by either gNB-CU or gNB-DU should first be confirmed by MN. Then gNB-CU should clearly inform gNB-DU whether MN accepts the modification initiated by SN or not. However, there’s no stage 2 procedures correspondingly as in TS 38.401, the secondary node modification (MN/SN initiated) procedures for CU-DU architecture are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add MN initiated secondary node modification procedure in EN-DC - Add SN initiated secondary node modification procedure in EN-DC with gNB-CU/gNB-DU triggered modification Impact analysis: This CR has no impact with the previous version of the specification, because the impact can be considered isolated, and there is no backward compatible issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The secondary node modification procedure in EN-DC would not work well. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev# - : R3-191930 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

8.4.X	Secondary Node Modification (MN/SN initiated)

8.4.X.1	EN-DC

This clause gives the SgNB modification procedure in EN-DC given that the en-gNB consists of a gNB-CU and gNB-DU(s).

MN initiated SN Modification

Figure 8.4.3.1-1: SN Modification procedure in EN-DC (MN initiated)

1: refer to TS 37.340 [12]

a1. After receiving SGNB MODIFICATION REQUEST message from MeNB, gNB-CU may send UE CONTEXT MODIFICATION REQUEST message including CGConfigInfo to gNB-DU.

a2. The gNB-DU responds to the gNB-CU with UE CONEXT MODIFICATION RESPONSE message, which includs modified CellGroupConfig correspondingly.

2-7: refer to TS 37.340 [12]

a3. Based on the result of step 4, i.e., respond from MeNB, the gNB-CU initiates UE context modification procedure by sending UE CONTEXT MODIFICATION REQUEST message, which includes RRC Reconfiguration Complete Indicator.

a4. The gNB-DU responds to the gNB-CU with UE CONEXT MODIFICATION RESPONSE message.

SN initiated SN Modification

Figure 8.4.3.1-1: SN Modification procedure in EN-DC (SN initiated)

There’re two sub-cases for the SN initiated SN Modificaiton procedure, one is modification triggered by gNB-CU, and another is modification initiated by gNB-DU. For both sub-cases, L1/L2 modifications for UE are expected.

- modification initiated by gNB-CU

a1. The gNB-CU initiates UE context modification procedure by sending UE CONTEXT MODIFICATION REQUEST message.

a2. The gNB-DU responds to the gNB-CU with UE CONEXT MODIFICATION RESPONSE message including CellGroupConfig.

- modification initiated by gNB-U

b1. The gNB-DU initiates UE context modification procedure by sending UE CONTEXT MODIFICATION REQUIRED message with CellGroupConfig.

1~5: refer to TS 37.340 [12]

After step 1-5, there’re remaining steps for the two sub-cases.

- modification initiated by gNB-CU

a3. Based on the result of step 4, i.e., respond from MeNB, the gNB-CU initiates UE context modification procedure by sending UE CONTEXT MODIFICATION REQUEST message, which includes RRC Reconfiguration Complete Indicator.

a4. The gNB-DU responds to the gNB-CU with UE CONEXT MODIFICATION RESPONSE message.

- modification initiated by gNB-DU

b2. The gNB-CU responds to the gNB-DU with UE CONEXT MODIFICATION CONFIRM/REFUSE message, based on the result of step 4, i.e., respond from MeNB.

<<<<<<<<<<<<<<<<<<<< Changes End>>>>>>>>>>>>>>>>>>>>