3GPP TSG-RAN WG3 Meeting #104	R3-192767

Reno, USA, 13 – 17 May, 2019

Agenda item:	25.2.3.5

Source:	Nokia, Nokia Shanghai Bell

Title:	CCO: Supplementary Uplink (SUL) Parameters Optimization

Document for:	Text Proposal

# 1	Introduction

This paper focuses on the optimization of Supplementary Uplink (SUL) configuration. SUL allows the network to configure a UE with 2 ULs for one DL of the same cell (TS 38.300).  Uplink transmissions on those two ULs are controlled by the network. For random access in a cell with SUL, the network can either explicitly signal which carrier to use (UL or SUL) or the UE can select the SUL carrier if and only if the measured quality of the DL is lower than a broadcast threshold. This paper analyses scenario with potential mismatch between the SUL configuration and the actual radio coverage of SUL carrier. The paper also provides test proposal for SUL optimization solution for NR.

# 2	Discussion

The feature of Supplementary Uplink (SUL) has been introduced in 5G to enhance the UL coverage. UL coverage is usually more limited compared to DL due to smaller transmit power at UEs. In case of SUL, a UE can be configured with two ULs for one DL in the same cell (TS 38.300). To enable a wider coverage area of the SUL compared to the Normal Uplink (NUL), the SUL carriers can be deployed in the lower spectrum bands like 800 MHz.

Figure 1 below depicts the coverage area of a cell configured with SUL. The dark blue area represents the limited NUL coverage, whereas the light blue color represents the SUL coverage as well as the DL coverage of the cell. To help the UE switch between SUL and NUL, gNB can configure a parameter rsrp-ThresholdSSB-SUL (TS 38.321 and TS 38.331). The UE selects the SUL carrier if the RSRP of the downlink pathloss reference is less than this threshold (far away from cell center) and selects the NUL otherwise.

Figure 1: Cell coverage with SUL configuration

In some deployments, there could be a mismatch between the rsrp-ThresholdSSB-SUL and the actual NUL coverage as shown in Figure 1. The dotted oval represents the border as defined by the rsrp-ThresholdSSB-SUL to allow the switching between NUL and SUL, which is bigger than the NUL coverage. In such a case the red UE would try UL on the NUL carrier as it has not crossed the rsrp-ThresholdSSB-SUL threshold, but it will fail as it is out of NUL radio coverage. The legacy behavior is that the UE would re-try RACH access with incrementally increasing the transmit power in each attempt unless it reaches its maximum transmit power limit. If the UE is still not able to make successful RACH access, it will bar the cell for further attempts and try to search for other suitable cells. This would result into service interruption and negatively affect the UE quality of experience.

Observation: a mismatch between rsrp-ThresholdSSB-SUL and the actual radio coverage of SUL and NUL carriers would lead to improper selection of UL carrier and potential service interruption negatively affecting the UE quality of experience.

The solution therefore aims at detecting such cases of a mis-match between rsrp-ThresholdSSB-SUL threshold and the actual SUL/NUL radio coverage and to report this to the network in order to re-configure the rsrp-ThresholdSSB-SUL threshold for better selection of SUL and NUL carriers by the UE.

Proposal: It is proposed to agree on the TP in annex of this paper.

# 3	Conclusion

Observation: a mismatch between rsrp-ThresholdSSB-SUL and the actual radio coverage of SUL and NUL carriers would lead to improper selection of UL carrier and potential service interruption negatively affecting the UE quality of experience.

Proposal: It is proposed to agree on the TP in annex of this paper.

# Annex	- TP for TR 37.816

<<< start of changes >>>

## 5.1	Capacity and Coverage Optimization

### 5.1.1	Use case description

Editor Note: capture the use cases description and benefits of the use cases

Coverage and Capacity Optimization (CCO) is one of the typical operational tasks to optimize the radio access network (RAN). CCO has been identified as a key use case for SON since LTE, which aims to provide the required capacity in the targeted coverage areas and to minimize the interference and maintain an acceptable quality of service in an autonomous way. There is a trade-off between coverage and capacity optimization, capacity enhancements are usually at the expense of service coverage degradations, and vice versa. There is a need to balance and manage the trade-off between the two.

CCO allows the system to periodically adapt to the changes in traffic (i.e. load and location) and the radio environment

by automatically adjusting coverage for the cells that serve a certain area for a particular traffic situation. Due to the introduction of beam based antenna structures the set of configurable antenna and RF parameters are multi-dimensional. It is very complex to find the mapping between network configurations with target coverage and capacity performance. To make it feasible, some kind of machine learning techniques could be utilized which will leverage on the data collected in the RAN network. The collected data could be UE measurements, performance measurements, events and other monitoring information, also taking into account beamforming and massive MIMO - related information. These inputs could help the operator firstly identify the coverage and capacity problems, for instance, coverage hole, weak pilot pollution, overshoot coverage and DL and UL channel coverage mismatch and further perform the coverage and capacity optimization.

Coverage holes with unbalanced DL and UL channel coverage require consideration. The concept of RLF reporting from UE to RAN node and corresponding analysis provide the means to identify coverage holes and separate them from mobility related RLF failures, but a more detailed analysis of the root cause of a coverage hole is needed for an efficient CCO algorithm to detect UL channel coverage holes. Providing knowledge about root cause of a coverage hole by more elaborated analysis in the RAN node can trigger the right countermeasures quicker and more reliably.

### 5.1.2	Solution description

Editor Note: Capture the solutions for the use case, including the procedure for configuration and collection of measurements, necessary procedures and information exchange required for the solution, as well as comparison and evaluation on potential alternative solutions.

#### 5.1.2.x	SUL Parameters Optimisation

The feature of Supplementary Uplink (SUL) has been introduced in 5G to enhance the UL coverage. UL coverage is usually more limited compared to DL due to smaller transmit power at UEs. In case of SUL, a UE can be configured with two ULs for one DL in the same cell [TS 38.300]. To enable a wider coverage area of the SUL compared to the Normal Uplink (NUL), the SUL carriers can be deployed in the lower spectrum bands like 800 MHz.

Figure 1 below depicts the coverage area of a cell configured with SUL. The dark blue area represents the limited NUL coverage, whereas the light blue color represents the SUL coverage as well as the DL coverage of the cell. To help the UE switch between SUL and NUL, gNB can configure a parameter rsrp-ThresholdSSB-SUL [TS 38.321 and TS 38.331]. The UE selects the SUL carrier if the RSRP of the downlink pathloss reference is less than this threshold and selects the NUL otherwise.

Figure 1: Cell coverage with SUL configuration

In some deployments, there could be a mismatch between the rsrp-ThresholdSSB-SUL and the actual NUL coverage as shown in Figure 1. The dotted oval represents the border as defined by the rsrp-ThresholdSSB-SUL to allow the switching between NUL and SUL, which is bigger than the NUL coverage. In such a case the red UE would try UL on the NUL carrier as it has not crossed the rsrp-ThresholdSSB-SUL threshold, but it will fail. The legacy behavior is that the UE would re-try RACH access with incrementally increasing the transmit power in each attempt unless it reaches its maximum transmit power limit. If the UE is still not able to make successful RACH, it will bar the cell for further attempts and try to search for other suitable cells. This would result into service interruption and negatively affect the UE quality of experience.

Therefore, the solution aims at detecting such cases of a mismatch between rsrp-ThresholdSSB-SUL threshold and the actual SUL/NUL radio coverage and to report this to the network in order to re-configure the rsrp-ThresholdSSB-SUL threshold for better selection of SUL and NUL carriers by the UE.

SUL optimization is supported by UE reporting of random-access (RA) attempts on SUL and NUL carriers.

If the UE is supposed to use NUL based on the rsrp-ThresholdSSB-SUL but is unable to perform successful RA to the cell on NUL, the UE would store the details of each unsuccessful attempt like RA resource, power, time, DL RSRP, rsrp-ThresholdSSB-SUL, etc.

The UE can first try RA on SUL carrier before performing cell barring on this cell and trying to re-select to another cell.

If the UE is successful in its RA attempt on the SUL carrier, the UE reports the un-successful attempts on the NUL and the associated parameters it used for those attempts to the gNB.

If the UE is unsuccessful in its RA attempt even on SUL carrier, it will search for a suitable cell. Once the UE is connected to the new cell, the UE can report the unsuccessful RA attempts and associated parameters to the new cell.

The new cell can then forward those reports to the original serving cell.

The original serving gNB can analyse those reports (coming either directly from the UE or from the neighboring gNBs) and modify the network parameters like the rsrp-ThresholdSSB-SUL accordingly.

<<< end of changes >>>