3GPP TSG-RAN WG3 Meeting #104	R3-192818

Reno, Nevada, USA, 13th– 17th May, 2019	revision of R3-191844

Agenda Item:	13.2.4

Source: 	Huawei

Title:	Support of multiple connectivity in IAB network

Document for:	Discussion and Decision

Introduction

The WI on integrated access and backhaul for NR was setup in RAN#82[1], in which it mentions that support for NR-NR DC from the UE and IAB-node perspective.

The objectives of IAB for NR WI follow the recommendations by the study item on IAB for NR, which are defined in TR 38.874. With these recommendations, IAB supports the following features: - Multi-hop backhauling for flexible range extension for both FR1 and FR2. - Topology adaptation including redundant connectivity to optimize backhauling performance and to respond to backhaul (BH) link failure. - Mapping of UE bearers to backhaul RLC channels and QoS enforcement over backhaul RLC channels to meet E2E QoS requirements. - Scalability to a large number of UEs. - Flexible deployment allowing IAB-node operation in EN-DC mode with EPC or in SA-mode with 5GC. - Support for NR-NR DC from the UE and IAB-node perspective (see NOTE 1) - Efficient operation for both inband and out-of-band relaying. - OTA synchronization across IAB topology. - Support of Rel-15 UEs. NOTE 1: IAB relies heavily on existing Rel-15 functionality. It may leverage additional features/enhancements defined as part of other Rel-16 WIs, but it should not depend on the timely completion of these features. The scope of this section is limited to the aspects envisioned to be treated as part of the IAB WI.

And in the RAN2#105bis meeting [2], the following agreement was achieved for IAB multi-connectivity.

- R2 assumes that the NR DC framework (e.g. MCG SCG related procedures) is used to configure dual radio links used as IAB bh links with two parent nodes.

This paper discusses the issues and impacts related to multiple connectivity for IAB.

Discussion

In the TR 38.874 [3], two cases of multiple connectivity are studied, which are captured as followings:

-	Case 1: UE is multi-connected to the IAB-donor via redundant routes (traditional DC scenario).

-	Case 2: IAB-node is multi-connected to the IAB-donor node via redundant routes.

Both use cases are important to mitigate the impact of blockage for high frequency.

Case 1. Supporting UE multi-connectivity

Since the case 1 is traditional DC scenario, the NR-NR DC procedure can be reused from UE’s point of view. An example of the procedure is given as Figure 1.

Figure 1. Procedure for adding SCG link for the UE connecting to the IAB-donor via redundant routes

The procedure contains the following steps:

1.	A connection to the SCG on established for the dual-connecting UE using Rel-15 NR procedures.

2.	The IAB donor CU configures a new adaptation-layer route (SCG-route) as well as BH RLC channels on the wireless backhaul between new IAB-donor DU and the SCG IAB-node.

3.	The IAB donor CU may add new F1-U GTP-U tunnel for the dual-connecting UE towards the SCG IAB node DU via the new IAB donor DU, and establish new F1-C towards SCG IAB node via the new IAB donor DU, the new F1-U and F1-C towards the SCG IAB node will be carried by the configured new route. In addition, mapping rules about the new F1-U related UE bearers will be configured for the involved nodes in the new route.

It is proposed to define the procedure for adding SCG link for the UE to connect the IAB-donor via redundant routes.

Case 2. Supporting IAB node multi-connectivity

Based on the RAN2 105bis meeting agreements shown in the introduction section, the IAB node can use NR DC frame work to support dual radio links towards two parent nodes. The F1*-U and F1*-C terminate at the access IAB node DU, thus, multiple connecting IAB node can forward the F1*-U and F1*-C packets from either route towards the IAB donor CU.

Although it seems no obvious functional difference between the master and secondary route for IAB node, we still use the term MCG and SCG to differentiate the two path towards IAB donor CU. For simplicity, the first built route is defined as MCG path, and the second built route is defined as SCG path. The procedure for adding SCG link and redundant route to dual-connecting IAB-node should be defined also, an example similar to the section 9.7.9 of TR 38.874 [2] is shown in Figure 2.

1.	A connection to the SCG on established for the dual-connecting IAB-node MT using Rel-15 NR procedures.

2.	The IAB donor CU configures a new adaptation-layer route (SCG-route) as well as BH RLC channels on the wireless backhaul between dual-connecting IAB-node and IAB-donor DU via the SCG IAB-node.

3.	The gNB-CU further redirects the F1-U tunnel for UE and the F1-C connection towards the dual-connecting IAB node from the old route to the new route. Mapping rules update for the BH RLC channels along the new route may also necessary.

RAN3 takes the above procedure for adding SCG link for the IAB node as baseline for supporting BH multi-connectivity.

Figure 2. Procedure for adding SCG link for the UE connecting to the IAB-donor via redundant routes

One of the principle of designing an IAB network is to be agnostic to the Rel-15 UE. Functions for Rel-15 UE should be supported in the IAB network as much as possible. PDCP duplication is an essential feature of Rel15 UE to support higher reliability. There are two type PDCP duplications, i.e. CA based PDCP duplication and DC based PDCP duplication. On the other hand, supporting PDCP duplication is beneficial for reliability guarantee in case the blockage issue for IAB-node operating in high frequency. Therefore, both the two multiple connectivity cases should support UE PDCP duplication.

PDCP duplication should be supported. It is propose to discuss how to support CA based and DC based PDCP duplication in both cases.

Conclusion

In this paper, we discuss the issues and impacts related to multiple connectivity for IAB network and we propose:

- It is proposed to define the procedure for adding SCG link for the UE connecting to the IAB-donor via redundant routes.

- RAN3 takes the above procedure for adding SCG link for the IAB node as baseline for supporting BH multi-connectivity.

PDCP duplication should be supported. It is propose to discuss how to support CA based and DC based PDCP duplication in both cases.

- Reference

- RP-182882. New WID: Integrated Access and Backhaul for NR. Qualcomm.

- Chairman notes of RAN2#105bis meeting.

- TR 38.874, v16.0.0. Study on integrated access and backhaul.