3GPP TSG-RAN WG3 #104	R3-193189

Reno, Nevada, US, 13th May – 17th May 2019

Agenda Item:	25.2.3.7

Source:	Ericsson

Title:	Discussion on Energy Saving for RAN-centric DCU

Document for:	Discussion, Decision

# TP to TR 37.816

------------------------------------Start of Changes------------------------------------

The following use cases for R16 energy saving should be studied:

- Inter-system inter-RAT energy saving

Scenario a) The gNB connects with 5GC to provide boost capacity, the LTE eNB connects with EPC to provide basic coverage. The NR capacity booster cells may be switched off. The LTE eNB is allowed to activate the dormant capacity booster NR cell.

- Energy Efficiency of base stations

To calculate the energy efficiency of base stations, ES 203 228 (“Environmental Engineering (EE); Assessment of mobile network energy efficiency”) defines the following high-level EE KPI:

In which Mobile Network data Energy Efficiency (EEMN,DV) is the ratio between the performance indicator (i.e. Data Volume DVMN) and the energy consumption (ECMN).

3GPP SA5 with ETSI TC EE have identified the list of existing 3GPP performance measurements to be used, per pre-5G radio access technology (GERAN, UTRAN, E-UTRAN), for the calculation of the EE KPI. It should be studied how to achieve the same measures for NG-RAN. .

### 5.6.2 	Solution description

Editor Note: Capture the solutions for the use case, including the procedure for configuration and collection of measurements, necessary procedures and information exchange required for the solution, as well as comparison and evaluation on potential alternative solutions.

- Energy Efficiency of base stations

Data volume per site could be calculated by using the PDCP PDU data volume Measurement defined in TS 28.552, section 5.1.3.6.1. These measurements calculate the UL and DL PDCP data volume for a gNB-CU-UP.

If it can be assumed that a site is served by one CU-UP, then these measurements could already provide the desired statistics to calculate the energy per bit per site.

When looking at future cases it could be plausible to assume that one CU-UP may serve gNB-DUs at different sites. In this case the measurements in TS 28.552 could be taken as reference to define new measurements, taken at gNB-DU, and measuring the PDCP data throughput incoming to and outgoing from a gNB-DU

### 5.6.3 	Conclusion

RAN3 concluded that solutions for calculation of data volumes for the calculation of energy efficiency for a base station can be treated during normative phase according to the solution description above.