3GPP TSG-RAN WG3 Meeting #104	R3-192597

Reno, USA, May 13 – 17, 2019

Agenda item:	18.3

Source:	Qualcomm Incorporated

Title:	Lossless Make-Before-Break (MBB) HO support for RLC-UM

WID/SID:	NR mobility enhancements

Document for:	Discussion

# Introduction

The justification of New WI “NR mobility enhancements” [1] has the below justification:

“In Rel-15 NR, 0ms interruption time can be achievable by using intra-cell using beam mobility and addition/release of SCell for CA operation. However, there is demand to achieve 0ms interruption time in more scenarios especially in URLLC type of service which requires 1ms of end-to-end delay in some scenarios. However, there is demand to achieve 0ms interruption time in more scenarios especially in URLLC type of service which requires 1ms of end-to-end delay in some scenarios.”

We agree that there is a need to optimize mobility performance for URLLC type of services and other services with low-latency and high reliability performance requirements. In this paper, we discuss the performance needs of such services, the typical configurations for such services and the need for changes in the handover handling to support such services.

# Discussion

In LTE, RLC UM DRB is used for VoLTE/ViLTE. Almost all the other services use RLC AM. VoLTE/ViLTE is delay sensitive service. Packets beyond the delay budget are discarded. LTE handover usually has 50ms user plane interruption, which makes the forwarded data packets in handover exceed the delay budget typically. On the other hand, VoLTE/ViLTE can tolerant high packet error rate. So, LTE didn’t define lossless handover for RLC-UM.

Observation 1: LTE does not support lossless HO for RLC-UM because RLC-UM is used almost only for VoLTE/ViLTE, which is packet loss tolerant and delay sensitive.

URLLC is a key service of 5G. In order to achieve low latency, URLLC services typically use RLC UM on the radio interface as they cannot tolerate the RLC AM related retransmission latencies.As they also have a high reliability requirement, these services are sensitive to packet losses both on the air interface and backend during mobility.

Observation 2. Low latency and high reliability services should use RLC UM for the associated DRBs due to small PDB not allowing the RLC AM retransmission latencies.

In Rel.15 NR, U-plane handling is defined separately for RLC AM and RLC UM bearers to support additional reliability for RLC AM bearers and below is the text from [3] for PDCP SN handling.

For RLC-AM bearers:

-	For in-sequence delivery and duplication avoidance, PDCP SN is maintained on a per DRB basis and the source gNB informs the target gNB about the next DL PDCP SN to allocate to a packet which does not have a PDCP sequence number yet (either from source gNB or from the UPF).

-	For security synchronisation, HFN is also maintained and the source gNB provides to the target one reference HFN for the UL and one for the DL i.e. HFN and corresponding SN.

-	In both the UE and the target gNB, a window-based mechanism is used for duplication detection and reordering.

For RLC-UM bearers:

-	The PDCP SN and HFN are reset in the target gNB;

-	No PDCP SDUs are retransmitted in the target gNB;

-	The target gNB prioritises all downlink SDAP SDUs forwarded by the source gNB over the data from the core network;

Observation 3. PDCP SN and HFN are reset in the target gNB for RLC-UM bearers during an NR Rel.15 HO.

As highlighted above, Rel.15 assumes that PDCP SN continuity is only essential for RLC AM. However, as we discussed in [4], UE can receive DL data simultaneously on both source connection and target connection during an MBB HO. Also, UE does not reset the PDCP entity and maintains a common PDCP entity to support reordering/duplicate discard function at the PDCP layer for both RLC AM and RLC UM DRBs during an MBB HO.

In order to support this common PDCP entity reordering/duplicate discard function at the PDCP layer for RLC UM during MBB HO, the target gNB should not reset the PDCP SN for RLC-UM bearers. Instead, the source gNB and target gNB should support PDCP SN continuity for the RLC-UM bearers during MBB HO to avoid packet losses due to PDCP SN reset.

Observation 4. To handle the security key change during Inter-CU MBB HO without any PDCP reestablishment, we are proposing that UE supports a common PDCP entity to handle the source cell and target cell RLC AM/RLC UM bearers during MBB HO.

Observation 5. UE common PDCP entity can receive DL data simultaneously on both source connection and target connection during MBB HO. Thus, resetting the SN for RLC UM bearers during MBB HO could impact the reordering/duplicate discard function of the common PDCP entity and result in some packet loss for RLC UM bearers during MBB HO.

Observation 6. Packet loss for the RLC UM bearers during MBB HO is not suitable for the low latency and high reliability services.

Proposal 1. Support PDCP SN continuity by SN Status Transfer for both RLC-AM and RLC-UM bearers during an MBB HO.

# 3. Conclusion

Based on the above discussions, we recommend RAN2 discusses the following observations and proposals:

Observation 1. LTE does not support loss-less HO for RLC-UM because RLC-UM is used almost only for VoLTE/ViLTE, which is packet loss tolerant and delay sensitive..

Observation 2. Low latency and high reliability services should use RLC UM for the associated DRBs due to small PDB not allowing the RLC AM retransmission latencies.

Observation 3. PDCP SN and HFN are reset in the target gNB for RLC-UM bearers during an NR Rel.15 HO.

Observation 4. To handle the security key change during Inter-CU MBB HO without any PDCP reestablishment, we are proposing that UE supports a common PDCP entity to handle the source cell and target cell RLC AM/RLC UM bearers during MBB HO.

Observation 5. UE common PDCP entity can receive DL data simultaneously on both source connection and target connection during MBB HO. Thus, resetting the SN for RLC UM bearers during MBB HO could impact the reordering/duplicate discard function of the common PDCP entity and result in some packet loss for RLC UM bearers during MBB HO.

Observation 6. Packet loss for the RLC UM bearers during MBB HO is not suitable for the low latency and high reliability services.

Proposal 1. Support PDCP SN continuity by SN Status Transfer for both RLC-AM and RLC-UM bearers during an MBB HO.

# 4. References

[1] RP-181433   New WID: NR mobility enhancements

[2] TS 23.501 V15.2.0 System Architecture for the 5G System; Stage 2

[3] TS 38.300 V15.3.1 NR and NG-RAN Overall Description; Stage 2

[4] R2-1903645 User Plane changes to minimize the mobility interruption during Make-Before-Break

(MBB) HO