3GPP TSG-RAN3 Meeting #104	R3-192930

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enforcement of UP integrity protection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In [S2-1902738], SA2 agreed the the enforcement of the UP integrity protection. - it is responsibility of the NG-RAN to enforce that the maximum UP integrity protection data rate delivered to the UE in downlink is not exceeding the maximum supported data rate for integrity protection. At RAN3#103 meeting, it was agreed that the both the UL maximum integrity protected data rate and DL maximum integrity protected data rate are provided to the NG-RAN. Hence it requires to clearly indicate that only the DL IP is enforced by the NG-RAN no matter whether both the DL and UL IP data rate are provided respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description on the enforcement of the maximum bit rate for integrity protection. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) - This CR has an impact under functional point of view. - The impact can be considered isolated because the change affects the UP integrity protection enforcement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous regarding the enforcement of the maximum bit rate for integrity protection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

13.1	Overview and Principles

<Unchanged Text Omitted>

The maximum supported data rate for integrity protected DRBs is a UE capability indicated at NAS layer, with a minimum value of 64 kbps and a maximum value of the highest data rate supported by the UE. The maximum supported data rate for integrity protected DRBs in the DL is enforced by the gNB. The maximum supported data rate for integrity protected DRBs in the UL is enforced by the UE. In case of failed integrity check (i.e. faulty or missing MAC-I), the concerned PDU shall be discarded by the receiving PDCP entity.

<Unchanged Text Omitted>