3GPP TSG-RAN WG3 Meeting #104	R3-192979

# Reno, Nevada, US, 13th-17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0062 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | gNB-CU-UP SCTP association addition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently there is no detailed description on the procedure for managing multiple TNLAs for E1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We propose to provide the procedure for managing multiple TNLAs for E1. Impact assessment towards the previous version of the specification (same release): This CR is backward compatible since it has no impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the current specification, there does not exist a detailed description for managing multiple TNLAs for E1 while such description exist for F1-C. The lack of such description for the E1 could potentially result in misinterpretations on the procedure for managing multiples TNLAs for E1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

## 8.x	Multiple TNLAs for E1

In the following, the procedure for managing multiple TNLAs for E1 is described.

Figure 8.x-1: Managing multiple TNLAs for E1.

1.	Either the gNB-CU-CP or gNB-CU-UP establishes the first TNLA with the gNB-CU-UP or gNB-CU-CP respectively using a configured TNL address.

NOTE: 	The gNB-CU-CP and gNB-CU-UP may use different source and/or destination IP end point(s) if the TNL establishment towards one IP end point fails. How the gNB-CU-CP or gNB-CU-UP gets the remote IP end point(s) and its own IP address are outside the scope of this specification.

2-3.	Once the TNLA has been established gNB-CU-CP or gNB-CU-UP initiates the E1 Setup procedure to exchange application level configuration data

NOTE: 	If the gNB-CU-UP initiates the first TNL association, it shall also initiate the gNB-CU-UP E1 Setup procedure. If the gNB-CU-CP initiates the first TNL association, it shall also initiate the gNB-CU-CP E1 Setup procedure. In the latter case the exchange in steps 2 and 3 would correspond to the exchange of GNB-CU-CP E1 SETUP REQUEST message accompanied by a GNB-CU-CP E1 RESPONSE message in the opposite direction.

4-6.	The gNB-CU-CP may add additional TNL Endpoint(s) to be used for E1 signalling between the gNB-CU-CP and the gNB-CP-UP pair using the gNB-CU-CP Configuration Update procedure. The gNB-CU-UP Configuration Update procedure also allows the gNB-CU-CP to request the gNB-CU-UP to modify or release TNLA(s).

7-9.	The gNB-CU-UP may add additional TNL association(s) to be used for E1 signalling using a gNB-CU-CP endpoint already in use for existing TNL associations between the gNB-CU-CP and the gNB-CU-UP pair. The NB-CU-UP CONFIGURATION UPDATE message including the gNB-CU-UP ID shall be the first E1AP message sent on an additional TNLA of an already setup E1 interface instance after the TNL association has become operational. The E1AP UE TNLA binding is a binding between a E1AP UE association and a specific TNL association for a given UE. After the E1AP UE TNLA binding is created, the gNB-CU-CP can update the UE TNLA binding by sending the E1AP message for the UE to the gNB-CU-UP via a different TNLA. The gNB-CU-UP shall update the E1AP UE TNLA binding with the new TNLA.

<<< END OF CHANGES >>>