3GPP TSG RAN WG3#104 	                                                                               R3-193209

Reno, USA, 13th – 17th  May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.821 | CR | pCR | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for TR38.821 Clarification of NTN Architecture Aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2019-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current baseline TR38.821, for gNB on board case in sub-section 5.2.1.1, it has been described that: “The gNB on board different satellites may be connected to the same 5GCN on the ground.” and “If the satellite hosts more than one gNB, the same SRI will transport all the corresponding NG interface instances.”. Similarly, for gNB-DU on board case in sub-section 5.3.2.1, it has been also described that:” DU on board different satellites may be connected to the same CU on ground. and “If the satellite hosts more than one DU, the same SRI will transport all the corresponding F1 interface instances.” However, the counterpart statements for transpararent satellite case is still missing, hence different NTN architecture options were not well described with the same level of details. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add up statements in sub-section 5.1.1 that “Different transparent satellites may be connected to the same gNB on the ground.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion or incompleteness exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change, ommited text not changed ***

### 5.1.1	Overview

The satellite payload implements frequency conversion and a Radio Frequency amplifier in both up link and down link direction. It corresponds to an analogue RF repeater.

Hence the satellite repeats the NR-Uu radio interface from the feeder link (between the NTN gateway and the satellite) to the service link (between the satellite and the UE) and vice versa.

The Satellite Radio Interface (SRI) on the feeder link is the NR-Uu. In other words, the satellite does not terminate NR-Uu.

The NTN GW supports all necessary functions to forward the signal of NR-Uu interface.

Different transparent satellites may be connected to the same gNB on the ground.

Figure 5.2-1: Networking-RAN architecture with transparent satellite

Note: Whilst several gNBs may access a single satellite payload, the description has been simplified to a unique gNB accessing the satellite payload, without loss of generality.

*** End change ***