3GPP TSG RAN WG3 Meeting #104 	R3-193277

Reno, 13-17th May 2019														revision of R3-192914, 193213

Title:	CB: # 60_NG-RAN_impacts_split_arch

Source: 	Thales, HNS

Type:	pCR

Document for:	Agreement

Agenda Item:	20.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at correcting the chapter 5.2.2.3 of the TR 38.821 based on the outcomes of the NTN on line session held in RAN3#104.

Hereunder are recalled the minutes captured by the RAN3 chair in its notes:

CB: # 60_NG-RAN_impacts_split_arch

-  give background, explanation

- F1 impacts are only on implementation?

- clarify

(Th)

Rev in R3-193213

Th: the RTT for LEO is comparable with what was studied for F1

E///: depends on what we compare; RTT for LEO, I think, are fundamentally different

- “5.2.2.3	NG-RAN impacts

RRC and other Layer3 processing are terminated in the gNB-CU on ground, and is are subject to stricter timing constraints.

The use of this architecture option for LEO (Low Earth Orbit) or even for GEO (Geostationary Earth Orbit) systems may impact current F1 implementation (e.g. timer extensions). Such impact for LEO will be much less significant than for GEO.”

Rev in R3-193277 Agreed unseen

# Discussion

Based on the current analysis, no showstopper have been identified on the applicability of the gNB split architecture to LEO or even GEO satellite, it is suggested to reword clause 5.2.2.3 as in the TP below.

# Conclusion

Proposal 1: Reflect the above proposals in the TR 38.821 (See TP below)

# Text Proposal for TR 38.821 v0.6.0

START OF CHANGES

### 5.2.2	gNB-DU processed payload

…

5.2.2.3	NG-RAN impacts

RRC and other Layer3 processing are terminated in the gNB-CU on ground, and are subject to stricter timing constraints.

The use of this architecture option for LEO (Low Earth Orbit) or even for GEO (Geostationary Earth Orbit) systems may impact current F1 implementation (e.g. timer extensions). Such impact for LEO will be much less significant than for GEO.

END OF CHANGES