3GPP TSG-RAN WG3 Meeting #105	R3-193744

# Ljubljana, Slovenia, August 26th – 30th 2019

Agenda Item:	13.2.2

Source:	Ericsson

Title:		IAB DL Flow Control - General Principles

Document for:	Agreement

# Introduction

The issue of IAB flow control has not yet been discussed in the RAN3 WI, and it was a subject of an RAN2 email discussion. This paper is an attempt to agree on some basic principles of IAB DL flow control and pave the way for progress in this area.

# Discussion

The ground principles of IAB DL flow control are discussed below.

## The purpose of flow control

Flow control (FC) is often mistaken for congestion control. Namely, congestion control refers to mechanisms invoked after congestion has occurred, while FC aims is to keep the buffers on a path small enough to avoid queue build-up in the first place. Moreover, the FC mechanism must also “constantly fill the pipe” with packets, for efficient use of resources. In other words, FC is not about reactive handling overflows at intermediate IAB nodes on a path, but rather to keep in check the queues in the intermediate nodes.

Observation 1: Flow control is not about reactive handling overflow at intermediate IAB nodes on a path but rather to keep in check the queues in the intermediate nodes.

The possible use case for F1 flow control could include:

- Active Queue Management of in the CU-UP, i.e. selective dropping of packet above the PDCP layer, to improve TCP performance.

- Improve performance for split bearers in the case of DC. E.g. by trying to keep the delay the same on both paths to avoid that the receiver needs to perform too much re-ordering which would increase the overall delay and buffering requirements.

Observation 2: The possible use case for F1 flow control could include:

- Active Queue Management of in the CU-UP, i.e. selective dropping of packet above the PDCP layer, to improve TCP performance.

- Improve performance for split bearers in the case of DC. E.g. by trying to keep the delay the same on both paths to avoid that the receiver needs to perform too much re-ordering which would increase the overall delay and buffering requirements.

Proposal 1: RAN3 to agree that the purpose of IAB DL flow control is in line with the definition of flow control, i.e. to prevent queue build-up and to ensure efficient use of resources by injecting an appropriate amount of traffic into the network.

Feedback is without exception an inherent part of any given flow control mechanism, where the natural recipient of the feedback is the node responsible for injecting the traffic into the network or a segment of the network. In the IAB context, this node is the donor, or, more accurately the donor CU-UP entity. Accordingly, the recipient of feedback in the IAB DL flow control mechanism should be the donor CU or donor CU-UP. Any other alternative would mean that the pushback is intended to some other intermediate node, which may lead to queue growth, since the node that injects the traffic (i.e. the donor CU-UP) will be unaware and continue to inject the traffic into the network uninterruptedly.

Observation 3: Feeding back the IAB DL flow control report to any node other than the donor CU (i.e. the donor CU-UP) may lead to queue growth at intermediate nodes, since the node that injects the traffic (i.e. the donor CU-UP) will be unaware and will continue to inject the traffic into the network uninterruptedly.

Proposal 2: RAN3 to agree that the recipient of a flow control feedback message shall be the donor CU-UP, or donor CU (in case of no CP-UP split).

## The transport of flow control signalling

Another key issue to decide is which entity is responsible for flow control i.e. whether flow control feedback messages are carried on the UP or CP. Considering that flow control is an UP function, this should not change for IAB networks, since otherwise it would breach the CP-UP separation principle. Thus, flow control feedback from IAB nodes should be carried on the UP, just as it is specified for regular CU-DU split.

Observation 4: Flow control signalling in regular CU-DU split case is carried on the UP, which is in line with the CP-UP separation principle.

As explained earlier, the task of FC is to keep the buffers on a path small enough to avoid queue build-up in the first place, which may incur frequent signalling for flow adjustment. In other words, the signalling for FC scales with the UP traffic and it does not seem reasonable to carry this signalling on the CP.

Observation 5: The signalling for flow control scales with the UP traffic and it does not seem reasonable to carry this signalling on the CP.

Proposal 3: RAN3 to agree that the IAB flow control signaling should be carried on the UP, just as it is specified for regular CU-DU split case.

# Conclusion

This paper discusses the general principles of IAB. The following is observed:

Observation 1: Flow control is not about reactive handling overflow at intermediate IAB nodes on a path but rather to keep in check the queues in the intermediate nodes.

Observation 2: The possible use case for F1 flow control could include:

- Active Queue Management of in the CU-UP, i.e. selective dropping of packet above the PDCP layer, to improve TCP performance.

- Improve performance for split bearers in the case of DC. E.g. by trying to keep the delay the same on both paths to avoid that the receiver needs to perform too much re-ordering which would increase the overall delay and buffering requirements.

Observation 3: Feeding back the IAB DL flow control report to any node other than the donor CU (i.e. the donor CU-UP) may lead to queue growth at intermediate nodes, since the node that injects the traffic (i.e. the donor CU-UP) will be unaware and will continue to inject the traffic into the network uninterruptedly.

Observation 4: Flow control signalling in regular CU-DU split case is carried on the UP, which is in line with the CP-UP separation principle.

Observation 5: The signalling for flow control scales with the UP traffic and it does not seem reasonable to carry this signalling on the CP.

Based on the observations, the following is proposed:

Proposal 1: RAN3 to agree that the purpose of IAB DL flow control is in line with the definition of flow control, i.e. to prevent queue build-up and to ensure efficient use of resources by injecting an appropriate amount of traffic into the network.

Proposal 2: RAN3 to agree that the recipient of a flow control feedback message shall be the donor CU-UP, or donor CU (in case of no CP-UP split).

Proposal 3: RAN3 to agree that the IAB flow control signaling should be carried on the UP, just as it is specified for regular CU-DU split case.