3GPP TSG-RAN3 Meeting #105	R3-193755

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility Robustness Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to capture the stage 2 text for support of Mobility Optimization function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 2 text description for Mobility Optimization function is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of Mobility Optimization function can not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

### X.1	Support for Mobility Robustness Optimisation

#### X.1.1	Connection failure due to intra-system mobility

Failure event definition:

- [Intra-system Too Late Handover] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

- [Intra-system Too Early Handover] An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

- Intra-system Handover to Wrong Cell] An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

Detection mechanism

Detection occurs either after an RRC re-establishment attempt or after a successful RRC connection setup. The NG-RAN node receiving the RRC re-establishment attempt or the RLF Report (in case of successful RRC connection setup) from the UE may forward the information to the NG-RAN node that served the UE before the reported connection failure using the RLF INDICATION message on Xn or NG interface.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

- [Intra-system Too Late Handover]
There is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)

- [Intra-system Too Early Handover]
There is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect is the cell that served the UE at the last handover initialisation.

- [Intra-system Handover to Wrong Cell]
There is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

For the latter two cases, the report HANDOVER REPORT message on Xn or NG.is used to transfer information from NG-RAN node that served the UE before the reported connection failure to the NG-RAN node that served the UE before the last handover initialization.

#### X.1.2	Connection failure due to inter-system mobility

Failure events definition

- [Inter-system/ Too Late Handover] An RLF occurs after the UE has stayed in a NG-RAN cell which connects with 5GC for a long period of time; the UE attempts to re-establish to an E-UTRAN cell which connects with EPC.

- [Inter-system/ Too Early Handover] An RLF occurs shortly after a successful handover from a E-UTRAN cell which connects with EPC to a target cell in a NG-RAN cell which connects with 5GC; the UE attempts to re-establish to the source cell or to another E-UTRAN cell which connects with EPC.

Detection mechanism

The UE makes the RLF Report available to the NG-RAN node after reconnecting from idle mode. [FFS whether we should also cover re-establishment attempt case, since UE RLF does not exist in NR rel15]

The NG-RAN node receiving the RLF Report from the UE may forward the report to the NG-RAN node that served the UE before the reported connection failure using the RLF INDICATION message over Xn or by means of the Downlink RAN Configuration Transfer procedure and Uplink RAN Configuration Transfer procedure over NG.

- [Too Late Inter-system Handover]
The connection failure occurs while being connected to an NG-RAN cell, and there is no recent handover for the UE prior to the connection failure i.e., the UE reported timer is absent or larger than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect/re-establish is a E-UTRAN cell which connect with EPC.

- [Too Early Inter-system Handover]
The connection failure occurs while being connected to an NG-RAN cell, and there is a recent inter-system handover for the UE prior to the connection failure i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect/re-establish and the cell that served the UE at the last handover initialisation are both E-UTRAN cell which connect with EPC.

#### X.1.3	Inter-system HO ping-pong

Failure events definition

A UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE. The event may occur more than once.

Detection mechanism

The statistics regarding ping-pong occurrence may be based on evaluation of the UE History Information IE in the HANDOVER REQUIRED message. If the evaluation indicates a potential ping-pong case and the source NG-RAN node of the 1st inter-system handover is different than the target NG-RAN node of the 2nd inter-system handover, the target NG-RAN node may use the HANDOVER REPORT message on Xn or NG interface to indicate the occurrence of potential ping-pong cases to the source NG-RAN node.

#### X.1.4    SN change failure in case of EN-DC, NGEN-DC, NE-DC and NR-DC

SN change-related failures can be categorized as follows:

-	Failures due to too late SN change triggering: an SCG failure occurs after the UE has stayed for a long period of time in the cell of the SN; the MN makes decisions for UE, making UE to establish the radio link connection in a different SN.

-	Failures due to too early SN change triggering: an SCG failure occurs shortly after a successful SN change from a source SN to a target SN or a SN change failure occurs during the SN change procedure; the MN makes decisions for UE, making UE to re-establish the radio link connection in the source SN.

-	Failures due to change to wrong SN triggering: an SCG failure occurs shortly after a successful SN change from a source SN to a target SN or a SN change failure occurs during the SN change procedure; the MN makes decisions for UE, making UE to establish the radio link connection in a SN other than the source SN or target SN.

The detection mechanisms for too late SN change, too early SN change and SN change to wrong cell are carried out through the following:

-	[Too Late SN change]
After MN receive SCG failure information, MN may then use this information to determine whether the failure occurred in the serving cell.

-	[Too Early SN change]
If the target Scell belongs to an SN B different from the SN A that controls the source Scell, the MN may send a SCG failure information indicating a Too Early SN change event to SN A upon MN receives an SCG failure information from UE and MN determines that the UE needs to establish source SN.

-	[To Wrong SN change]
If the target Scell belongs to an SN B different from the SN A that controls the source Scell, the MN may send a SCG failure information indicating a To Wrong SN change event to SN A upon MN receives an SCG failure information from UE and MN determines that the UE needs to establish a new SN.

#### X.1.5    Successful HO Report

Successful handover report include every successful handover between two cells of the following information for the handover parameter optimization:

- The UE related measurements at the handover trigger

- The UE related measurements at the end of handover execution

- The UE related measurements shortly after handover execution (first few seconds)

The target NG-RAN node should forward the Successful Handover Report to the source NR-RAN node via UE CONTEXT RELEASE message after receiving the report from the UE to indicate failures experienced so as to optimize the mobility parameters during a successful handover event.

End of the first change