3GPP TSG-RAN3 Meeting #105	R3-193764

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0184 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Slice Availability over NG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Signaling interaction is added to obtain slice-support information to avoid mobility failures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the neighbor cells where Xn connection is not available, the gNB can request the IE corresponding only to slice support information of these detected neighbor cells to AMF in the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) - This CR has an impact under functional point of view. - The impact can be considered isolated because the change affects the Downlink RAN configuration transfer function and Uplink RAN configuration transfer function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility failures due to the lack of slice-support information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2.2; 9.3.3.7; 9.3.3.8; 9.3.3.x; 9.4.5; 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

### 8.8.2	Downlink RAN Configuration Transfer

#### 8.8.2.1	General

The purpose of the Downlink RAN Configuration Transfer procedure is to transfer RAN configuration information from the AMF to the NG-RAN node. This procedure uses non-UE associated signalling.

#### 8.8.2.2	Successful Operation

Figure 8.8.2.2-1: Downlink RAN configuration transfer

<Unchanged Text Omitted>

If the NG-RAN node is configured to use one IPsec tunnel for all NG and Xn traffic (IPsec star topology) then the traffic to the peer NG-RAN node shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

If the NG-RAN node receives the SON Information IE containing the SON Information Reply IE including the TA Supported Slice List IE as an answer to a former request, it may use it for further network slice related decisions.

#### 8.8.2.3	Abnormal Conditions

Void.

Next Change

#### 9.3.3.7	SON Information

This IE identifies the nature of the configuration information transferred, i.e., a request, a reply or a report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information | M |  |  |  |
| >SON Information Request |  |  |  |  |
| >>SON Information Request | M |  | ENUMERATED (Xn TNL Configuration Info, …, TA Slice Support Info) |  |
| >SON Information Reply |  |  |  |  |
| >>SON Information Reply | M |  | 9.3.3.8 |  |

Next Change

#### 9.3.3.8	SON Information Reply

This IE contains the configuration information to be replied to the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Xn TNL Configuration Info | O |  | 9.3.3.9 |  |
| TA Supported Slice List | O |  | 9.3.3.x |  |

Next Change

#### 9.3.3.x	TA Supported Slice List

This IE contains a list of supported slices per TA in the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TA Supported Slice Item |  | 1..<maxnoofTACs> |  |  |
| >TAC | M |  | 9.3.3.10 | Broadcast TAC |
| >Broadcast PLMN List |  | 1 |  |  |
| >>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  |
| >>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN |
| >>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TA. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

Next Change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-TASupportedSliceList,

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

<Unchanged Text Omitted>

SONInformationReply ::= SEQUENCE {

xnTNLConfigurationInfo		XnTNLConfigurationInfo									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SONInformationReply-ExtIEs} }		OPTIONAL,

...

}

SONInformationReply-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-TASupportedSliceList		CRITICALITY ignore	EXTENSION TASupportedSliceList		PRESENCE optional},

...

}

SONInformationRequest ::= ENUMERATED {

xn-TNL-configuration-info,

...,

ta-slice-support-info

}

<Unchanged Text Omitted>

TASupportedSliceList ::= SEQUENCE (SIZE(1..maxnoofTACs)) OF TASupportedSliceItem

TASupportedSliceItem ::= SEQUENCE {

tAC									TAC,

broadcastPLMNList		BroadcastPLMNList,

iE-Extensions		ProtocolExtensionContainer { { TASupportedSliceItem-ExtIEs} } OPTIONAL,

...

}

TASupportedSliceItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<Unchanged Text Omitted>

Next Change

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-TASupportedSliceList								ProtocolIE-ID ::= xxx

End Change