3GPP TSG-RAN WG3 Meeting #105	R3-193799

Ljubljana, Slovenia, 26 – 30 August, 2019

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN3 aspects of user data congestion analysis in the NWDAF

Document for:	Discussion and Decision

# 1	Introduction

In their LS S2-1906797 [1], SA2 informs about their work on user data congestion related analytics in the NWDAF and invites SA5 and RAN3 in destination of the LS to respond with any comments and/or recommendations.

In this paper we analyse RAN3 impacts related to the mentioned analytics and propose an answer to SA2.

# 2	Discussion

According to SA2’s LS and attached pCR, a Network Function or Application Function may request user data congestion analytics from the NWDAF, which will collect Performance Measurements (PM) from OAM per cell in the area of interest and provide analysis back to the requestor. The following PM are given as example: UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management, and Radio Resource Utilization as defined in TS 28.552. Analytics for both user plane and control plane congestion shall be enabled in Rel-16.

Feasibility of these measurements, defined in SA5 specification, have already been confirmed by RAN2 and RAN3. Also, SA5 may be the main concerned group to comment on applicability of these measurements for the purpose described by SA2, e.g. use of UE Throughput measurement may require access to some reference UE Throughput measurement (or history) to identify a congestion scenario, which is out of RAN3 scope.

From RAN3 side it may still be commented that analysis of data congestion needs to be done per network slice, which is the method used in NG-RAN nodes to isolate resources (transport, processing, radio) and hence control or limit the impact of any congestion scenario. E.g. user data congestion in control plane may require knowledge of available resource in slice for CIoT UEs.

We therefore propose the following answer to SA2:

“RAN3 would like to draw the attention of SA2 and SA5 to the fact that resources in 5G nodes under RAN3 responsibility may be reserved per network slice. RAN3 therefore believes that the NWDAF will need measured information per slice for user data congestion analytics in a given geographical area.”

Proposal: Send a reply LS to SA2 and SA5 as described above.

# 3	Conclusion

We have suggested a reply to SA2 and SA5.

Proposal: Send a reply LS to SA2 and SA5 as described above.

A draft LS is submitted to this meeting in [2].

# References

[1]		R3-193325_S2-1906797, LS on User Data Congestion Analytics, SA2

[2]		R3-193800, [DRAFT] Reply LS on User Data Congestion Analytics (to SA2, SA5), Nokia, Nokia Shanghai Bell