3GPP TSG-RAN3 Meeting #105	R3-194059

Ljubljana, Slovenia, 26-30 August, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0406 | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference timing delivery over F1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IIOT |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to introduce the reference timing unicast delivery functionality over F1 interface, the following procedure of the reference timing information delivery should be specified. - The gNB-CU initiates the procedure by sending a REFERENCE TIMING INFORMATION REQUEST message, i.e. TimeReferenceInfo-r16 included in the SIB9 to the gNB-DU. - Upon reception of the REFERENCE TIMING INFORMATION REQUEST, the gNB-DU shall respond with the REFERENCE TIMING INFORMATION RESPONSE to the gNB-CU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The gNB-CU initiates the procedure by sending a REFERENCE TIMING INFORMATION REQUEST message, i.e. TimeReferenceInfo-r16 included in the SIB9 to the gNB-DU. - Upon reception of the REFERENCE TIMING INFORMATION REQUEST, the gNB-DU shall respond with the REFERENCE TIMING INFORMATION RESPONSE to the gNB-CU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The accurate reference timing delivery functionality for IIOT is not supported for the disaggregated gNB case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Omitted>

----------------------------------------------------------Next Change--------------------------------------------------------

8.X	Reference Timing Information Delivery

8.X.1	General

The purpose of the reference timing delivery procedure is to command the gNB-DU to send the requested the reference timing information to gNB-CU. The procedure uses non-UE associated signalling.

8.X.2	Successful Operation

Figure 8.x.2-1: Reference Timing Information Delivery procedure. Successful operation.

The gNB-CU initiates the procedure by sending REFERENCE TIMING INFORMATION REQUEST message to the gNB-DU.

Upon reception of the REFERENCE TIMING INFORMATION REQUEST message, the gNB-DU shall deliver the TimeReferenceInfo IE in the REFERENCE TIMING INFORMATION RESPONSE message.

8.X.3	Abnormal Conditions

Not applicable.

----------------------------------------------------------Next Change--------------------------------------------------------

9.2.X	Reference Timing Information Delivery messages

9.2.x.1	REFERENCE TIMING INFORMATION REQUEST

This message is sent by the gNB-CU and is used to request the gNB-DU to deliver the reference timing information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |

9.2.x.2	REFERENCE TIMING INFORMATION RESPONSE

This message is sent by the gNB-DU to transfer the reference timing information associated to an F1-C interface instance.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| TimeReferenceInfo | M |  | OCTET STRING | TimeReferenceInfo as defined in TS 38.331 [8]. | YES | reject |