3GPP TSG-RAN3 Meeting #105	R3-194090

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.300 for RACH optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to capture the stage 2 function description for support of RACH optimization function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 2 function description for RACH optimization is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of RACH optimization can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.401 CR 00XX |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS 38.423 CR 00XX |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS 38.473 CR 00XX |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

15.x	Support for RACH Optimisation

RACH optimization is supported by UE RACH report and by PRACH parameters exchange over Xn between gNBs. Both the PRACH parameters of normal UL carrier and SUL carrier are exchanged over Xn.

Upon receiving the polling message requesting RACH report, The UE shall report the below RACH information:

Editor Note: detailed parameters in UE RACH report are pending to RAN2 discussion.

Based on the UE report received from the UEs and PRACH paramters of neighbour cells received over Xn, the setting of RACH parameters that can be optimized includes:

-	PRACH configuration

-	RACH preamble split (among dedicated, group A, group B)

-	RACH backoff parameter value

-	RACH transmission power control parameters

End of change